3GPP TSG-RAN WG4 Meeting #101-bis-e													R4-2203091

Electronic Meeting, January 17th- 25th 2022

Agenda item:			6.19.4

Source:	Intel Corporation

Title:	WF on demodulation requirement for Enhancement on HST-SFN deployment

Document for:	Approval

# Background

This contribution is to capture the agreements and the directions for the further studies of performance requirements definition for Rel-17 HST-SFN enhancements.

The following sub-agenda items from e-mail thread [101-bis-e][320] NR_FeMIMO_Demod_NWM are considered:

- Sub-topic 3-5: Test Scope on Enhancement on HST-SFN deployment

- Sub-topic 3-8: Test setup for demodulation requirement for HST-SFN enhancement

The detailed discussions can be found in the e-mail discussion summary [R4-2203118]

# WF on requirements definition for HST-SFN enhancements

## Scope of PDSCH requirements

- Introduce PDSCH requirements for HST SFN scheme A - FFS on HST SFN scheme B - Further evaluate impact on UE receive processing for SFN scheme B - FFS on test design and channel model definition Candidate options: - Option 1: Introduce PDSCH requirements for SFN scheme B. - Option 2: Introduce only PDSCH requirements for SFN scheme A - Option 3: Do not introduce PDSCH requirements for SFN scheme B and define the following test applicability rule to guarantee performance with this scheme: - If UE passes the existing test cases (demodulation requirement for HST-SFN with high Doppler shift), the performance of SFN scheme B is guaranteed - Option 4: Introduce PDSCH requirements for both SFN scheme A and SFN scheme B with introduction of the following test applicability rule: - If UE passes the existing test cases (demodulation requirement for HST-SFN with high Doppler shift), the performance of SFN scheme B is guaranteed

## Scope of PDCCH requirements

Candidate options: - Option 1: Define PDCCH requirements for HST SFN scenario - Option 2: RAN4 discusses and decides whether to still have PDCCH demodulation requirement if intra-slot PDCCH repetition demodulation requirement is agreed to be introduced - Option 3: Do not define any PDCCH requirements for HST scenario but define PDCCH requirements for Scheme A for non-HST scenario. - Option 4: Define test case when both channels (PDSCH/PDCCH) are transmitted using SFN scheme A and verify performance of PDSCH only - Option 5: Do not define PDCCH requirements for HST SFN scenario

## PDSCH CA requirements

Candidate options: - Option 1: Define PDSCH CA requirements for HST SFN scenario - Option 2: Do not define PDSCH CA requirements for HST SFN scenario - Option 3: Define single carrier requirement firstly

## Test Case design for PDSCH requirement for SFN scheme A with Single Carrier

Test setup - Option 1: - Reuse existing Rel-16 HST-SFN test set-up as a baseline - PDCCH/PDSCH/ SFN transmitted from two RRHs - TCI state 1 and TCI state 2 applied for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately Parameter; Value. FDD 15 kHz SCS; TDD 30 kHz SCS. CBW; 10 MHz; 40 MHz. Antenna configuration; 2x2; 2x4. DMRS type; Type 1. Number of DMRS symbols; 1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. TRS configuration; 10ms, 2 slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds =700m; Dmin=150m. Test metric; SNR @70% of maximum throughput. - Other options are not precluded. Maximum Doppler shift - 15 kHz SCS: - Option 1: 972 Hz - Option 2: 870 Hz - Other options are not precluded - 30 kHz SCS: - Option 1: 1667 Hz - Other options are not precluded MCS and Rank - Option 1: MCS 17 with Rank 2 from MCS Table 1 as a starting point - Other options are not precluded Channel Model - Option 1: - HST SFN channel model specified in B.3.2 of TS 38.101-4 reused as a baseline - MCS 13, MCS17 with Rank 2 from MCS Table 1 - Option 2: - Reusing the existing Rel-16 HST-SFN channel model (Ds=700m, Dmin=150m) with removing the two furthest paths corresponding to the two furthest TRP - Option 3: - HST-SFN for PDSCH, PDCCH, DMRS with 2 RRHs per cell deployment - Single TRP Tx for TRS - Other options are not precluded

## Test Case design for PDSCH requirement for SFN scheme B with Single Carrier (If introduced)

MCS and Rank - MCS 17 with rank 2 as a starting point Test setup and Channel Model definition - Option 1: Reuse existing Rel-16 HST-SFN test set-up as a baseline - Two TCI states with QCL A type information, and another one configured QCL type B information - PDCCH/PDSCH/PBCH SFN transmitted from two RRHs - TCI state 1 and TCI state 2 applied for for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately - HST SFN channel model specified in B.3.2 of TS 38.101-4 reused without modelling Doppler shift - Option 2: Reuse existing Rel-16 HST-SFN test set-up as baseline. Reuse the existing Rel-16 HST-SFN channel model (Ds=700m, Dmin=150m) with removing the two furthest paths corresponding to the two furthest TRP Select typical network implementation and consider the network implementation as a part of channel model (i.e., specify the function between the time and the pre-compensation value) to make sure TE implementation of pre-compensation has no impact on the UE performance during the test