3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2203092

# Electronic Meeting, 17th - 25th Jan, 2022

Agenda item:			6.19.4

Source:	Huawei, HiSilicon

Title:	WF on demodulation requirement for Enhancement on Multi-TRP

Document for:	Approval

Introduction

This WF capture all agreements and open issues for topic 3-2, 3-3, 3-6 and 3-7 in [101bis-e][320] NR_FeMIMO_Demod_NWM.

Reference

- R4-2203118, Email discussion summary for [101-bis-e][320] NR_FeMIMO_Demod_NWM, RAN4#101bis-e, Samsung

Topic #3: Enhancement on multi-TRP

Sub-topic 3-2: Test Scope on Multi-TRP enhancement  for PDCCH, PUSCH and PUCCH

Issue 3-2-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

- Option 1:  Yes

- Option 2:  No

Issue 3-2-2: Whether to define PDSCH requirement to verify whether UE is with proper behaviour of rate matching around the two linked PDCCH.

- Option 1:  Yes

- Option 2:  No

Issue 3-2-3: Whether to define PUCCH/PUSCH requirement for multi-TRP repetition transmission schemes

- Agreement

  - Do not define any PUCCH/PUSCH requirement for multi-TRP repetition transmission schemes

Sub-topic 3-3: Test Scope on Multi-TRP inter-cell operation

Issue 3-3-1: Whether to define PDSCH requirement for Multi-TRP inter-cell operation

- Option 1:  Yes

- Option 2:  No

  - Option 2a: Discuss the following alternatives how to guarantee demodulation performance for inter-cell multi-DCI multi-TRP Tx scheme

    - Alt1: Define applicability rule for UE that supports “IntCell-Mtrp” feature that if such UE satisfied Rel-16 minimum requirements for PDSCH multi-DCI based transmission scheme, inter-cell operation can be also guaranteed.

    - Alt2: Add a note to specification that if UE supports “IntCell-Mtrp” feature, minimum requirements for PDSCH multi-DCI based transmission scheme is applicable for this UE, but test configuration (i.e., RRC, SSB) should reflects inter-cell operation mode. Applicability rule between requirements for intra-cell and inter-cell scenarios can be further discussed.

  - Option 2b: Not to define PDSCH/PDCCH demodulation requirement for inter-cell Multi-TRP transmission if intra-slot PDCCH repetition demodulation requirement is agreed to be introduce

Sub-topic 3-6: Test setup for PDCCH requirment for Multi-TRP

Issue 3-6-1: Multi-TRP repetition transmission schemes for PDCCH requirements (if introduced)

- Option 1: Following repetition transmission schemes can be considered

  - FR1 FDM with intra-slot repetition

  - FFS: FR1 TDM with intra-slot repetition

- Other option is not precluded

Issue 3-6-2: Simulation Assumption for PDCCH with FDM repetition scheme (if introduced)

  - Option 1:

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FDD 15 kHz SCS |  | TDD 30 kHz SCS |
| CBW | 10 MHz |  | 40 MHz |
| Antenna configuration | 2x2; 2x4 (2Tx for each TRP) |  |  |
| CORESET RB | 24 | 48 |  |
| CORESET Duration | 2 |  |  |
| Aggregation level | 4, 8 |  |  |
| CCE-REG mapping | Non-interleaved |  |  |
| REG bundle size | 6 |  |  |
| Propagation conditions | TDLA30-10 |  |  |
| Test metric | SNR @1% Probability of missed downlink scheduling grant |  |  |

  - Option 2: Reuse the configuration of the existing cases, such as

    - Test 1 in Table 5.3.2.1.1-1/Table 5.3.3.1.1-1 for 1T2R and 1T4R

    - Test 2 in Table 5.3.2.1.2-1/Table 5.3.3.1.2-1 for 2T2R and 2T4R

  - Other option is not precluded

Sub-topic 3-7: Test setup for PDSCH requirement for inter-cell operation

Issue 3-7-1:  Simulation assumption for PDSCH requirement (if introduced)

- Option 1:  Reusing test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TP1 and TP2 i.e.

  - Time offset/frequency offset: -0.5us /200Hz for FR1 FDD 15kHz SCS; -0.25us/300Hz for FR1 TDD 30kHz SCS

  - RB allocation: frequency non-overlapping

  - MCS: 64QAM 1/2

  - PCI ID: [0] for TP1, [3] for TP2

  - SSB transmission: SSB 1 for TP1, SSB 2 for TP2

- Option 2:

  - RB allocation: frequency overlapping

- Other Option is not precluded