3GPP TSG-RAN WG4 Meeting # 101-bis-e	                                                              R4-2203105

Electronic Meeting, January 17-25, 2022

Agenda item:			6.8.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [101-bis-e][313] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 6.8.3.1~6.8.3.2. Two topics are included in total, including PDSCH requirements for CA scenarios and enhanced transmission schemes. The agreed way forward in previous meeting is in R4-2120700.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  6.8.3.1 and 6.8.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2201422 | Summary for FR1 HST demodulation results | Ericsson |  |
| R4-2200267 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Do not introduce additional UE capability for HST-SFN CA. Proposal #2: Rel-17 FR1 HST PDSCH demod requirements for CA are defined as release independent if RRM requirements are agreed to be release independent and align the srelease number of demodulation requirements with RRM requirements |
| R4-2200268 | Draft CR on HST DPS CA requirements for 4Rx | Apple |  |
| R4-2200364 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Proposal 1: Define the new Rel-17 UE capability to indicate the support of HST-SFN CA |
| R4-2200521 | Discussion on HST FR1 CA PDSCH performance requirements | Intel Corporation | Proposal #1: Define per-UE Rel-17 UE capability to indicate the support of HST-SFN CA. Proposal #2: Define HST-DPS CA requirements in release independent from Rel-15. Proposal #3: Apply HST-SFN CA requirements only from Rel-17. Proposal #4: Do not define HST-SFN CA network assistance signalling. |
| R4-2200625 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC |  |
| R4-2200633 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Observation 1: if HST-SFN + CA ≠ HST-SFN CA is a valid case, it means that it is not suitable to reuse the UE capability (demodulationEnhancement-r16) introduced for HST single carrier case to indicate the support of HST-SFN CA. Proposal 1: it is proposed to define a new UE capability to indicate the support of HST-SFN CA. Proposal 2: the new UE capability for HST-SFN CA is proposed to be a per-UE capability. Proposal 3: it is proposed that Rel-17 FR1 HST PDSCH CA requirements, including HST-DPS CA requirements and HST-SFN CA requirements, are release independent from Rel-15. |
| R4-2200835 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Proposal 1 :To introduce UE capability for HST-SFN CA. Proposal 2 :Option 1b is preferred for UE capability of HST-SFN CA. Proposal 3 :To support option 1 or option 3 for release independent feature. |
| R4-2200998 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Proposal 1: Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17, that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier. Proposal 2: Report per band combination to allow UE to report the supporting HST-SFN CA in some band combinations. Proposal 3: Send LS to RAN2 to inform that Rel-16 signalling already covers SFN demodulation CA case and new feature is defined for Rel-17 for SFN demodulation CA that is different from Rel-16. Proposal 4: HST-DPS CA requirements are release independent from Rel-15 and HST-SFN CA requirements are applicable from Rel-17. |
| R4-2200999 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon |  |
| R4-2201423 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: Introduce a new per-UE capability to indicate whether the UE is capable of supporting the PDSCH demodulation requirements with HST-SFN CA defined in Rel-17. Proposal 2: For the PDSCH demodulation requirements for FR1 HST defined in Rel-17, • HST-DPS CA requirements are release independent from Rel-15, • HST-SFN CA requirements are applicable for UE capable of the new Rel-17 HST CA demodulation capability. |
| R4-2201424 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson |  |
| R4-2201876 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel Corporation |  |
| R4-2202006 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Proposal 1: Use same rule for release independence of demod requirements, as agreed in RRM requirements. |
| R4-2200641 | Discussion on UE capability for FR1 HST enhancement | CMCC |  |
| R4-2200544 | Discussion on Rel-17 RAN4 UE feature list | Intel |  |

Open issues summary

Issue 1-1: UE capability for HST-SFN CA

- Agreements in RAN4#101-e meeting:

- Option 1: define a new Rel-17 UE capability to indicate the support of HST-SFN CA

- 	Option 1a: the granularity of the capability is a per-UE

- 	Option 1b: the granularity of the capability is per band combination

- 	Option 1c: the granularity of the capability is per band

- Option 2: Do not introduce additional UE capability for HST-SFN CA.

- Proposals in RAN4#101-bis-e meeting:

  - Option 1 (DOCOMO): define a new UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC, Intel, Ericsson): the granularity of the capability is a per-UE

    - 1b (ZTE, Huawei): the granularity of the capability is per band combination

  - Option 2 (Apple,): Do not introduce additional UE capability for HST-SFN CA.

- Recommended WF

  - Define a new UE capability to indicate the support of HST-SFN CA

  - Discuss the granularity of the capability in 1st round

Issue 1-2: UE feature list for FR1 HST demodulation

- Proposal in R4-2200641:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x-x | UE capability for demodulation for FR1 HST-SFN CA | UE processing for HST-SFN CA, as specified in TS 38.101-4 |  | FFS |  |  | FFS | NO | FR1 only |  |  | FFS |

- Proposal in R4-2200544:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | Rel-16 RAN4 feature 10-2 | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |

- Recommended WF

  - Excepted for the “Type” of this feature discussed in issue 1-1, please provide your comments on other parts of this feature and the FFS in the above table

  - The target is to agree on the feature list for FR1 HST demodulation after 1st round and capture it in UE feature list.

Issue 1-3: Release independent

- Agreements in RAN4#101-e meeting:

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

- Option 2:

  - HST-DPS CA requirements are release independent from Rel-15

  - HST-SFN CA requirements are applicable from Rel-17

- Option 3:

  - Align the release number of demodulation requirements with RRM requirements.

  - No more release independent discussion for demodulation requirements.

- Proposals in RAN4#101-bis-e meeting:

- Option 1 (CMCC, ZTE): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15.

- Option 2 (Intel, ZTE, Huawei, Ericsson):

    - HST-DPS CA requirements are release independent from Rel-15

    - HST-SFN CA requirements are applicable from Rel-17

- Option 3 (Apple, Qualcomm): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined as release independent from Rel-15.

- Recommended WF

  - Option 3

Issue 1-4: Network assisted signalling

- Proposals in RAN4#101-bis-e meeting:

  - Option 1 (Intel): Do not define HST-SFN CA network assistance signalling.

  - Option 2 (Huawei): Send LS to RAN2 to inform that Rel-16 signalling already covers SFN demodulation CA case and new feature is defined for Rel-17 for SFN demodulation CA that is different from Rel-16.

- Recommended WF

  - Do not define HST-SFN CA network assistance signalling

  - Send LS to RAN2 to inform that Rel-16 signalling already covers SFN demodulation CA case and new feature is defined for Rel-17 for SFN demodulation CA that is different from Rel-16.

    - Comments are welcome on the exact wording to inform RAN2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 1-1: UE capability for HST-SFN CA Option 1a. As some companies mentioned in previous meetings, considering the complex processing, power consumption, processing resource and buffering for HST-SFN CA for NR, depending the real product implementation, it is possible case that HST-SFN + CA ≠ HST-SFN CA From this point of view, considering different UE implementation, we are OK to introduce UE capability for HST-SFN CA. As for the granularity of this capability, since the main concern is about the power consumption, processing resource and buffering, which is more like a baseband limitation. From this point of view, it could be a per-UE capability. Issue 1-2: UE feature list for FR1 HST demodulation Pending on the conclusion of Issue 1-1. Issue 1-3: Release independent Both option 1 and option 3 are OK for us. Issue 1-4: Network assisted signalling Firstly, we would like to provide some background. In RAN4 #98-bis-e, in the email thread of FR1 HST demodulation enhancement, we already discussed the issue on network signalling, and companies noticed that existing HST network assisted signaling can cover CA scenario. We had following agreements. Agreements in R4-2106098 - Network-assisted signalling - Existing HST network assisted signaling can cover CA scenario. In last meeting, RRM session discussed the network signalling. Since there is no conclusion on whether to reuse Rel-16 network indication or to introduce new R17 network indication, it was agreed that it is up RAN2 decision on how to define signalling. The common understanding is to inform RAN2 that Rel-16 signalling already covers CA case and inform that Rel-17 we have new feature for RRM CA. It is up RAN2 decision on how to define signalling. In summary, the current situation is that RRM experts and demodulation experts have different approach to handle the RAN 2 over-design issue for Rel-16. From our point of view, either way is OK for us, the key point is that in RAN4 spec, it is clearly stated with which network indication, the requirements are applied. And we would like to hear companies’ view on this. If companies prefer to send LS to RAN2, we are also fine. And the exact wording is suggested as following, which is aligned with the wording of RRM LS to RAN2(R4-2120286). Suggested wording of the LS to RAN2 - RAN4 notices that the IE highSpeedDemodFlag-r16 is signalled per serving cell basis in both ServingCellConfigCommonSIB and ServingCellConfigCommon, however the Rel-16 HST WI only considers single carrier scenario. The enhancement of CA requirements is under discussion in Rel-17 NR FR1 HST, and RAN4 agreed that network needs to inform UE whether to apply the enhanced PDSCH requirements for CA specified in TS38.101-4. The signalling design is up to RAN2. |
| Ericsson | Issue 1-1: UE capability for HST-SFN CA Option 1a. We are ok with the recommended WF. Regarding the granularity, we prefer per-UE capability same as RRM. Since HST-SFN is only for FR1 and this advanced receiver should be independent of operating bands, we think per-UE capability is sufficient. We would like to listen the reason from the proponents of per-BC capability why this feature should be per band combination. Issue 1-2: UE feature list for FR1 HST demodulation We are ok to introduce a new UE feature corresponding to demodulation for FR1 HST-SFN CA. In our understanding, same as single carrier case, gNB should know whether UE support this feature and it should be optional with capability signalling. Issue 1-3: Release independent Our proposal is the combination of options 2 and 3: - If Rel-17 RRM requirements for HST are defined as release independent from Rel-15, Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, - Otherwise, Rel-17 FR1 HST PDSCH CA requirements are applicable from Rel-17 - HST-DPS CA requirements are release independent from Rel-15 Issue 1-4: Network assisted signalling Support the recommended WF. We can reuse ‘R4-2120286 LS on signalling for RRM enhancements for NR FR1 HST’ agreed in RAN4#101. We are fine with the wording suggested by CMCC above. We can also inform the UE capability for PDSCH demodulation for HST-SFN CA if RAN4 agree in this meeting. |
| Intel | Issue 1-1: UE capability for HST-SFN CA We are fine to define specific HST-SFN CA capability signalling to not mandate implementation of advancer HST-SFN receiver for CA operation. As for granularity, we prefer to define such capability per-UE basis. If UE does not have ability to process large aggregated CBW with advanced receiver, we do not think it will process it with conventional receiver. According to our assessments for previous RAN4 meeting, there is no big difference in terms of processing complexity between conventional receiver and HST-SFN receiver. Issue 1-2: UE feature list for FR1 HST demodulation In the issue 1-2 a feature for RRM enhanced requirements was coped from our contribution (R4-2200544). We updated the issue to clarify our proposal for demod. We encourage other companies to double check correct proposal and provide view. Issue 1-3: Release independent We cannot accept definition of HST-SFN CA requirements from Rel-15/16 if Rel-17 capability signaling is introduced. It is to late (in Rel-17 time frame) to define requirements with advanced processing from Rel-15. Same time, definition of requirements from Re-17 does not preclude early UE implementation. Support Option 2. Issue 1-4: Network assisted signalling Do we really need to shift responsibility of network assistance signaling definition to RAN2? As CMCC pointed out, RAN4 has already agreed that existing network assistance can be used. We do not see any technical value to define additional assistance. In HST-SFN deployments if UE is scheduled with CA and UE signals HST-SFN CA capability, this UE can always assume that HST-SFN Tx scheme is used on all CC. No need to signal the same information in addition. |
| Qualcomm | Issue 1-1: UE capability for HST-SFN CA We are neutral on whether to define this capability. However, if it were to be defined, it should be per band combination because available UE resources will be determined by number of bands and their aggregated BW. For example, UE may be able to support 3CC case but not 4CC because latter will require larger number of resources for HST-SFN processing. Issue 1-2: UE feature list for FR1 HST demodulation Depends on the outcome of Issue 1-1. Proposal in R4-2200544 looks ok except Type. Issue 1-3: Release independent Ok with Ericsson’s proposal. Issue 1-4: Network assisted signalling Ok with recommended WF. Wording can be discussed after decision on Issue 1-1. |
| DCM | Issue 1-1: UE capability for HST-SFN CA We support Option 1. As for the granularity of the capability, we are neutral. Issue 1-3: Release independent We are ok with the recommended WF. |

| Apple | Issue 1-1: UE capability for HST-SFN CA Our preference is not to introduce such UE capability. But given that majority companies see the need to introduce it, we are fine to compromise and support defining UE capability. The purpose of this capability is for UE to indicate whether it can support HST-SFN CA in the max aggregated BW, it should be a per BC indication rather than per UE. Defining it with per UE granularity would mean that UE supports it in all band combinations or neither. Issue 1-2: UE feature list for FR1 HST demodulation Depends on Issue 1-1. Issue 1-3: Release independent We support the recommended WF and also proposal from Ericsson. Issue 1-4: Network assisted signalling We support the recommended WF |
| --- | --- |

| Huawei | Issue 1-1: UE capability for HST-SFN CA It is unreasonable to do the simple addition operation with implementation of more and more features introduced in the future and assume a Rel-15/16 UE must support all Rel-17 features. Also considering UE capability for HST-SFN CA has been introduced in RRM part, similarly, we propose to define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17, that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier. For the reporting granularity, we prefer to report per band combination to allow UE to report the supporting HST-SFN CA in some band combinations, considering baseband limitation such as power consumption, processing resource and buffer limitation for some band combinations. Issue 1-2: UE feature list for FR1 HST demodulation This issue is pending on Issue 1-1. Issue 1-3: Release independent We prefer Option 2. Issue 1-4: Network assisted signalling We are OK with the recommended WF and the wording for LS proposed by CMCC. |
| --- | --- |
| ZTE | Issue 1-1: UE capability for HST-SFN CA Option 1 and option 1b is preferred If HST SFN CA is supported, HST SFN CA should finally be reflected in the specific band combination. If the granularity is per band combination, UE will have the chance to report which band combination is supported for HST SFN CA. Issue 1-2: UE feature list for FR1 HST demodulation This issue can be discussed after issue 1-1 is determined Issue 1-3: Release independent We are fine with option 1 and option 3. Issue 1-4: Network assisted signalling We do not see the necessity of additional network signaling and we support the recommended WF. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |
| R4-2200625 Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx CMCC | Ericsson: Add test purpose like R4-2201876. |
|  | Intel: 1. Need to align main section title (CA or carrier aggregation) between different CRs. 2. Need to add test purpose. 3. Need to add test parameters since they are different from normal CA test parameters. 4. Need to align RMC indices once CR for RMC becomes stable. 5. There is a wrong reference in the note #1 in Table 5.2A.3.x-3 |

| CR tdoc number | Comments collection |
| --- | --- |
| R4-2200999 Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) Huawei,HiSilicon | Ericsson: Test parameters and test purpose is missing. |
|  | Intel: 1. Need to align main section title (CA or carrier aggregation) between different CRs. 2. Need to add test purpose. 3. Need to add test parameters since they are different from normal CA test parameters. 4. Need to align RMC indices once CR for RMC becomes stable. 5. Propagation conditions and antenna configuration should be aligned with SC requirements. |
|  | Qualcomm: Simulation results are not collected for FDD 35 and 45MHz. We are not sure where those numbers came from. SNR numbers for FDD 35-50MHz seem wrong. We suggest to leave SNRs for 35 and 45MHz as TBD and companies can submit simulation results in the next meeting. |
|  | Ericsson2: The reason we do not collect FDD 35/45MHz for Rel-17 HST CA is because these CBWs are introduced in another Rel-17 WI NR_FR1_35MHz_45MHz_BW, and the agreement was to define PDSCH demodulation requirements for normal PDSCH CA scenario only. We are open to include FDD 35/45MHz also for HST-SFN/HST-DPS CA if companies are interested in. In this case we need to revise FRC also. |
|  | Huawei: Thanks for your correction, new version will be uploaded to fix above issues. 20220119 Updated: We have upload the new version into the “Revised CR” folder. |

| CR tdoc number | Comments collection |
| --- | --- |
| R4-2201424 draft CR: FRC for CA PDSCH demodulation requirements for HST Ericsson | Ericsson: Need to update the Table title and FRC tags. According to TS38.101-4 V17.3.0, Use R.PDSCH.1-13.X FDD for HST-SFN FDD 15kHz FRC, where X=1, … Use R.PDSCH.1-14.Y FDD for HST-DPS FDD 15kHz FRC, where Y=1, … Use R.PDSCH.2-19.Z TDD for HST-SFN TDD 30kHz FRC, where Z=1, … Use R.PDSCH.2-20.A TDD for HST-DPS TDD 30kHz FRC, where A=1, … |

| CR tdoc number | Comments collection |
| --- | --- |
| R4-2201876 draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx Intel Corporation |  |

| CR tdoc number | Comments collection |
| --- | --- |
| R4-2200268 Draft CR on HST DPS CA requirements for 4Rx Apple | Qualcomm: This Tdoc is not available. |
|  | Apple: We withdrew based on the guidance/meeting arrangements that only Draft TPs were allowed for demod enh WI under demod WIs |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Issue 1-1: UE capability for HST-SFN CA

  - Option 1 (DOCOMO): define a new UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC, Intel, Ericsson): the granularity of the capability is a per-UE

    - 1b (ZTE, Huawei, Qualcomm, Apple): the granularity of the capability is per band combination/feature set

Agreement: Introduce a new capability for supporting HST-SFN CA in FR1

- The granularity: [per band combination]

Recommended WF for 2nd round: confirm the above agreements

Issue 1-2: UE feature list for FR1 HST demodulation

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | [Rel-16 RAN4 feature 10-2] | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | [per band combination] | No | FR1 only | N/A |  | Optional with capability signalling |

Recommended WF for 2nd round: Discuss the prerequisite feature groups.

Companies need to check: If UE indicates support of Rel-17 enhanced demodulation requirements for CA in HST SFN FR1, it means that UE can support enhanced demodulation requirements for single carrier. UE does not need to indicate Rel-16 feature 10-2.

Issue 1-4: Network assisted signalling

Agreement

  - No need to define new HST-SFN CA network assistance signaling from RAN4 perspective

  - Send LS to RAN2 to inform that Rel-16 signaling already covers SFN demodulation CA case and new feature is defined for Rel-17 for SFN demodulation CA that is different from Rel-16.

Suggested wording of the LS to RAN2 - RAN4 notices that the IE highSpeedDemodFlag-r16 is signalled per serving cell basis in both ServingCellConfigCommonSIB and ServingCellConfigCommon, however the Rel-16 HST WI only considers single carrier scenario. The enhancement of CA requirements is under discussion in Rel-17 NR FR1 HST, and RAN4 agreed that network needs to inform UE whether to apply the enhanced PDSCH requirements for CA specified in TS38.101-4. The signalling design is up to RAN2

Recommended WF for 2nd round: Discuss the LS in 2nd round based on above agreements.

Issue 1-3: Release independent

Agreement:

- HST-DPS CA requirements are release independent from Rel-15

- For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements

  - Option 1: Applicable from Rel-17

  - Option 2: Aligned with RRM agreement

  - Option 3: Release independent from Rel-15 /16

Recommended WF for 2nd round: Discuss the above options in 2nd round.

Discussion on 2nd round (if applicable)

Open issues summary

Issue 1-1: UE capability for HST-SFN CA

  - Option 1 (DOCOMO): define a new UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC, Intel, Ericsson): the granularity of the capability is a per-UE

    - 1b (ZTE, Huawei, Qualcomm, Apple): the granularity of the capability is per band combination/feature set

Agreement: Introduce a new capability for supporting HST-SFN CA in FR1

- The granularity: [per band combination]

Recommended WF for 2nd round: confirm the above agreements

Issue 1-2: UE feature list for FR1 HST demodulation

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | [Rel-16 RAN4 feature 10-2] | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | [per band combination] | No | FR1 only | N/A |  | Optional with capability signalling |

Recommended WF for 2nd round: Discuss the prerequisite feature groups.

Companies need to check: If UE indicates support of Rel-17 enhanced demodulation requirements for CA in HST SFN FR1, it means that UE can support enhanced demodulation requirements for single carrier. UE does not need to indicate Rel-16 feature 10-2.

Issue 1-3: Release independent

Agreement:

- HST-DPS CA requirements are release independent from Rel-15

- For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements

  - Option 1: Applicable from Rel-17

  - Option 2: Aligned with RRM agreement

  - Option 3: Release independent from Rel-15 /16

Recommended WF for 2nd round: Discuss the above options in 2nd round.

Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1: UE capability for HST-SFN CA We confirm the above agreements. Issue 1-2: UE feature list for FR1 HST demodulation We prefer to not link Rel-17 HST SFN CA feature and Rel-16 HST SFN feature together and UE does not need to indicate Rel-16 feature 10-2 considering that they are related to different UE capability. We can assume that UE support Rel-17 HST SFN CA feature can also support Rel-16 HST SFN single carrier feature by default. Issue 1-3: Release independent We prefer Option 1 considering that Rel-15/16 UE supporting HST single carrier should not be mandated to support HST CA. Since the per BC UE capability is defined, we can compromise to define HST SFN CA requirements release independent from Rel-16. |
| Ericsson | Issue 1-1: UE capability for HST-SFN CA We are ok with the agreements Issue 1-2: UE feature list for FR1 HST demodulation We prefer the following assumption: If UE indicates support of Rel-17 enhanced demodulation requirements for CA in HST SFN FR1, it means that UE can support enhanced demodulation requirements for single carrier. UE does not need to indicate Rel-16 feature 10-2 This assumption can avoid the discussion how to interpret the scenario when UE does not indicate Rel-16 feature 10-2 but it indicates this Rel-17 feature on HST-SFN CA. However we are also fine not to require Rel-16 feature 10-2 as the precondition as far as it is clarified the interpretation of the case, somewhere in the specification, when UE does not indicate Rel-16 feature 10-2 but it indicates this Rel-17 feature on HST-SFN CA. |
| Apple | Issue 1-2: UE feature list for FR1 HST demodulation If UE indicates capability for HST SFN CA it would mean that it can support single carrier. UE can indicate then separately, but it need not be a pre-requisite. But we don’t need to add any note that UE need not indicate capability for HST SFN single carrier if it supports HST SFN CA. Issue 1-3 After discussion in GTW and with introduction of new UE capability, defining requirements as release independent from Rel-15 doesn’t seem feasible. Demod requirements can be release independent then from Rel-16 but not earlier. |
| Qualcomm | Issue 1-1: UE capability for HST-SFN CA We confirm the agreements. Issue 1-2: UE feature list for FR1 HST demodulation We are ok with Ericsson’s suggestion. As long as it is clarified somewhere in the spec, we are ok. Issue 1-3: Release independent We are ok to make HST-CA requirements release independent from Rel-15 as long as the new UE capability is early implemented in RAN2. |
| Apple2 | Issue 1-2: UE feature list for FR1 HST demodulation Rather than capturing the clarification in spec it may be better to have pre-req FG as 10-2. There seems no harm in doing so. In RRM session there was agreement to include pre-req FG for CA enh. |
| CMCC | Issue 1-1: UE capability for HST-SFN CA We are OK with the Agreement Issue 1-2: UE feature list for FR1 HST demodulation We would like to provide some information on the RRM related discussion. For UE capability on FR1 HST RRM enhancement, it was agreed to use Rel-16 UE capability as prerequisite feature groups. On the other hand, according to companies’ comments, companies share the same view that if UE indicates support of Rel-17 enhanced demodulation requirements for CA in HST SFN FR1, it means that UE can support enhanced demodulation requirements for single carrier. In our understanding, this is the same meaning as that we use Rel-16 feature 10-2 as prerequisite feature groups. Taking above consideration into account, it is better to use Rel-16 feature 10-2 as prerequisite feature groups. Issue 1-3: Release independent For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements, prefer to be release independent from Rel-15. And we do not see any issues to have this release independent by using the approach of early implementation. |
| Intel | Issue 1-1: UE capability for HST-SFN CA In our understanding per band combination granularity relates to RF features. The target of this capability is to address potential baseband limitations such as power consumption, processing resource and buffer limitations that are limited not only by total aggregated CBW but also by other baseband related parameters like modulation, rank. In this case the general procedure is to use per feature set granularity. We encourage companies to check applicability of feature set and band combination granularities. At current stage we prefer to remove per band combination from tentative agreement, leave granularity field empty and capture two options for further discussion as per feature set and per band combination. Issue 1-2: UE feature list for FR1 HST demodulation It is common RAN4 understanding that if UE supports HST-SFN CA it definitely supports HST-SFN SC. From specification perspective, we should clarify this to avoid misinterpretations of situations when UE signals only HST-SFN CA support as Ericsson mentioned. The most straightforward way is to define HST-SFN SC as a pre-requisite feature group for HST-SFN CA. Issue 1-3: Release independent We are fine to compromise to apply HST-SFN CA requirements from Rel-16. In this case we need to inform RAN2 than early UE implementation procedure is assumed for HST-SFN CA requirements with Rel-17 capability signaling. |
| ZTE | Issue 1-1: UE capability for HST-SFN CA We support to confirm the agreement Issue 1-2: UE feature list for FR1 HST demodulation We are fine with the Prerequisite. Issue 1-3: Release independent We prefer to make HST-SFN CA requirements release independent from Rel-15 and we can compromise to define HST-SFN CA requirements release independent from Rel-16. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |
| Revised R4-2200999 Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) Huawei,HiSilicon | Apple: We suggest separating the requirements for 1 TCI and 2 TCI states into 2 tables since only either of them for the corresponding CBW would apply and for better readability. |
|  | Thanks for your suggestion, we have uploaded a new draft version to the inbox to apply it. |

| CR tdoc number | Comments collection |
| --- | --- |
| Revised R4-2201876 draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx Intel Corporation | Apple: We see rows for 35 and 45 MHz, but they are not there for other requirements. We either add these later based on whether we agree to define for these additional CBW or keep the SNRs as TBD. Qualcomm: We prefer to remove rows for 35 and 45MHz at this point since WI for those additional CBW had the agreement to only introduce normal CA requirements. Please also correct the SNR for FDD 50MHz and TDD 80MHz to [14.0] and [14.9]dB, respectively. |
| draft CR: FRC for CA PDSCH demodulation requirements for HST, Ericsson | Qualcomm: For better readability, we think it will be good to update Table titles to reflect which HST scheme those tables are for. Ericsson2: Thanks for your suggestion. We have updated the table title as follows: Table A.3.2.1.1-13 and 14: PDSCH Reference Channel for FDD HST-SFN with CA scenario Table A.3.2.1.1-15 and 16: PDSCH Reference Channel for FDD HST-DPS with CA scenario Table A.3.2.2.2-19, 20 and 21: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-SFN with CA scenario Table A.3.2.2.2-22, 23, and 24: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-DPS with CA scenario |
| Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx, CMCC | Qualcomm: SNR requirements do not match with the simulation results table in this meeting. Please update the SNRs based on the updated simulation results table in this meeting. CMCC: the SNR values will be updated accordingly. Thanks! |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 HST demodulation | CMCC |  |
| LS on network assisted signaling for FR1 HST CA demodulation | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200625 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC | Revise based on 1st round comments |  |
| R4-2200999 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon | Revise based on 1st round comments |  |
| R4-2201424 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson | Revise based on 1st round comments |  |
| R4-2201876 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel Corporation | Return to |  |

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2201422 | Summary for FR1 HST demodulation results | Ericsson | Noted |  |
| R4-2200267 | Discussion on PDSCH CA Requirements in HST | Apple | Noted |  |
| R4-2200268 | Draft CR on HST DPS CA requirements for 4Rx | Apple | Withdraw |  |
| R4-2200364 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Noted |  |
| R4-2200521 | Discussion on HST FR1 CA PDSCH performance requirements | Intel Corporation | Noted |  |
| R4-2200633 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Noted |  |
| R4-2200835 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Noted |  |
| R4-2200998 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Noted |  |
| R4-2201423 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Noted |  |
| R4-2202006 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Noted |  |
| R4-2200641 | Discussion on UE capability for FR1 HST enhancement | CMCC | Noted |  |
| R4-2200544 | Discussion on Rel-17 RAN4 UE feature list | Intel | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2202985 | WF on FR1 HST demodulation | CMCC | Agreeable |  |
| R4-2202984 | LS on network assisted signaling for FR1 HST CA demodulation | CMCC | Agreeable |  |

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2202981 revised from R4-2200625 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC | Agreeable |  |
| R4-2200625 revised from R4-2200999 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei, HiSilicon | Agreeable |  |
| R4-2202983 revised from R4-2201424 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson | Agreeable |  |
| R4-2203123 revised from R4-2201876 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel Corporation | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)