3GPP TSG-RAN WG4 Meeting #101-bis-e 												R4-2203107

Electronic Meeting, 17th Jan 2022 - 25th Jan 2022

Agenda item:			6.9.5.3

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [101-bis-e][315] NR_HST_FR2_Demod_Part2

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

## Scope

This T-doc will be used to guide and summarize the email discussion for the topic of RRel-17 NR FR2 HST BS Demod requirements (AI 6.9.5.3), with the email thread identifier “[101-bis-e][315] NR_HST_FR2_Demod_Part2”.

The scope of this email discussion are the Rel-17 NR FR2 HST BS Demod requirements, and in particular the agenda items:

6.9.5.3	BS demodulation requirements

6.9.5.3.1	PUSCH requirements

6.9.5.3.2	PUSCH with UL timing adjustment requirements

6.9.5.3.3	PRACH requirements

Priority topics are marked directly in the open issues’ summaries.

## Notes on email discussions

From the previous meeting arrangements:

- Delegates are strongly encouraged to provide comments/concerns asap - Silence within a reasonable timeframe means no objection - It is strongly encouraged that each company/delegate consolidate their comments/views and send them out in one email for each email thread - Length of file names shall be reduced, e.g. - At the beginning of first round, moderators share / ftp / tsg_ran / WG4_Radio / TSGR4_98_e / Inbox / Drafts / [98e][101] NR_NewRAT_SysParameters\Summary_101_1st round_v01.docx - After update by company A: Summary_101_1st round_v02_companyA - After update by company B: Summary_101_1st round_v03_companyA_companyB - After update by company C: Summary_101_1st round_v04_companyB_companyC

# Topic #1: PUSCH requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2200128 | CATT | Title: Discussion on PUSCH demodulation requirements for FR2 HST Proposal 1: To adopt the following manufacturer declaration for different additional DM-RS position for FR2 HST. Declaration identifier; Declaration; Description; Applicability (Note 1). BS type 1-H(Note 2); BS type 1-O; BS type 2-O. D.x; Additional DM-RS position for FR2 high speed train; Declaration of supported additional DM-RS position for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos2, pos3.; n/a; n/a; x. Proposal 2: To adopt the following test applicability rules for different additional DM-RS position for FR2 HST. Unless otherwise stated, tests shall apply for each additional DM-RS position declared to be supported for FR2 high speed train scenario for PUSCH and UL timing adjustment, If both pos2 and pos3 are declared to be supported, the tests shall be done for pos2 (see D.x in table 4.6-1). Proposal 3: To adopt MCS16 or MCS20. |
| R4-2200129 | CATT | Title: Simulation results for PUSCH demodulation requirements for FR2 HST No proposals, Simulation Tdoc. |
| R4-2200747 | Samsung | Title: Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST Test Applicability rule for RS configuration Observation 1: The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes. Observation 2: Similar performance can be achieved for 2 DMRS configuration and 3 DMRS configuration. Observation 3: Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration. Proposal 1: FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If the all the DMRS configuration or subset of DMRS configuration declared to be supported, the test shall be done for the minimum number of DMRS supported, or only one of supported DMRS configuration based on BS manufacture declaration. DMRS configuration declared to be supported; PUSCH requirement tests. {pos0, pos1, pos2}; Option 1: either test with pos0, or pos1,or pos2 only based on BS declaration Option 2: only test for pos0. {pos0, pos1}; Option 1: either test with pos0 or pos1 only based on BS declaration Option 2: only test for pos0. {pos0, pos2}; Option 1: either test with pos0 or pos2 only based on BS declaration Option 2: only test for pos0. {pos1,pos2}; Option 1: either test with pos1 or pos2 only based on BS declaration Option 2:only test for post1. {pos0}, or {pos1}, or {pos2}; Only test DMRS declared to be supported. D.1XX; PUSCH additional DM-RS positions; Declaration of the supported additional DM-RS position(s) for FR2 high speed train scenario, i.e., {pos0},{pos1},{pos2},{pos0,pos1}, {pos0 pos1}, {pos0,pos2}, {pos1,pos2} and all; n/a; n/a; x. FOE implementation and MCS Proposal 2: Both MCS 16 and MCS17 are feasible for FR2 HST PUSCH requirements Initial Simulation Results Observation 4: Around 2 or 3 dB difference between post-FFT and pre-FFT FOC methods for MCS 16 and MCS17 Observation 5: Around 7dB difference between post-FFT and pre-FFT FOC methods for MCS 20 Observation 6: Requirement with MCS16 to MCS 20 are feasible and can be testable, for both Post-FFT and Pre-FFT FOC operation. |
| R4-2201006 | Huawei, HiSilicon | Title: Discussion on PUSCH demodulation requirements for FR2 HST RS configurations for PUSCH and UL timing adjustment - Add a note to the performance requirements as following to ensure that only one case is tested. - Either pos 1, pos 2 or pos 3 may be used for the test FRC based on BS manufacturer declaration. A pass with either of these possibilities is sufficient to demonstrate compliance to the core requirement. - FOC implementation and MCS - Define performance requirements with only MCS20. |
| R4-2201522 | Ericsson | Title: HST PUSCH requirements Proposal 1: Create a single requirement based on [MCS20] assuming a post FFT receiver. |
| R4-2201844 | Nokia, Nokia Shanghai Bell | Title: On HST FR2 PUSCH Requirements On RS configuration for PUSCH and UL timing adjustment - Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario, i.e., pos0, or pos1/pos2, or both, applicable to BS Type 2-O. Define applicability rule in such a way that the requirements shall apply only for the supported DM-RS. If several DM-RS positions are supported, then only one of them shall be tested. On PUSCH performance and MCS selection - Based on our simulation results we do not observe a significant difference in pre-FFT and post-FFT FOC. SNR corresponding to 70% of maximum throughput for MCS 20 is around 10 dB, what is far from the limits of the test equipment. RS configuration; MCS; SNR at 70%maxTP, 50MHz; SNR at 70%maxTP, 200MHz. DMRS 1+0; MCS16; 6.72; 6.64. MCS17; 7.42; 7.22. MCS18; 7.94; 7.73. MCS19; 9.12; 8.85. MCS20; 10.15; 9.86. DMRS 1+1; MCS16; 6.57; 6.58. MCS17; 7.18; 7.20. MCS18; 8.06; 7.75. MCS19; 8.94; 8.71. MCS20; 10.16; 9.80. DMRS 1+1+1; MCS16; 6.48; 6.61. MCS17; 7.21; 7.23. MCS18; 7.71; 7.71. MCS19; 8.73; 9.02. MCS20; 10.17; 10.02. Use simulation results from Table 1 above for the definition of PUSCH performance requirements in HST FR2. Moderator: Proposal 3 has not been included in the summary. Please include the simulation results in the simulation result summary. RAN4 to define PUSCH requirements with two MCSs: MCS16 (16QAM) and MCS 20 (64QAM). RAN4 to define an applicability rule, that the PUSCH performance requirements needs to be tested only with highest supported MCS. If a large span in the PUSCH simulations results still persists, RAN4 to verify and compare the PUSCH perfromance in a simpler channel model: AWGN + fixed maximum Doppler offset of 19458 Hz. |
| R4-2201878 | Intel Corporation | Title: Views on FR2 HST PUSCH performance requirements Proposal #1: Define PUSCH requirements with MCS 16. Proposal #2: Define test applicability rule to allow testing with only one of the supported DMRS configuration. Proposal #3: Define manufacturer declaration on the supported DMRS configuration as {pos0, pos1, pos2 or any combination of them}. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

The agreements from the previous meetings are recorded in the corresponding WFs, i.e., R4-2115725, R4-2115726, and R4-2120703, respectively.

### Sub-topic 1-1: Test applicability rules and manufacturer declarations

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: Test applicability

- Prior agreement RAN4#100e [R4-2115726]

  - Test setup for UL timing adjustment requirements

    - RS configuration

      - Align RS configuration for UL timing adjustment requirement and PUSCH requirement

- Prior agreement RAN4#101e [R4-2120703]

  - RS configurations for PUSCH and UL timing adjustment

    - Define requirement with 1 DMRS + PT_RS (L=1, K=2) configuration

      - Define FRC for 1 DMRS + PT_RS (L=1, K=2)

    - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS+ PT_RS (L=1, K=2) and 3 DMRS + PT_RS (L=1, K=2)

      - Define FRC for 2 DMRS + PT_RS (L=1, K=2)

      - Define FRC for 3 DMRS + PT_RS (L=1, K=2)

    - Way forward

      - FFS, definition of test applicability rules based on BS manufacturer declaration

- Proposals

  - Option 1 (Samsung, Nokia, Intel): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, only one of supported DMRS configuration shall be tested based on BS manufacture declaration.

  - Option 2 (Samsung, Intel, Nokia, Ericsson, CATT): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported.

  - Option 3 (Huawei): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, a pass with either of the possibilities is sufficient to demonstrate compliance to the core requirement. Capture this in a note to the performance requirement.

  - Option 4 (CATT): Tests shall apply for each additional DM-RS position declared to be supported for FR2 high speed train scenario for PUSCH and UL timing adjustment, If both pos2 and pos3 are declared to be supported, the tests shall be done for pos2

  - Option X: Other options not precluded.

- Recommended WF

  - Please discuss in first round.

  - Moderator remark on option 1: If this option is chosen an additional manufacturer declaration to select the test is required.

| Company | Comments |
| --- | --- |
| Samsung | In Rel-15 BS demod requirement for FR2, both 1 DMRS and 2 DMRS configuration are configured for PUSCH requirement. The related test applicability and BS manufacture for difference RS configuration are defined as follows - Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.. Since the bottleneck of Doppler tracking is not the number of DMRS, we are open to discuss the test applicability in case more than one DMRS configuration is declared to be supported for FR2 HST. With more DMRS, the performance can be improved. We slightly prefer option 2, to test the minimum number of DMRS to guarantee the basic baseband processing. |
| Nokia | We agree that the test with the least number of DM-RS symbols is more challenging in HST FR2 deployments. Therefore, we are also OK with Option 2, because it will be sufficient to pass the test with the least number of supported DM-RS to confirm that the test with more DM-RS will pass as well. |
| Ericsson | We also somewhat prefer option 2; it is likely that the DM-RS will be configured as low as supportable and as pointed out by Samsung, the test is carried out with the largest data rate. |
| CATT | Since pos0 need PTRS to estimate the frequency offset, it is different from other additional DM-RS position. So we think passing pos0 can confirm that the test with other additional DM-RS position will pass. We are OK with option 4 as well as option 2. |
| Intel | We prefer to clearly define a rule which DMRS configuration should be used in case several are supported. There is no need to test BS with several DMRS configurations - one is sufficient. In this case Option 2 as using configuration with least number of DMRS symbols look reasonable to guarantee minimum demodulation performance. |
| Huawei | We prefer Option 3 that the tests shall be done for either one of pos0, pos1 or pos2. The method is similar as the selection of mapping type A or type B. Unless otherwise stated, PUSCH requirement tests shall apply only for the mapping type declared to be supported (see D.100 in table 4.6-1). If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B; the same chosen mapping type shall then be used for all tests.. We don’t think it reasonable to only consider the minimum number of DMRS for HST FR2 since maximum number of declared supporting DMRS (i.e. pos1 in {pos0, pos1}) is considered for legacy BS testing in Rel-15. |

Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support - PUSCH

- Proposals

  - Option 1 (Nokia, [Ericsson], Intel): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0, pos1/pos2, both}, applicable to BS Type 2-O.

  - Option 2 (Samsung, Intel, Ericsson, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0}, {pos1}, {pos2}, {pos0, pos1}, {pos0, pos1}, {pos0, pos2}, {pos1, pos2}, and all, applicable to BS Type 2-O.

  - Option 3 (CATT, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0, pos2, pos3}, applicable to BS Type 2-O.
Description reads: “Declaration of supported additional DM-RS position for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.”

  - Option 4: Other options not precluded.

- Recommended WF

  - Please discuss in first round.

  - It was not clear to the moderator, if the same declaration (HST FR2 DM-RS support) is shared between PUSCH and PUSCH UL TA, please comment in first round.

| Company | Comments |
| --- | --- |
| Samsung | We would like to list the detail RS configuration combination for declaration as option 2 |
| Nokia | Following the discussion at the previous RAN4 meetings, our understanding is that DM-RS pos2 and pos3 are considered as alternatives because the same requirements are defined for those. Therefore, we think that it is sufficient to define the following options for manufacturer declaration (Option 1): pos0; pos1; pos2; pos0 and pos1; pos 0 and pos 2, i.e., 5 in total. |
| Ericsson | There is only one requirement for pos2/pos3, but for the declaration the question is what is being declared; the pos that are supported or the pos that are available for testing. Regarding testing, if we define an applicability rule then anyhow only one pos will be tested. So probably option 2 makes sense then (declare all pos that are supported), but both option 1 and option 2 can work and are sufficient for deciding what to test. |
| CATT | From our understanding, option 2 and option3 are the same, while option2 lists all possible combinations. From the perspective of manufacture declaration, seem no need to list all possibility. For option 1, the pos1/pos2 may cause confusion and ambiguity. So we prefer the following table for simplification from perspective of manufacture declaration. Declaration identifier; Declaration; Description; Applicability (Note 1). BS type 1-H(Note 2); BS type 1-O; BS type 2-O. D.x; Additional DM-RS position for FR2 high speed train; Declaration of supported additional DM-RS position for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.; n/a; n/a; x. |
| Intel | Our understanding of the previous agreement is that BS may declare to support either pos2 or pos3 DMRS configuration, but the same requirement will be applied for both. In addition, declarations to support pos0 and combinations of pos0 with pos1 and pos2 are required. The main question is whether we need to define declaration to support both pos1 and pos2. In case of such declaration and applicability rule, that configuration with minimum number of DMRS symbols will be used for the test, the definition of such declaration makes no sense. Therefore, we are fine with Option 1 also in addition to our original proposal on Option 2. |
| Huawei | We prefer Option2 or new proposal by CATT. All possible single or multiple selections for different DMRS configurations should be supported to declare. |

### Sub-topic 1-2: MCS selection

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: Receiver baseline assumption for simulation

- Prior agreement [R4-2120703]

  - FOC method is up to BS implementation

  - Do not capture assumptions on FOC scheme in specification

  - Define PUSCH requirements with MCS(s) between MCS16-20 that is/are feasible and testable.

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Huawei, Intel, Huawei): Assume a receiver with post FFT FOC.

  - Option 2: Other options not precluded

- Recommended WF

  - Please discuss in first round. 
In particular, please comment on the need for a simulation baseline assumption for FOC and the kind of assumption that could be made.

| Company | Comments |
| --- | --- |
| Samsung | We are fine with option 1, pending on the simulation results alignment status. |
| Nokia | Following the prior agreements, we do not see a need to capture any assumption on the FOC implementation at the receiver in the specification. Even though our results do not demonstrate significant difference in the performance of post- and pre-FFT FOC, the post-FFT FOC is sub-optimal. Hence, post-FFT FOC can be assumed for the alignment of the results as the worse case. |
| Ericsson | Note that this is not any intention to override the previous agreement; just to try to get good alignment by using the same assumption. In the end implementation should not be constrained and the receiver type should not be written in the specification. |
| CATT | We are OK with option1 for simulation result alignment. |
| Intel | We are fine to have agreement to assume post-FFT FOC for simulation results alignment and requirement definition. |
| Huawei | We are OK with Option 1 for simulation results alignment and follow the previous agreement that FOC method is up to BS implementation and do not capture assumptions on FOC scheme in specification. |

Issue 1-2-2: MCS

- Proposals

  - Option 1 (Ericsson, Huawei, CATT, Nokia): Only 20.

  - Option 2 (Ericsson depending on simulations, Nokia, Huawei): Only 19.

  - Option 3 (Samsung): Both MCS 16 and MCS17 are feasible.

  - Option 4 (CATT, Intel, Samsung, Huawei): Only MCS16.

  - Option 5 (Nokia, Ericsson): Both MCS16 and MCS20.
With applicability rule that that the PUSCH performance requirements needs to be tested only with highest supported MCS.

  - Option 6 (Intel): Continue simulation results alignment with range as MCS16-MCS20.

- Recommended WF

  - Please discuss in first round.
Can option 1 be agreeable?

  - Moderator question: Option 5 is also asking for declarable MCS support?

| Company | Comments |
| --- | --- |
| Samsung | Based on existing simulation results, large performance among companies’ results, further checking is needed. Based on our results, the results with MCS 16 to MCS20 are feasible and can be testable. Compared with two kinds of FOC implementation, minor gap for MCS 16 and MCS17, while large gap for MCS20. Since the FOC method is up to BS implementation. It is better to close the MCS with same gap between two FOC implementation to requirement. Meanwhile, based on results summary, although up to 2dB performance gap is still existed, while performance gap for MCS 16 among companies is smallest, compared with other MCS. Soo, we are fine with option 4 with only MCS16 |
| Nokia | In our opinion, testing of MCS16 only, corresponding to 16QAM, does not demonstrate performance in a scenario typical to HST FR2. We see it beneficial to have a requirement for 64QAM modulation, especially in the case that is expected to be supported by the equipment. Therefore, we are supporting three options: Option 5, Option 1, and Option 2. Regarding the manufacturer declaration, it seems illogical to us to declare the support of selected MCSs only. Therefore, one solution could be to declare the highest supported MCS in HST FR2 scenario. Then, only highest MCS in the supported range will be required for testing. |
| Ericsson | In our understanding, since the UE is a train mounted UE and needs to deliver data to many users with in the train, and since high SNR will always be achieved, it is important for the receiver to support high data rates. Thus, a high MCS, as close to 20 as possible should be defined. We would prefer only one MCS, but allowing for MCS 16 in addition to MCS 20 and applying an applicability rule could also work as a compromise. It would be a not great compromise though as it would necessitate the BS declaring which highest MCS is supported. |
| CATT | We are OK with either option 1 or option 4. |
| Intel | It is hard to agree on MCS value with significant span among submitted results. We prefer to continue simulation results alignment and consider the same range as MCS16-MCS20. To move forward we suggest agreeing that only one value to be considered for requirements definition. |
| Huawei | We prefer to define only one requirements, so we are OK with Option 1, Option2 or Option 4. Maybe the MCS that result in smallest span can be selected for the requirements definition. |

### Sub-topic 1-3: Requirement selection

Sub-topic description

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1 (Nokia, Intel): If a large span in the PUSCH simulations results still persists, RAN4 to verify and compare the PUSCH performance in a simpler channel model: AWGN + fixed maximum Doppler offset of 19458 Hz (without bi-directional propagation channel).

  - Option 2 (CATT): Further check the span based on the consensus of Issue 1-2-1.

  - Option 3 (HW): Apply the method discussed in Sub-topic 4-3 to derive the performance requirements directly

- Recommended WF

  - Please comment on the need for option 1, given the results delivered to this meeting.

| Company | Comments |
| --- | --- |
| Samsung | Based on existing simulation results, large performance among companies’ results, further checking is needed. We still prefer the channel model agreed based on FR2 HST deployment scenario discussion to requirement selection. |
| Nokia | In reply to the comment by Samsung, we are not proposing to simplify or change already agreed model for the requirements. Our idea is to identify the source of the difference in the results based on the agreed model. For that purpose, we encourage other companies to provide simulation results, as a reference, in AWGN + fixed FO channel to remove the potential algorithmic impact of Doppler trajectory. |
| CATT | We can further check the span based on the consensus of Issue 1-2-1. |
| Intel | We support Option 1 that can help to align simulation results. At current stage it is not clear whether the difference comes from channel model implementation or Rx processing. |
| Huawei | Considering there is only 2 meetings for the deadline, we think it is reasonable to use the method discussed in Sub-topic 4-3 to derive the performance requirements directly and companies can provide updated simulation results in next meeting for final check. |

### Sub-topic 1-4: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| None |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 1-1 | Sub-topic 1-1: Test applicability rules and manufacturer declarations Issue 1-1-1: Test applicability Tentative agreements: None. Candidate options: - Option 2 (Samsung, Intel, Nokia, Ericsson, CATT): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported. - Option 3 (Huawei): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, a pass with either of the possibilities is sufficient to demonstrate compliance to the core requirement. Capture this in a note to the performance requirement. Recommendations for 2nd round: Continue discussion in second round. Option 2 is currently supported by the majority. Please try to foster understanding that can lead to consensus. Moderator question on option 3: Does choosing this option require an additional manufacturer declaration to indicate the tested configuration? The cited example of D.100 from early in Rel-15, does not have such a secondary declaration. Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support - PUSCH Tentative agreements: None. Candidate options: - Option 1 (Nokia, [Ericsson], Intel): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario. Choices are {pos0, pos1/pos2, both}, applicable to BS Type 2-O. - Option 2 (Samsung, Intel, Ericsson, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario. Choices are {pos0}, {pos1}, {pos2}, {pos0, pos1}, {pos0, pos1}, {pos0, pos2}, {pos1, pos2}, and all, applicable to BS Type 2-O. - Option 3 (CATT, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario. Choices are {pos0, pos2, pos3}, applicable to BS Type 2-O. Description reads: “Declaration of supported additional DM-RS position for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.” - Option 4 (Moderator): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario. Choices are (pos0), (pos1), (pos2), (pos0, pos1), (pos0, pos2), (pos1, pos2), and (pos0, pos1, pos2), applicable to BS Type 2-O. Description reads: “Declaration of all supported additional DM-RS positions for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.” Recommendations for 2nd round: The moderator has combined option 2 and option 4 to create the compromise proposal option 4. Please check in second round and comment. |
| Sub-topic 1-2 | Sub-topic 1-2: MCS selection Issue 1-2-1: Receiver baseline assumption for simulation Tentative agreements: Assume a receiver with post FFT FOC. Candidate options: None. Recommendations for 2nd round: Tentative agreements seems agreeable. Issue 1-2-2: MCS Tentative agreements: None. Candidate options: - Option 1 (Ericsson, Huawei, CATT, Nokia): Only 20. - Option 4 (CATT, Intel, Samsung, Huawei): Only MCS16. - Option 5 (Nokia, Ericsson): Both MCS16 and MCS20. With applicability rule that that the PUSCH performance requirements needs to be tested only with highest supported MCS. - Option 6 (Intel): Continue simulation results alignment with range as MCS16-MCS20. - Option 7 (Moderator): Use “MCS 20 only” as baseline. Change to “MCS 16 only”, if SNR after requirement derivation is larger than 20dB. Recommendations for 2nd round: The moderator has removed superseded options. Please continue discussion in second round. The moderator has proposed option 7 as a compromise. Please give feedback on this proposal. |
| Sub-topic 1-3 | Sub-topic 1-3: Requirement selection Tentative agreements: None. Candidate options: - Option 1 (Nokia, Intel): If a large span in the PUSCH simulations results still persists, RAN4 to verify and compare the PUSCH performance in a simpler channel model: AWGN + fixed maximum Doppler offset of 19458 Hz (without bi-directional propagation channel). - Option 2 (CATT): Further check the span based on the consensus of Issue 1-2-1. - Option 3 (HW): Apply the method discussed in Sub-topic 4-3 to derive the performance requirements directly. Recommendations for 2nd round: Continue to discuss in 2nd round. Interested companies are not forbidden from bringing results for option 1 in next meeting. It is recommended to down select the alignment scenario further. |
| Sub-topic 1-4 | Sub-topic 1-4: Other No issues raised. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
| #1 | WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

## Discussion on 2nd round

### Email comments

Dear All,

The formal version of the WF has been uploaded to the Inbox with TDoc ID R4-2203006.

The only difference to the previous version from the draft folder is that most of the color-coding was cleaned:

R4-2203006_WF_on_BS_demod_for_R17_FR2_HST_v10_Ericsson_WFModerator2_Clean.docx

Kind regards,

Dmitry

=====

Dear all

The final simulation results for FR2 HST demodulation is uploaded into

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-bis-e/Inbox/R4-2200743.zip

BR

Yunchuan Yang

===

Dear All,

The second round draft summary for [315] has been uploaded:

Round 2

It captures all the discussions that were cleaned from the WF final draft and makes the following tdoc recommendation:

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203006 | WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell | Agreeable | Current latest draft: draft_WF_on_BS_demod_for_R17_FR2_HST_v10_Ericsson_WFModerator2.docx |

Note: That the chair has decided all draftCRs to be postponed already in the first round.

Please check and comment, if you spot any issues.

Regards,

Axel

Nokia Shanghai Bell

===

Dear NR FR2 HST BS Demod Contributors,

Following the discussions on the WF, we want to clean up the WF and reflect some of the progress in the discussions.

WF folder

New tentative agreements and other notable updates in v6:

Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support - PUSCH

All companies seem to be aligned now that

1) Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario.

2) All combinations of pos0, pos1, and pos2 should be possible to declare

But the exact wording cannot be agreed right now.

Hence the moderator would like to propose the following tentative agreement:

Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario.

The intention is that all combinations of pos0, pos1, and pos2 should be possible to declare.

Exact wording is FFS.

Issue 1-3-1: Requirement selection

Captured Nokia’s text in the WF.

Please note that this text does not constitute an agreement. It is still marked as yellow as this text will remain in the WF unless contested.

Capture in the WF:

Interested companies are invited to voluntarily bring results for the propagation conditions in option 1, at least for the test case {Post-FFT/[Pre-FFT]; Type B, 10 Symbols, 120kHz/200MHz; MCS20; DM-RS 1+1}.

2.2 FRC

Following the discussion, Option 1 with the typo corrected seems agreeable.

4.3 Requirement derivation

The tentative agreement with Samsung’s note on ideal span is seems agreeable.

Use the legacy way to derive performance requirements as baseline:

Based on all contributed results, using outlier removal with [2] dB ideal span threshold / [4] dB impairment span threshold, and averaging result [R4-1904713] [R4-19004714].

The value of ideal span can be further discussed pending on the simulation results alignment.

Yellow highlights changed to green in

Issue 1-2-1: Receiver baseline assumption for simulation

Issue 2-1-1: Test applicability

Issue 2-1-2: Manufacturer declaration on HST FR2 DM-RS support – UL TA

Please check the changes we made.

Yellow tentative agreements will become full green agreements unless contested (contesting is possible until 16h00 UTC on Monday).

Please note that the final draft WF shared on Friday 19h00 UTC will be a cleaned version, i.e., it will remove all comments, all change marks, and all colouring!

Though, we will copy paste all of this into the 2nd round summary before, so the comments are not lost.

The moderator will recommend to the chair for

Issue 1-2-1: MCS

Issue 1-3-1: Requirement selection

to be discussed in a GtW, if some time remains, next week.

Regards,

Axel

Nokia Shanghai Bell

### WF comments

1	WF on topic#1: PUSCH requirements

  - Test applicability rules and manufacturer declarations

Issue 1-1-1: Test applicability

- Option 2 (Samsung, Intel, Nokia, Ericsson, CATT): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported.

- Option 3 (Huawei): FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, a pass with either of the possibilities is sufficient to demonstrate compliance to the core requirement. Capture this in a note to the performance requirement.

Recommended WF:

Moderator question on option 3: Does choosing this option require an additional manufacturer declaration to indicate the tested configuration? The cited example of D.100 from early in Rel-15, does not have such a secondary declaration.

| Company | Comments |
| --- | --- |
| Huawei | For Option 3, we clarify that same manufacturer declaration method as Option 2 is declared with no additional manufacture declaration. We prefer to not mandate to test certain number of DMRS if BS support different configurations, and BS should freely select one supporting configuration to pass the test as did for PUSCH mapping Type A and Type B. Still prefer Option 3. |
| Nokia, Nokia Shanghai Bell | Initially, we had a similar view as Huawei. However, the configuration with less DM-RSs is more challenging in HST FR2 conditions. Therefore, if the test has passed with fewer DM-RSs we can be sure that it passed also with more DM-RSs. However, the opposite might not be true. Hence, we prefer Option 1 more. |
| Intel | Configuration with smaller number of DMRS symbols implies higher constraints on receive processing implementation. In this case we do not think that different DMRS configurations are equivalent as proposed in Option 2, especially for high speed conditions. Therefore, we suggest defining clear rule to apply test with the smallest number of DMRS symbols in case several configurations are supported. |
| Ericsson | We continue to prefer option 2, because as pointed out earlier in the discussion the lowest number of DM-RS will test at the highest data rate, and it is more likely that the lowest number of DM-RS will be used in the deployment. |

Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support - PUSCH

- Option 1 (Nokia, [Ericsson], Intel): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0, pos1/pos2, both}, applicable to BS Type 2-O.

- Option 2 (Samsung, Intel, Ericsson, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0}, {pos1}, {pos2}, {pos0, pos1}, {pos0, pos1}, {pos0, pos2}, {pos1, pos2}, and all, applicable to BS Type 2-O.

- Option 3 (CATT, Huawei): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0, pos1, pos2}, applicable to BS Type 2-O.
Description reads: “Declaration of supported additional DM-RS position for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.”

- Option 4 (Moderator): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are (pos0), (pos1), (pos2), (pos0, pos1), (pos0, pos2), (pos1, pos2), and (pos0, pos1, pos2), applicable to BS Type 2-O.
Description reads: “Declaration of all supported additional DM-RS positions for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2.”

Recommended WF:

Tentative agreement:
Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario.
The intention is that all combinations of pos0, pos1, and pos2 should be possible to declare.
Exact wording is FFS.

| Company | Comments |
| --- | --- |
| Huawei | From our understanding, Option 3 is equivalents to Option4 since they both allow all possible single/multiple selections. For the detailed wording, we slightly prefer Option 3 since it is simpler. |
| Samsung | We prefer to list all the combination since it is limited. Regarding option 3, since only up to 3 DMRS is considered, while the number of DMRS is pos3? |
| Huawei | We think there is a typo for Option 3. It should be {pos0, pos1, pos2}. Moderator: Yes, this was a typo caused by the moderator. Thank you for fixing it. |
| CATT | We share the similar view as Huawei. Based on our interpretation, option 3 is exactly the same as option 4. For simplicity we prefer option 3. |
| Nokia, Nokia Shanghai Bell | From the company comments we see that curly brackets might not automatically mean that all combinations are possible. At least, in the current specifications it is usually specified whether both options are supported. See for example TS 38.141-2, Clause 4.6, D.101 PUSCH additional DM-RS positions Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. Hence, if the companies still see a need to declare all possible combinations of pos0, pos1, and pos 2, then it is much clearer to list all of them like in Option 4. Otherwise, complicated and misleading wording might be needed. We can agree on Option 4 in addition to Option 1. Could Samsung clarify their comment a bit more? |
| WFModerator (Nokia) | All companies seem to be aligned now that 1) Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario. 2) All combinations of pos0, pos1, and pos2 should be possible to declare But the exact wording cannot be agreed right now. Hence the moderator would like to propose the following tentative agreement: Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario. The intention is that all combinations of pos0, pos1, and pos2 should be possible to declare. Exact wording is FFS. Please check that this agreeable before the deadline or this will become agreement in the final draft. |
| Intel | Proposal from moderator is fine for us. We can finally resolve this issue during a CR review next meeting. |
| Ericsson | The WF is OK. Maybe the final wording for the declaration can just say “All DM-RS configurations supported by the BS”. |

  - MCS selection

Issue 1-2-1: Receiver baseline assumption for simulation

Agreement:

Assume a receiver with post FFT FOC.

Recommended WF:

Tentative agreements seem agreeable.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with the recommended WF. |
| CATT | OK with recommended WF. |
| Nokia, Nokia Shanghai Bell | Even though we observe that pre-FFT results are better aligned, i.e., only one clear outlier for pre-FFT vs two for post FFT, we think that post FFT assumption can be used unless we observe that the gap in the simulation results does not allow achieving the alignment. OK with WF. |

Issue 1-2-1: MCS

- Option 1 (Ericsson, Huawei, CATT, Nokia): Only 20.

- Option 4 (CATT, Intel, Samsung, Huawei): Only MCS16.

- Option 5 (Nokia, Ericsson): Both MCS16 and MCS20.
With applicability rule that that the PUSCH performance requirements needs to be tested only with highest supported MCS.

- Option 6 (Intel): Continue simulation results alignment with range as MCS16-MCS20.

- Option 7 (Moderator): Use “MCS 20 only” as baseline. Change to “MCS 16 only”, if SNR after requirement derivation is larger than 20dB.

- Option 8 (Nokia): Continue simulation results alignment with MCS16 and MCS20 only.

Recommended WF:

The moderator has removed superseded options. Please continue discussion in second round.
The moderator has proposed option 7 as a compromise. Please give feedback on this proposal.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with the recommended WF. |
| Samsung | We prefer to further check results firstly. Meanwhile, we should select the MCS result with small performance gap among companies and small performance gap for two kinds of FOC method, at least, based on our results, with MCS16 and MCS17, the performance gap is minor compared with MCS20. |
| CATT | OK with the recommended WF. |
| Nokia, Nokia Shanghai Bell | We are fine with the compromise proposal option 7 (in addition to O1 and O5). In response to Samsung’s comment, we don’t think that requirement alignment should factor into the decision between MCS16 and MCS20. If option 7 is not agreeable to all, could we at least reduce simulation load, by agreeing to continue simulation alignment with only MCS16 and MCS20? New option 8 has been added. |
| Intel | Can we add also MCS17 to Option 8? As Samsung mentioned results with MCS16 an MCS17 have the smallest gap. Also, as we understand we do not need to provide results for pre-FFT next meeting, that will reduce simulation load further. |
| Ericsson | We disagree to option 7. If MCS20 has SNR greater than 20dB, in our view we should take the next lowest MCS that has SNR<20dB. Option 8 could be OK as long as it is clear that it is for alignment only. The final requirement could then be based on MCS17 or MCS18 if necessary. We do not think that the MCS should be decided on the basis of which MCS produces alignment. Regarding pre-FFT, we do not need to have that for the final requirement. One friendly request to Intel though; could you double check your results for pre-FFT; they seem to be quite different to the others. (See, for example MCS20, 1+1 DM-RS, 200MHz) Could it be that what is causing the difference also impacts your post FFT ? Thanks 😊 |

  - Requirement selection

Issue 1-3-1: Requirement selection

- Option 1 (Nokia, Intel): If a large span in the PUSCH simulations results still persists, RAN4 to verify and compare the PUSCH performance in a simpler channel model: AWGN + fixed maximum Doppler offset of 19458 Hz (without bi-directional propagation channel).

- Option 2 (CATT): Further check the span based on the consensus of Issue 1-2-1.

- Option 3 (HW): Apply the method discussed in Sub-topic 4-3 to derive the performance requirements directly.

Recommended WF:

Capture in the WF:
Interested companies are invited to voluntarily bring results for the propagation conditions in option 1, at least for the test case {Post-FFT/[Pre-FFT]; Type B, 10 Symbols, 120kHz/200MHz; MCS20; DM-RS 1+1}.

| Company | Comments |
| --- | --- |
| Huawei | We prefer Option 3 considering the deadline is coming. |
| Samsung | We do not prefer option 1, which need more effort to align, and it not aligned with deployment scenario. We are ok to further check, if the results gap is still existed, option 3 can be applied for requirement derivation as starting point, while the gap value can be further discussed pending on the simulation results |
| CATT | OK with option 3. |
| Nokia, Nokia Shanghai Bell | We also agree that option 3 should ultimately be used to derive the requirements, however it cannot help to inform the current discussion on which MCS should be chosen. Though, we think option 3 should be used to define which MCS result in feasible test requirements. We propose the following to be captured in the WF: “Interested companies are invited to voluntarily bring results for the propagation conditions in option 1, at least for the test case {Post-FFT/[Pre-FFT]; Type B, 10 Symbols, 120kHz/200MHz; MCS20; DM-RS 1+1}.” To repeat from the earlier discussion, it is not our intention to propose to change the propagation conditions used for requirement captured in the spec. We want to make sure that the observed simulation misalignment is not due to implementation differences in the new bi-directional propagation channel. The test case above was chosen from the simulation results spreadsheet, as the case with large misalignment where all contributors have brought results before. |
| WFModerator (Nokia) | Captured Nokia’s text in the WF. Please note that this text does not constitute an agreement. It is still marked as yellow as this text will remain in the WF unless contested. |
| Intel | We are fine with the latest moderator proposal. |
| Ericsson | The moderator proposal is OK; the amount of simulation cases is kept small and it may help with alignment. Possibly you could add “The results are for alignment only and not intended for deriving the requirement” to be clear. |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2203006 | Agreeable |

# Topic #2: PUSCH with UL timing adjustment requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2200130 | CATT | Title: Simulation results for UL timing adjustment demodulation requirements for FR2 HST No proposals, Simulation results |
| R4-2200748 | Samsung | Title: Simulation results of UL timing adjustment requirement for Rel-17 FR2 HST Test Applicability rule for RS configuration Observation 1: Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration. Proposal 1: FR2 HST PUSCH UL timing adjustment requirement test shall apply only for the additional DM-RS position declared to be supported. If the all the DMRS configuration or subset of DMRS configuration declared to be supported, the test shall be done for the minimum number of DMRS supported, or only one of supported DMRS configuration based on BS manufacture declaration. DMRS configuration declared to be supported; PUSCH requirement tests. {pos0, pos1, pos2}; Option 1: either test with pos0, or pos1,or pos2 only based on BS declaration Option 2: only test for pos0. {pos0, pos1}; Option 1: either test with pos0 or pos1 only based on BS declaration Option 2: only test for pos0. {pos0, pos2}; Option 1: either test with pos0 or pos2 only based on BS declaration Option 2: only test for pos0. {pos1,pos2}; Option 1: either test with pos1 or pos2 only based on BS declaration Option 2:only test for post1. {pos0}, or {pos1}, or {pos2}; Only test DMRS declared to be supported. D.1XX; PUSCH additional DM-RS positions; Declaration of the supported additional DM-RS position(s) for FR2 high speed train scenario, i.e., {pos0},{pos1},{pos2},{pos0,pos1}, {pos0 pos1}, {pos0,pos2}, {pos1,pos2} and all; n/a; n/a; x. FRC Proposal 2: FRC for FR2 HST PUSCH UL timing adjustment with different RS configuration as following Table 2-1 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (16QAM, R=658/1024) Reference channel; G-FR2-Ax-xx; G-FR2-Ax-xx. Subcarrier spacing [kHz]; 120; 120. Allocated resource blocks; 16; 66. CP-OFDM Symbols per slot (Note 1); 9; 9. Modulation; 16QAM; 16QAM. Code rate (Note 2); 658/1024; 658/1024. Payload size (bits); 4480; 1843. Transport block CRC (bits); 24; 24. Code block CRC size (bits); -; 24. Number of code blocks - C; 1; 3. Code block size including CRC (bits) (Note 2); 4504; 6176. Total number of bits per slot with PT-RS (Note 3); 6624; 27324. Total symbols per slot with PT-RS; 1656; 6831. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1.. Table 2-2 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024) Reference channel; G-FR2-Ax-xx; G-FR2-Ax-xx. Subcarrier spacing [kHz]; 120; 120. Allocated resource blocks; 16; 66. CP-OFDM Symbols per slot (Note 1); 8; 8. Modulation; 16QAM; 16QAM. Code rate (Note 2); 658/1024; 658/1024. Payload size (bits); 2976; 16392. Transport block CRC (bits); 24; 24. Code block CRC size (bits); -; 24. Number of code blocks - C; 1; 2. Code block size including CRC (bits) (Note 2); 2992; 8232. Total number of bits per slot with PT-RS (Note 3); 5888; 24288. Total symbols per slot; 1472; 6072. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1.. Table 2-3 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos2 and 1 transmission layer (16QAM, R=658/1024) Reference channel; G-FR2-Ax-xx; G-FR2-Ax-xx. Subcarrier spacing [kHz]; 120; 120. Allocated resource blocks; 16; 66. CP-OFDM Symbols per slot (Note 1); 7; 7. Modulation; 16QAM; 16QAM. Code rate (Note 2); 658/1024; 658/1024. Payload size (bits); 3496; 14344. Transport block CRC (bits); 24; 24. Code block CRC size (bits); -; 24. Number of code blocks - C; 1; 2. Code block size including CRC (bits) (Note 2); 3512; 7208. Total number of bits per slot with PT-RS (Note 3); 5152; 21252. Total symbols per slot; 1288; 5313. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1.. |
| R4-2201007 | Huawei, HiSilicon | Title: Discussion on PUSCH with UL timing adjustment requirements for FR2 HST No proposals, Simulation results |
| R4-2201523 | Ericsson | Title: PUSCH with UL timing adjustment requirements No proposals, Simulation results |
| R4-2201845 | Nokia, Nokia Shanghai Bell | Title: On HST FR2 PUSCH UL Timing Adjustment Requirements SCS and BW; MCS; Channel; DM-RS mapping; Length of data; RS configuration; SNR, dB. 120kHz, 50MHz; MCS16; AWGN; B; 10; DMRS 1+0; 6.75. 120kHz, 50MHz; MCS16; AWGN; B; 10; DMRS 1+1; 6.46. 120kHz, 50MHz; MCS16; AWGN; B; 10; DMRS 1+1+1; 6.73. 120kHz, 200MHz; MCS16; AWGN; B; 10; DMRS 1+0; 6.60. 120kHz, 200MHz; MCS16; AWGN; B; 10; DMRS 1+1; 6.47. 120kHz, 200MHz; MCS16; AWGN; B; 10; DMRS 1+1+1; 6.47. - Use the results in the table above for the alignment of PUSCH UL Timing Adjustment demodulation performance. Moderator: Proposal 7 has not been included in the summary. Please include the simulation results in the simulation result summary. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

The agreements from the previous meetings are recorded in the corresponding WFs, i.e., R4-2115725, R4-2115726, and R4-2120703, respectively.

### Sub-topic 2-1: Test applicability rules and manufacturer declarations

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: Test applicability

- Prior agreement RAN4#100e [R4-2115726]

  - Test setup for UL timing adjustment requirements

    - RS configuration

      - Align RS configuration for UL timing adjustment requirement and PUSCH requirement

- Prior agreement RAN4#101e [R4-2120703]

  - RS configurations for PUSCH and UL timing adjustment

    - Define requirement with 1 DMRS + PT_RS (L=1, K=2) configuration

      - Define FRC for 1 DMRS + PT_RS (L=1, K=2)

    - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS+ PT_RS (L=1, K=2) and 3 DMRS + PT_RS (L=1, K=2)

      - Define FRC for 2 DMRS + PT_RS (L=1, K=2)

      - Define FRC for 3 DMRS + PT_RS (L=1, K=2)

    - Way forward

      - FFS, definition of test applicability rules based on BS manufacturer declaration

- Proposals

  - Option 1 (Samsung): FR2 HST PUSCH UL timing adjustment requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, only one of supported DMRS configuration shall be tested based on BS manufacture declaration.

  - Option 2 (Samsung): FR2 HST PUSCH UL timing adjustment requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported.

  - Option 3: Other options not precluded.

- Recommended WF

  - Please discuss in first round.
Can the applicability rule and/or manufacturer declaration shared with PUSCH?

  - Moderator remark on options 1 and 2: 
It was not clear to the moderator, if the same declaration is shared between PUSCH and PUSCH UL TA, please comment in first round.
If option 1 is chosen an additional manufacturer declaration to select the test is required.

| Company | Comments |
| --- | --- |
| Samsung | In Rel-15 BS demod requirement for FR2, both 1 DMRS and 2 DMRS configuration are configured for PUSCH requirement. The related test applicability and BS manufacture for difference RS configuration are defined as follows - Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.. Since the bottleneck of Doppler tracking is not the number of DMRS, we are open to discuss the test applicability in case more than one DMRS configuration is declared to be supported for FR2 HST. With more DMRS, the performance can be improved. We slightly prefer option 2, to test the minimum number of DMRS to guarantee the basic baseband processing. Regarding the applicability rule, it can be shared with PUSCH. |
| Nokia | We propose to share the same applicability rule with PUSCH. |
| Ericsson | We should use the same applicability rule as for PUSCH |
| CATT | To align with PUSCH. |
| Intel | We support to consider the same applicability rule for PUSCH and UL TA requirements on DMRS configuration. |
| Huawei | We propose to align with PUSCH. |

Issue 2-1-2: Manufacturer declaration on HST FR2 DM-RS support – UL TA

- Proposals

  - Option 1 (Samsung): Define manufacturer declaration for PUSCH additional DM-RS in FR2 HST scenario.
Choices are {pos0}, {pos1}, {pos2}, {pos0, pos1}, {pos0, pos1}, {pos0, pos2}, {pos1, pos2}, and all, applicable to BS Type 2-O.

  - Option 2: Other options not precluded.

- Recommended WF

  - Please discuss in first round.
Can the manufacturer declaration shared with PUSCH?

  - Moderator remark on option 2: 
It was not clear to the moderator, if the same declaration is shared between PUSCH and PUSCH UL TA, please comment in first round.

| Company | Comments |
| --- | --- |
| Samsung | We would like to list the detail RS configuration combination for declaration as option 1 |
| Nokia | Our preference is already listed in Topic#1, Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support – PUSCH, i.e., pos0, pos1/pos2, or both. We think that the only one manufacture declaration can be used in HST FR2 deployment both for PUSCH and UL TA tests. |
| Ericsson | We can use the same declaration as for PUSCH. It need not be listed as a separate declaration; the declared configurations are supported both for PUSCH and for TA. |
| CATT | To align with PUSCH. |
| Intel | The same manufacturer declaration should be used for UL TA as for PUSCH. Do not need to discuss this separately. We suggest agreeing that the same applicability rule and manufacturer declaration on DMRS configuration is used for PUSCH and UL TA requirements. |
| Huawei | We propose to align with PUSCH. |

### Sub-topic 2-2: FRC

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1 (Samsung): FRC for FR2 HST PUSCH UL timing adjustment with different RS configuration as following

Table 2-1 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 4480 | 1843 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 3 |
| Code block size including CRC (bits) (Note 2) | 4504 | 6176 |
| Total number of bits per slot with PT-RS (Note 3) | 6624 | 27324 |
| Total symbols per slot with PT-RS | 1656 | 6831 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-2 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 2976 | 16392 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 2992 | 8232 |
| Total number of bits per slot with PT-RS (Note 3) | 5888 | 24288 |
| Total symbols per slot | 1472 | 6072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-3 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos2 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 7 | 7 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 3496 | 14344 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 3512 | 7208 |
| Total number of bits per slot with PT-RS (Note 3) | 5152 | 21252 |
| Total symbols per slot | 1288 | 5313 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

  - Option 2 (CATT): Wait for MCS decision.

  - Option 3: Other options not precluded

- Recommended WF

  - Take option 1 as baseline for CR drafting.

| Company | Comments |
| --- | --- |
| Samsung | Companies are encouraged to further check whether the calculation is correct or not. Meanwhile, please companies to check whether the terminology of “Total number of bits per slot with PT-RS (Note 3)” is fine? which was discussed in previous meeting to differentiate the total number of bits with and without PT-RS configuration |
| Nokia | We have noticed that in Table 2-1, there is misprint in the payload for 200MHZ CBW. It shall be 18432. We are still checking the FRFs and will provide further comments if needed. |
| CATT | We can wait for MCS decision. |

### Sub-topic 2-3: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| YYY | Title, Source |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 2-1 | Sub-topic 2-1: Test applicability rules and manufacturer declarations Issue 2-1-1: Test applicability Tentative agreements: Share the same applicability rule with PUSCH. Candidate options: None. Recommendations for 2nd round: Tentative agreement seems agreeable. Issue 2-1-2: Manufacturer declaration on HST FR2 DM-RS support – UL TA Tentative agreements: Share the same manufacturer declaration with PUSCH. Candidate options: None. Recommendations for 2nd round: Tentative agreement seems agreeable. |
| Sub-topic 2-2 | Sub-topic 2-2: FRC Tentative agreements: None. Candidate options: - Option 1 (Samsung): FRC for FR2 HST PUSCH UL timing adjustment with different RS configuration as following - [Moderator: Please find tables above in sub-topic 2-2 open issues summary.] - Option 2 (CATT): Wait for MCS decision. - Option 3: Other options not precluded Recommendations for 2nd round: Is option 2 agreeable? In the meantime all companies are invited to check correctness of the already provided FRCs until the next meeting (or earlier). |
| Sub-topic 2-3 | Sub-topic 2-3: Other: No issues raised. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
| None |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

## Discussion on 2nd round

### Email comments

None

### WF comments

2	WF on topic#2: PUSCH with UL timing adjustment requirements

  - Test applicability rules and manufacturer declarations

Issue 2-1-1: Test applicability

Agreement:

Share the same applicability rule with PUSCH.

Recommended WF:

Tentative agreement seems agreeable.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with the recommended WF. |
| CATT | OK with recommended WF. |
| Nokia, Nokia Shanghai Bell | Fine with agreement. |
| Ericsson | OK |

Issue 2-1-2: Manufacturer declaration on HST FR2 DM-RS support – UL TA

Agreement:

Share the same manufacturer declaration with PUSCH.

Recommended WF:

Tentative agreement seems agreeable.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with the recommended WF. |
| CATT | OK with the recommended WF. |
| Nokia, Nokia Shanghai Bell | Fine with agreement. |
| Ericsson | OK |

  - FRC

- Option 1 (Samsung): FRC for FR2 HST PUSCH UL timing adjustment with different RS configuration as following

Table 2-1 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 4480 | 18432 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 3 |
| Code block size including CRC (bits) (Note 2) | 4504 | 6176 |
| Total number of bits per slot with PT-RS (Note 3) | 6624 | 27324 |
| Total symbols per slot with PT-RS | 1656 | 6831 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-2 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 2976 | 16392 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 2992 | 8232 |
| Total number of bits per slot with PT-RS (Note 3) | 5888 | 24288 |
| Total symbols per slot | 1472 | 6072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-3 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos2 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 7 | 7 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 3496 | 14344 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 3512 | 7208 |
| Total number of bits per slot with PT-RS (Note 3) | 5152 | 21252 |
| Total symbols per slot | 1288 | 5313 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

- Option 2 (CATT): Wait for MCS decision.

- Option 3: Other options not precluded

Recommended WF:

Option 1 with the typo corrected is agreeable.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with Option 1. |
| Samsung | Only MCS16 is considered for requirement, This is a typo, 1843 should be 18432, thanks for Nokia carefully checking |
| CATT | OK with option 1. |
| Nokia, Nokia Shanghai Bell | Ok with Option 1, after typo correction |
| WFModerator (Nokia) | Following the discussion, Option 1 with the typo corrected is agreeable. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

# Topic #3: PRACH requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2200131 | CATT | Title: Simulation results for PRACH demodulation requirements for FR2 HST No proposals, Simulation results |
| R4-2200749 | Samsung | Title: Simulation results of PRACH requirement for Rel-17 FR2 HST No proposals, Simulation results |
| R4-2201008 | Huawei, HiSilicon | Title: Discussion on PRACH demodulation requirements for FR2 HST No proposals, Simulation results |
| R4-2201009 | Huawei, HiSilicon | Title: Draft CR on PRACH minimum requirements for high speed train (38.104) draftCR |
| R4-2201010 | Huawei, HiSilicon | Title: Draft CR on PRACH test requirement for high speed train (38.141-2) draftCR |
| R4-2201521 | Ericsson | Title: FR2 HST PRACH requirement No proposals, Simulation results |
| R4-2201003 | Huawei, HiSilicon | Title: Draft CR on HST FR2 BS manufacturer's declarations and applicability rule (38.141-2) draftCR Moderator: Originally submitted to AI 6.9.5.1. Only impacts BS demod requirements and has been moved to AI 6.9.5.3 by chair. Treated in this section (6.9.5.3.3) to bundle with other draftCRs. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

The agreements from the previous meetings are recorded in the corresponding WFs, i.e., R4-2115725, R4-2115726, and R4-2120703, respectively.

No proposals have been made in this AI.

DraftCRs were submitted. Please see handling below.

### Sub-topic 3-1: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| Ericsson | We realized that we have mistakenly submitted an incorrect result for PRACH. The correct value should be -12.6 dB (ideal). We will update the results spreadsheet. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

Moderator: Please note the guidance of the co-chair given along with the email thread list:

| 315 | NR_HST_FR2_Demod_Part2 | [101-bis-e][315] NR_HST_FR2_Demod_Part2 | NR_HST_FR2-Perf | Rel-17 NR FR2 HST BS Demod requirements | 6.9.5.3 | Axel Muller | Handling of draft CRs postponed to future RAN4 meetings |
| --- | --- | --- | --- | --- | --- | --- | --- |

Hence the CR/TP comments collection below is only intended for early offline discussion. 
A recommendation to “postpone” will be given after the 2nd round discussion to all submitted CRs/TPs!
Still, please feel free to voluntarily highlight issues and observations here that would lead to comments in a future meeting.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2201009 | Draft CR on PRACH minimum requirements for high speed train (38.104), Huawei, HiSilicon |
|  | Company A |
|  | Company B |
| R4-2201010 | Draft CR on PRACH test requirement for high speed train (38.141-2), Huawei, HiSilicon |
|  | Company A |
|  | Company B |
| R4-2201003 | Draft CR on HST FR2 BS manufacturer's declarations and applicability rule (38.141-2), Huawei, HiSilicon |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 3-1 | Sub-topic 3-1: Other Please note the remark from Ericsson that their PRACH simulation result was significantly corrected in the results collection spreadsheet. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
| None |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2201009 | Return to |
| R4-2201010 | Return to |
| R4-2201003 | Return to |

## Discussion on 2nd round

### Email comments

None

### WF comments

3	WF on topic#3: PRACH requirements

No agreements.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

# Topic #4: Other

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| None |  |  |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

### Sub-topic 4-1: Confirmation of offline changes to CR split

Sub-topic description:

The CR split was left as informative in the last meeting’s WF, as some double request could not be immediately resolved. In between meetings, the contested entries were discussed offline between the signed-up companies.

The outcome of this offline exchange is captured here for confirmation and approval (in the WF).

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1:

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Samsung |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | Samsung, Nokia |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | Huawei |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, Samsung |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | Ericsson |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |

  - Option 2: Other options not precluded.

- Recommended WF

  - Take option 1, remove square brackets, and capture in WF.

  - Moderator: A work split proposal, including BS demod and matching the above, was submitted to 6.9.5.1 for approval. AI 6.9.5.1 is handled in [314].
Please discuss this CR work split in the email thread and summary of [101-bis-e][314] NR_HST_FR2_Demod_Part1.

| Company | Comments |
| --- | --- |
| Samsung | As rapporteur company, we appreciated companies’ contribution for FR2 HST WI. If companies have interest for the same topic, joint contribution is highly appreciated. If companies identify additional CR work is needed, the CR work arrangement can be discussed. |
| Nokia | We do not have an further comment to the CR work split listed in the table above. |
| Moderator | A work split proposal, including BS demod and matching the above, was submitted to 6.9.5.1 for approval. AI 6.9.5.1 is handled in [314]. Please discuss this CR work split in the email thread and summary of [101-bis-e][314] NR_HST_FR2_Demod_Part1. |
| CATT | We are OK with the recommended WF. |

### Sub-topic 4-2: Simulation result collection

Sub-topic description:

Many companies have brought simulation result contributions to this meeting.
In the last meeting Samsung has created the simulation result summary template [R4-2117591]. Would Samsung volunteer to continue to keep it updated until the end the of the BS demodulation performance work, or would another volunteer need to be found?

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1 (Moderator): In the last meeting Samsung has created the simulation result summary template [R4-2117591]. Would Samsung volunteer to continue to keep it updated until the end the of the BS demodulation performance work, or would another volunteer need to be found?

  - Option 2: Other options not precluded.

- Recommended WF

  - Capture responses to option 1 in first round.
A new tdoc can be requested in case the volunteer wants to update the summary already for this meeting.

  - Moderator: A simulation summary excel [R4-2200743], including BS demod, was reserved for Information, under AI 6.9.5.1. AI 6.9.5.1 is handled in [314].
Samsung has also uploaded the BS demod parts to the draft folder of [315].
Please verify, include, and correct your BS demod contributed simulation results in the [315] draft folder. It will be merged into the final version of R4-2200743 and noted in [314].

| Company | Comments |
| --- | --- |
| Samsung | As rapporteur companies, we would like to take the simulation results summary work for FR2 HST WI performance part. Please companies’ double check whether the results are captured correctly. Meanwhile, we appreciated that if companies would like to update impairment results if you have not uploaded yet |
| Nokia | Agree with the comment by Samsung. |
| Moderator | A simulation summary excel [R4-2200743], including BS demod, was reserved for Information, under AI 6.9.5.1. AI 6.9.5.1 is handled in [314]. Samsung has also uploaded the BS demod parts to the draft folder of [315]. Please verify, include, and correct your BS demod contributed simulation results in the [315] draft folder. It will be merged into the final version of R4-2200743 and noted in [314]. |

### Sub-topic 4-3: Requirement derivation

Sub-topic description

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1 (Moderator): Use the legacy way to derive performance requirements based on all contributed results, using outlier removal with [2] dB ideal span threshold / [4] dB impairment span threshold, and averaging result [R4-1904713] [R4-19004714].

  - Option 2: Other options not precluded.

- Recommended WF

  - Please comment in first round in case the legacy procedure is not applicable to HST FR2.

| Company | Comments |
| --- | --- |
| Samsung | We can use it as reference, pending on the simulation results alignment status. Based on existing results, the performance gap is still very diverse. Further checking simulation results are needed |
| Nokia | Option 1 approach should be used as a baseline. If the source of difference in the simulation results cannot be identified and the difference persists at the next meeting, then another solution shall be found. We also encourage companies to check their simulations results in a simpler channel model (Sub-topic 1-3) and provide them for comparison with other companies at the next meeting. |
| CATT | We support option 1. |
| Intel | We are fine to consider the legacy approach as a baseline. |

### Sub-topic 4-4: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 4-1 | Sub-topic 4-1: Confirmation of offline changes to CR split Recommendations for 2nd round: Moderator: A work split proposal, including BS demod and matching the above, was submitted to 6.9.5.1 for approval. AI 6.9.5.1 is handled in [314]. The final outcome will be captured in the BS demod WF for information. |
| Sub-topic 4-2 | Sub-topic 4-2: Simulation result collection Recommendations for 2nd round: A simulation summary spreadsheet [R4-2200743], including BS demod, was reserved for Information, under AI 6.9.5.1. AI 6.9.5.1 is handled in [314]. Samsung has also uploaded the BS demod parts to the draft folder of [315]. Please verify, include, and correct your BS demod contributed simulation results in the [315] draft folder. It will be merged into the final version of R4-2200743 and noted in [314]. In the next meeting it is expected that the simulation summary is split into an UE demod and a BS demod submission. |
| Sub-topic 4-3 | Sub-topic 4-3: Requirement derivation Tentative agreements: Use the legacy way to derive performance requirements as baseline: Based on all contributed results, using outlier removal with [2] dB ideal span threshold / [4] dB impairment span threshold, and averaging result [R4-1904713] [R4-19004714]. Candidate options: None. Recommendations for 2nd round: Tentative agreement seems agreeable. |
| Sub-topic 4-3 | Sub-topic 4-3: Other No issue raised. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
| None |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

## Discussion on 2nd round

### Email comments

None

### WF comments

4	WF on topic#4: Other

  - Requirement derivation

Agreement:

Use the legacy way to derive performance requirements as baseline:

Based on all contributed results, using outlier removal with [2] dB ideal span threshold / [4] dB impairment span threshold, and averaging result [R4-1904713] [R4-19004714].
The value of ideal span can be further discussed pending on the simulation results alignment.

Recommended WF:

Tentative agreement with note on ideal span is agreeable.

| Company | Comments |
| --- | --- |
| Huawei | We are OK with the recommended WF. |
| Samsung | In general, we are ok with this way to derive requirement, The value of ideal span can be further discussed pending on the simulation results alignment |
| CATT | OK with the recommended WF. |
| Nokia, Nokia Shanghai Bell | Fine with recommended WF. |
| WFModerator (Nokia) | The tentative agreement with Samsung’s note on ideal span is seems agreeable. |

  - CR split (informative)

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Samsung |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | Samsung, Nokia |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | Huawei |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, Samsung |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | Ericsson |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| None |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2201009 | Draft CR on PRACH minimum requirements for high speed train (38.104) | Huawei, HiSilicon | Return to Moderator from the chair’s notes: Postponed. | To be postponed in 2nd round. |
| R4-2201010 | Draft CR on PRACH test requirement for high speed train (38.141-2) | Huawei, HiSilicon | Return to Moderator from the chair’s notes: Postponed. | To be postponed in 2nd round. |
| R4-2201003 | Draft CR on HST FR2 BS manufacturer's declarations and applicability rule (38.141-2) | Huawei, HiSilicon | Return to Moderator from the chair’s notes: Postponed. | To be postponed in 2nd round. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2203006 | WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Axel Mueller | axel.mueller@nokia-bell-labs.com |
| Nokia, Nokia Shanghai Bell | Dmitry Petrov | Dmitry.a.petrov@nokia-bell-labs.com |
| Ericsson | Tom Chapman | Thomas.chapman@ericsson.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| Intel Corporation | Artyom Putilin | artyom.putilin@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add your name as suffix after company name, when making comments, i.e. Company A (XX, XX).