3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2203109

Electronic Meeting, 17th – 25th January 2022

Agenda item:			6.12.2.1, 6.12.2.2

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [101-bis-e][317] NR_perf_enh2_Demod_Part2

Document for:	Information

# Introduction

The scope of this email thread is the following topics of Rel-17 Further enhancement on NR demodulation performance WI:

- MMSE-IRC receiver for inter-cell interference

- MMSE-IRC receiver for intra-cell inter-user interference

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Discussion on open issues

  - Collection of comments on MMSE-IRC MU-MIMO TPs

- 2nd round:

  - WFs preparation

  - Revision of MMSE-IRC MU-MIMO TPs

# Topic #1: MMSE-IRC receiver for inter-cell interference – Demodulation requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200269 | Apple | Proposal #1: Only consider synchronized network for ICI requirements. Proposal #2: Configure non-overlapping SSBs on target and interfering cells. Observation #1: The operating SINR and gain with MMSE-IRC are very comparable between 1 and 2 interference cells in both scenarios. Observation #2: The gain with MMSE-IRC is more with scenario 1 compared to scenario 2 since the INR levels are higher. Proposal #3: For homogeneous network define requirements with 1 interference cell with INR 5.49 dB. Proposal #4: For HetNet deployment define requirements with 1 interference cell. Proposal #5: Define requirements with time offset of interference cell in TDD as 1us. |
| R4-2200366 | NTT DOCOMO, INC. | Proposal 1: For SSB configuration, use Option 1 configuration ( i.e. serving cell and interference cells SSBs are in the same time/frequency resources). Proposal 2: Our preference is Option 2, but Option 3A is also acceptable for us. |
| R4-2200377 | MediaTek inc. | Proposal 1: Consider INR value 5.49 dB for Homogeneous deployment assumptions. Proposal 2: Consider INR values 11.39 dB and 5.45 dB for HetNet deployment assumptions. Proposal 3: Consider different number of explicitly modeled interference cells for different deployment scenarios: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions. Proposal 4: Consider two test cases for FDD with the following settings: - Test case for 2Rx: TDLA 30-10, INR1: 11.39 dB, INR2: 5.45 dB - Test case for 4Rx: TDLC 300-100, INR1: 5.49 dB |
| R4-2200500 | Nokia, Nokia Shanghai Bell | Observation 1: Synchronized deployment is significantly better than asynchronized deployment regarding interference mitigation. Observation 2: Gain of IRC receiver in asynchronous networks is diminished compared to synchronous networks. Proposal 1: Don’t make separate requirements for asynchronized network deployment. UEs in asynchronized networks should be held to the same (relative IRC gain) performance requirements as in synchronized. Proposal 2: Use SSB Option 1 for all tests. All SSBs (serving cell and interference cell(s)) are at the same time/frequency resources. Observation 3: Gain of IRC receiver is highest for option 1 in homogenous deployment. We have a slight preference for option 1, but are open to compromise. Proposal 3: Option 1 for defining requirements. Observation 4: Substantial gain with proposed option for both 2 interferers and 1 interferer cases and the corresponding SINR in all cases is > -6 dB. Proposal 4: To use baseline INR option to define the requirements Observation 5: SINR at 70% throughput of IRC receiver is ~1 dB different with 1 interfering cell as compared to 2 interfering cells. Proposal 5: To use option 3A to define the requirements. Proposal 6: For TDD 30kHz, we propose to use option 1 for TDD: The time offset is 3 us for interfering cell 1 and –1 us for interfering cell 2. |
| R4-2200504 | Nokia, Nokia Shanghai Bell | Proposal 1: Discuss among the companies with CR responsibly, what information is captured in which section/table. We currently assume that, at least, the interference related parameters captured in TS36.101 section 8 tables of LTE will also be included in NR (in an adapted version). Proposal 2: Change Es⁄Noc to INR in parameter table and add a new row with number of non-dominant interferers while defining the NR tables. Proposal 3: Discuss the usage of a generic beamforming model. Proposal 4: Discuss the usage of separate interfering UEs along with PRB allocation in interference modelling. We assumed a single UE with full CBW PRB allocation in drafting the TP. Proposal 5: Discuss among the companies with CR responsibly, if ‘slot’ or ‘TDRA’ should be used, while defining the interference model randomization. Proposal 6: NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario. The codebook is TBD. |
| R4-2200512 | Intel Corporation | Observation #1: In case of random per-slot interference precoder, MMSE-IRC performance benefits for asynchronous scenarios are less in comparison to synchronous scenarios. Observation #2: In case of fixed or periodic interference precoder, MMSE-IRC performance benefits for asynchronous scenarios are rather close to performance synchronous scenarios. Proposal 1: Further discuss the following options: - Option 1: Define requirements for asynchronous scenario with fixed or periodic interference precoder assumptions in Rel-17 - Option 2: Further discuss the definition of requirements for asynchronous scenario in Rel-18 Proposal 2: Use different SSB configuration assumptions for different deployment scenarios: overlapping for homogeneous and non-overlapping for heterogeneous or vice versa. Proposal 3: Consider INRs 7.77 and 2.29 dB (2 interference cells) or INR 5.49 dB (1 interference cell) for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario with Homogeneous deployment assumptions. Proposal 4: Consider INRs 11.39 and 5.45 dB (2 interference cells) or INR 7.58 dB (1 interference cell) for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario with Heterogeneous deployment assumptions. Observation #3: If 1 interference cell is explicitly modelled then the contribution of the total receive signal power from dominant interference cell to the total receive signal power from all interference cells is 50% or less for the 50% of user. Observation #4: In 2 interference cells are explicitly modelled then the contribution of the total receive signal power from dominant interference cells to the total receive signal power from all interference cells is 73% or less for the 50% of user Proposal 5: Use explicit modelling of 2 interference cells for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario. Proposal 6: Use the following time offset configuration for TDD requirements: 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modelled). Proposal 7: Use MCS 13 for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference Homogeneous and Heterogeneous scenario. Observation #5: MMSE-IRC performance benefits for scenario with INRs 5.43 and -1.50 dB and with 2 Rx are less than 1 dB. Observation #6: MMSE-IRC performance benefits for scenario with INR 3.10 dB are less than 1 dB. Observation #7: MMSE-IRC SINR operating for 4 Rx case and INRs 11.39 and 5.45 is rather close to -6 dB. Proposal 8: Reuse the following Scenario 1 assumptions for further performance analysis for Scenario 2: - Common parameters: - Synchronized network - SCS/CBW: FDD 15kHz/10MHz, TDD 30kHz/40MHz - PDCCH allocation in time domain: symbols #0 and #1 of each slot - PDSCH allocation in frequency domain for all cells: Full PRB - TRS/CSI-RS configuration: Colliding among the cells - Propagation conditions: TDLC300-100 or TDLA30-10 - Antenna configuration: 2x2 and 2x4 ULA Low - Serving cell parameters - PDSCH FRC for serving cell: Rank 1, MCS 13 - PDSCH allocation in time domain: Mapping type A, Start symbol 2, Duration 12 - DMRS configuration: DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data - PRB bundle size: Set PRB bundle size as 2 for target PDSCH - HARQ process number: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline - Precoding model: Random precoder with Type I SP codebook Proposal 9: Consider the following interference modelling assumptions for Scenario 2: - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) - Precoding: Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. - Modulation order: 16QAM randomly modulated symbols. - PDSCH mapping in time domain and DMRS configuration: - Scenario 2-1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both interference cells, Single symbol front-loaded DMRS and no additional DMRS. - Scenario 2-2: Type A PDSCH mapping with starting symbol 2 and duration 5 for interference cell #1 and Type B PDSCH mapping with starting symbol 7 and duration 7 for interference cell #2, Single symbol front-loaded DMRS and no additional DMRS. Proposal 10: Consider the following reference receiver assumptions for Scenario 2: - MMSE-IRC with DMRS based covariance matrix estimation - Use covariance matrix from the first DMRS for demodulation of resource elements in the first half of slot and covariance matrix from the second DMRS for demodulation of resource elements in the second half of slot. Observation #8: Enhanced MMSE-IRC processing with per-half slot processing leads to additional 0.7 dB performance improvement in comparison to MMSE-IRC processing for Scenario 2-1. Observation #9: Enhanced MMSE-IRC processing with per-half slot processing leads to additional 1.6 dB performance improvement in comparison to MMSE-IRC processing for Scenario 2-2. |
| R4-2200513 | Intel Corporation | Summary of PDSCH simulation results for inter-cell interference suppression |
| R4-2200795 | CMCC | Draft CR for TS38.101-4 PDSCH TDD demodulation requirements for inter-cell interference MMSE-IRC |
| R4-2200798 | CMCC | Proposal 1: Include FDD asynchronized network type in FR1. Proposal 2: - SSB Option 1 for homogeneous deployment assumptions - SSB Option 2 for heterogeneous deployment assumptions Proposal 3: 2 interference cells for homogeneous deployment assumptions and 1 interference for heterogeneous deployment assumptions. |
| R4-2200902 | China Telecom | Observation 1: 1) The performance of MMSE receiver for sync and async is similar, since the interference power level is the same for each slot in the neighbor cell. 2) Performance degradation for MMSE-IRC in async scenario compared with sync scenario is observed, due to the lower accuracy of Rnn estimation. 3) The MMSE-IRC receiver can still achieve reasonable performance gain over MMSE receiver under async scenarios, only when the UE use the averaged estimated Rnn on the 2 DMRS symbols. Proposal 1: Include MMSE-IRC performance requirements for async FDD scenarios, to better verify the UE MMSE-IRC implementation since we cannot always assume the real networks are well synchronized. Proposal 2: Support using aligned SSB configuration for HomNet and we also slightly prefer using aligned SSB configuration for HetNet scenario. Proposal 3: We are ok to use MCS13 for both HomNet and HetNet scenarios. Observation 2: With the INR value of option 2, MMSE-IRC receiver can achieve 1.5~2.7 dB performance gain over MMSE receiver. Proposal 4: For the INR values for HomNet scenario, reuse the interference value assumptions for LTE, i.e., use INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell. Observation 3: Under the agreed baseline INR value for HetNet scenario, MMSE-IRC receiver can achieve around 3 dB performance gain over MMSE receiver, which is enough for requirement definition. Proposal 5: For HetNet scenarios, confirm to use INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. Proposal 6: Cover both 1 cell and 2 cells for the MMSE-IRC testing. Fine to use different cell number assumptions for different deployment scenarios. |
| R4-2200985 | Huawei,HiSilicon | Observation 1: Large gain of MMSE-IRC over MMSE-MRC can be observed for all cases with 2 interference cells compared to that with 1 interference cell and 1interferecnce cell scenario has been covered in CQI test Proposal 1: Consider 2 interference cell for demodulation test. Observation 2: The gain of MMSE-IRC over MMSE-MRC for some cases with INRs specified in option 1 is only about 1 dB Proposal 2: Use INRs 7.77 and 2.29 dB. Observation 3: There is almost no performance degradation for both MMSE-IRC and MMSE-MRC in asynchronized scenario. Proposal 4: If asynchronized scenario is introduced, use following configuration and test applicability rule: - DIP1= -2.23dB and DIP2= -8.06dB (INR1=3.87dB and INR2= -1.96dB) - Time offset: 0.33ms for interference cell 1 and 0.67ms for interference 2 - For FDD, if a UE has passed test for synchronized scenario, there is no need to test the asynchronized scenario. Observation 6: For HetNet scenario.i.e.INR1=11.39dB and INR2=5.45dB with 4RX, target SNR for PBCH is higher than that of PDSCH, therefore is a risk that tested UE can’t access the network. Observation 7: SSB colliding will cause poor accuracy of time/frequency tracking, which will degrade the performance of PDSCH. Proposal 5: Consider no overlapping PBCH resources configuration for serving cell and interference cells. Proposal 6: Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition Proposal 7: Use serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled) |
| R4-2200986 | Huawei,HiSilicon | Simulation results for inter cell MMSE-IRC receiver |
| R4-2200988 | Huawei,HiSilicon | Draft CR for introduction of general applicability section of inter-cell MMSE-IRC receiver in TS 38.101-4 |
| R4-2201215 | Ericsson | Observation 1: All SSBs (serving cell and interference cell(s)) in the same time/frequency resources had already agreed in RAN4 to define RRM requirements with SNR=-6dB. Observation 2: MIB decoding requirement is defined based on multiple trials other than one shot. Proposal 1: RAN4 to define the test cases with the same SSB time/frequency resources for interfering inter-cells. Proposal 2: To avoid the severe SSB interference from neighbour cell, RAN4 can define test case with a higher SINR test point. Proposal 3: RAN4 to consider INRs 5.43 dB and -1.5 dB in homogeneous scenario. Proposal 4: RAN4 to consider 2 interference cells for all tests. |
| R4-2201216 | Ericsson | Observation 1: Compared with MRC, IRC receiver can have about 2dB or even more gain in 70% of maximum throughput SNR point. Observation 2: IRC receiver have gains in both TDLA and TDLC channels. |
| R4-2201965 | Qualcomm Incorporated | Observation 1: Gain of MMSE-IRC over MMSE-MRC is higher with 1 interfering cell compared to 2 interfering cells. Proposal 1: Only consider 1 interfering cell for defining the requirements. Proposal 2: Use 11.39dB and 5.43dB INRs for defining heterogeneous and homogeneous network Intercell Interference requirements, respectively. Proposal 3: Time offset from serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (if modeled). Proposal 4: Intercell Interference tests can be release independent from Rel-15. |

Moderator’s note: Based on guidelines for this meeting, all Draft CR related discussion will be postponed to the next RAN4 meeting (i.e. R4-2200504, R4-2200795 and R4-2200988 will be postponed)

## Open issues summary

### Sub-topic 1-1: Common test parameters for scenario 1

Issue 1-1-1: Network type

- Background

  - Synchronized network is baseline assumption, interested companies are encouraged to bring results for async scenario under the baseline assumption of MMSE-IRC receiver.

    - For asynchronized scenario, reusing LTE configuration of time/frequency offset as starting point.

- Proposals

  - Option 1 (Apple, Nokia): Only consider synchronized network

  - Option 2 (Intel): Further discuss the following options:

    - Option 1: Define requirements for asynchronous scenario with fixed or periodic interference precoder assumptions in Rel-17

    - Option 2: Further discuss the definition of requirements for asynchronous scenario in Rel-18

  - Option 3 (CMCC, China Telecom): Include FDD asynchronized network type in FR1

  - Option 4 (Huawei): Include FDD asynchronized network type with applicability rule: if a UE has passed test for synchronized scenario, there is no need to test the asynchronized scenario

- Recommended WF

  - Collect detailed companies views on pros and cons and whether there are any technical issues to include asynchronized scenario in the scope

Issue 1-1-2: SSB configuration

- Background

  - Option 1: Use SSB Option 1 (All SSBs are in the same time/frequency resources) for all test

  - Option 2: Use SSB Option 2 (Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources) for all test

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

    - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

- Proposals

  - Option 1 (DOCOMO, Nokia, China Telecom, Ericsson)

    - Ericsson: To avoid the severe SSB interference from neighbour cell, RAN4 can define test case with a higher SINR test point.

  - Option 2 (Apple, Huawei)

  - Option 3A (Intel, CMCC)

  - Option 3B (Intel)

- Recommended WF

  - Check whether we can consider Option 1 based on majority companies views or use Option 3A as compromise solution

### Sub-topic 1-2: Target PDSCH parameters for scenario 1

Issue 1-2-1: MCS for synchronous network

- Background

  - Use MCS 13 for homogeneous deployment assumptions

  - Use MCS 13 as baseline for heterogeneous deployment assumptions

    - It can be revised in the next RAN4 meeting in case any technical issue is raised

- Proposals

  - Option 1 (Intel, China Telecom): Confirm MCS 13

- Recommended WF

  - Check whether there are any concerns to use MCS 13 for requirements definition

### Sub-topic 1-3: Interference model for scenario 1

Issue 1-3-1: INR values for Homogeneous deployment assumptions

- Background

  - Further discuss the following options for PDSCH requirements definition for synchronous network

    - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

    - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

  - FFS assumptions for asynchronous network

- Proposals

  - Option 1 (Apple, MediaTek, Nokia, Intel, Huawei): INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

  - Option 2 (China Telecom, Ericsson): INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

  - Option 3 (Qualcomm): Use 5.43 dB in case of 1 interference cell

- Recommended WF

  - Check whether Option 1 can be considered based on majority companies views

Issue 1-3-2: INR values for HetNet deployment assumptions

- Background

  - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

  - Baseline option can be updated in case technical issue will be observed

- Proposals

  - Option 1 (MediaTek, Nokia, China Telecom): INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

  - Option 2 (Intel): INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 7.58 dB in case of 1 interference cell.

  - Option 3 (Qualcomm): INR 11.39 dB in case of 1 interference cell.

- Recommended WF

  - Check whether baseline option (i.e. Option 1) can be confirmed

Issue 1-3-3: Number of explicitly modeled interference cells

- Background

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference cells for heterogeneous deployment assumptions

- Proposals

  - Option 1 (Apple, Qualcomm)

  - Option 2 (DOCOMO, Intel, Huawei, Ericsson)

  - Option 3A (DOCOMO, Nokia, CMCC, China Telecom)

  - Option 3B (MediaTek, China Telecom)

- Recommended WF

  - Check whether Option 3A can be consider as compromise solution

Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS

- Background

  - Option 1: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

  - Option 2: The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

  - Other options are not precluded

- Proposals

  - Option 1 (Nokia, Intel)

  - Option 2 (Apple, Huawei, Qualcomm)

- Recommended WF

  - Collect views on options above

Issue 1-3-5: INR values for asynchronous network (if introduced)

- Proposals

  - Option 1 (Huawei): DIP1= -2.23dB and DIP2= -8.06dB (INR1=3.87dB and INR2= -1.96dB)

- Recommended WF

  - Check views on option above

Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced)

- Proposals

  - Option 1 (Huawei): 0.33ms for interference cell 1 and 0.67ms for interference 2

- Recommended WF

  - Check views on option above

Issue 1-3-7: Details of interference modelling

- Proposals

  - Option 1 (Nokia):

    - Discuss the usage of a generic beamforming model.

    - Discuss the usage of separate interfering UEs along with PRB allocation in interference modelling. We assumed a single UE with full CBW PRB allocation in drafting the TP.

    - NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario. The codebook is TBD.

- Recommended WF

  - Check views on option above

### Sub-topic 1-4: Test cases for scenario 1

Issue 1-4-1: Test cases for FDD synchronous network

- Background

  - Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases:

    - One test case applied for Homogenous (TDLC300-100) for each duplex mode and 2Rx/4Rx

    - One test case applied for HetNet (TDLA30-10) for each duplex mode and 2Rx/4Rx

    - If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode

- Proposals

  - Option 1 (MediaTek): Consider two test cases for FDD with the following settings:

    - Test case for 2Rx: TDLA 30-10, INR1: 11.39 dB, INR2: 5.45 dB

    - Test case for 4Rx: TDLC 300-100, INR1: 5.49 dB

- Recommended WF

  - Check views on option above

### Sub-topic 1-5: Receiver assumptions for scenario 1

Issue 1-5-1: TRS-IC/IM

- Background

  - Further discuss whether to add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

  - No such clarification will be added in the TS 38.101-4

  - Interested companies are encouraged to check with performance for scenarios with and without TRS-IC/IM

- Proposals

  - Option 1 (Huawei): Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

- Recommended WF

  - Check views on option above

### Sub-topic 1-6: Release independency of requirements for scenario 1

Issue 1-5-1: Release independency of Demodulation requirements

- Background

  - RAN4 discuss whether the UE demodulation with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption

- Proposals

  - Option 1 (Qualcomm): Intercell Interference tests can be release independent from Rel-15.

- Recommended WF

  - Check whether it is possible to agree on release independency issue this meeting

### Sub-topic 1-7: Common test parameters for scenario 2

Issue 1-7-1: Common test parameters

- Proposals

  - Option 1 (Intel): Reuse the following Scenario 1 assumptions for further performance analysis for Scenario 2:

    - Synchronized network

    - SCS/CBW: FDD 15kHz/10MHz, TDD 30kHz/40MHz

    - PDCCH allocation in time domain: symbols #0 and #1 of each slot

    - PDSCH allocation in frequency domain for all cells: Full PRB

    - TRS/CSI-RS configuration: Colliding among the cells

    - Propagation conditions: TDLC300-100 or TDLA30-10

    - Antenna configuration: 2x2 and 2x4 ULA Low

- Recommended WF

  - Collect views from interested companies

### Sub-topic 1-8: Target PDSCH parameters for scenario 2

Issue 1-8-1: Target PDSCH parameters

- Proposals

  - Option 1 (Intel): Reuse the following Scenario 1 assumptions for further performance analysis for Scenario 2:

    - PDSCH FRC for serving cell: Rank 1, MCS 13

    - PDSCH allocation in time domain: Mapping type A, Start symbol 2, Duration 12

    - DMRS configuration: DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data

    - PRB bundle size: Set PRB bundle size as 2 for target PDSCH

    - HARQ process number: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline

    - Precoding model: Random precoder with Type I SP codebook

- Recommended WF

  - Collect views from interested companies

### Sub-topic 1-9: Interference model for scenario 2

Issue 1-9-1: Interference model

- Proposals

  - Option 1 (Intel): Consider the following interference modelling assumptions for Scenario 2:

    - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s)

    - Precoding: Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

    - Modulation order: 16QAM randomly modulated symbols.

    - PDSCH mapping in time domain and DMRS configuration:

      - Scenario 2-1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both interference cells, Single symbol front-loaded DMRS and no additional DMRS.

      - Scenario 2-2: Type A PDSCH mapping with starting symbol 2 and duration 5 for interference cell #1 and Type B PDSCH mapping with starting symbol 7 and duration 7 for interference cell #2, Single symbol front-loaded DMRS and no additional DMRS.

- Recommended WF

  - Collect views from interested companies

### Sub-topic 1-10: Receiver assumptions for scenario 2

Issue 1-10-1: Receiver assumptions

- Proposals

  - Option 1 (Intel): Consider the following reference receiver assumptions for Scenario 2:

    - MMSE-IRC with DMRS based covariance matrix estimation

    - Use covariance matrix from the first DMRS for demodulation of resource elements in the first half of slot and covariance matrix from the second DMRS for demodulation of resource elements in the second half of slot.

- Recommended WF

  - Collect views from interested companies

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 1-1: Common test parameters for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1-1: Network type Prefer option 4 Based on our simulation results, there are no much performance degradation if INRs of LTE are reused. Therefore, we agree to introduce this scenario if operators have this deployment in real world. But we don’t think UE is necessary to pass both cases with sync and async because there it has no impact on demodulation processing. What’s more, we prefer to reuse the INRs from LTE which has lower interference power than that of sync scenario, hence we prefer to define the applicability rule that if a UE pass the sync scenario. i.e. high interference level, it can skip the test with async scenario. i.e. lower interference level. Issue 1-1-2: SSB configuration Option 2. - From our simulation results, the SNR @ 1 % BLER of PBCH is higher than SNR @ 70% of max TP pf PDSCH for HetNet scenario. We find there is much simulation results span for PBCH between Huawei and Intel. We clarify our simulation assumptions as follows: - MMSE-MRC (No IRC) processing was used for PBCH - DMRS blind detection was used. - 4 times PBCH combining was performed Based on our understanding, if UE can’t decode PBCH correctly, UE can’t access the network immediately and would try to decode the PBCH in the next SSB period, we think it will waste the test time and is not practical in the real world. - Based on the summary of simulation results from companies, our concern is that: in some cases, the target SNR is lower than INR (For example, case with TDLA,4RX, INRs value equals to 11.39dB and 5.45dB ), how to always guarantee UE access the serving cell. i.e. SNR with lower value rather than interference cell. i.e. INR with higher value if SSB resources from two cells are colliding ? We think it is more practical that UE access the cell with high SNR - What’s more, PBCH is also used for time/frequency tracking. SSB colliding will cause poor accuracy of time/frequency tracking, which will degrade the performance of PDSCH. Based on our initial evaluations, we observed up to 1.8dB for 64QAM and 2.1dB for 256QAM performance gain with SSB-IM under the low network load compared to without IM receiver, it means the interference from the SSB of neighbour cells in the real network due to the colliding SSB configuration has serious impact on PDSCH performance, specific study needs to be conducted in Rel-18 to improve UE performance under network with colliding SSB configuration. Also in order to avoid the case that UE fails the test due to the poor performance of PBCH or poor time/frequency tracking accuracy rather than improper interference handling, it is better to configure no overlapping SSB resources for serving cell and interference cells. |
| Intel | Issue 1-1-1: Network type Option with fixed or periodic interference precoder is suggested to solve the issue that interference level/structure will be different within the slot (which was raised in the previous RAN4 meeting). Probably such assumption is also closer to practical conditions, because precoder is changed with periodic manner in case PMI-based precoder is considered. Same time, in case scenario with testable MMSE-IRC and per-Slot precoder change will be observe, we are fine to consider the definition of requirements for such scenario. As for applicability rule between sync and async case, we need more discussion on procedure how to select the scenarios for testing. Issue 1-1-2: SSB configuration All options are fine for us. We don’t see any issue with PBCH decoding and Time/frequency offset tracking in case PDSCH with MCS13 is considered. |
| CMCC | Issue 1-1-1: Network type Option 3. We understand Option 4 is trying to find a compromise way for both sides. However, with such applicability rule, we have the concerns that async requirements will not be tested, since all the UE will first past the sync requirements. Issue 1-1-2: SSB configuration Option 3A is our first preference, it also considers test coverage. |
| MediaTek | Issue 1-1-1: Network type We do not see the impact to the MMSE-IRC receiver processing under asynchronous network. Hence, to reduce the extra evaluation effort, we prefer not to define test cases for asynchronous network. Issue 1-1-2: SSB configuration We can go for Option 3 to move forward. |
| China Telecom | Issue 1-1-1: Network type Based on ours and Intel’s simulation result, with interference cell’s precoding matrix is updated for each cell, there will be clear performance degradation from sync to async scenario. In addition, we think for NR, it will be more practical for BS to update the precoding matrix more frequently rather than fixed precoding in LTE assumptions. Therefore, we support to include FDD async scenario for test requirement definition for IRC. Same time, we are fine to keep the test case number will be not increased by having some test applicability. Based on the agreements in the last meeting, we are having the following test cases (Copied from Intel’s comment in issue 1-4-1): 2 Rx UE, FDD only – HomoNet 2 Rx FDD test and HetNet 2 Rx FDD test 2 Rx UE, TDD only – HomoNet 2 Rx TDD test and HetNet 2 Rx TDD test 2 Rx UE, FDD and TDD – HomoNet 2 Rx FDD test and HetNet 2 Rx TDD test 4 Rx UE, FDD only – HomoNet 4 Rx FDD test and HetNet 4 Rx FDD test 4 Rx UE, TDD only – HomoNet 4 Rx TDD test and HetNet 4 Rx TDD test 4 Rx UE, FDD and TDD – HomoNet 4 Rx FDD test and HetNet 4 Rx TDD test We propose the following: For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for aync scenario and 1 HetNet test for sync scenario. For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively. Issue 1-1-2: SSB configuration We support option 2 and we can also compromise to option 3A to have non-overlapped SSB for HetNet only. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Network type We thank Intel for their interesting contribution. We would like to clarify what were the assumption on the DMRS detection of interference cells to find the covariance in the case of asynchronous. E.g, were the DMRS positions perfectly or blindly detected and/or shift compensated Baseline reference receiver may not be the same in asynchronous case and hence option 4 does not look valid. The IRC receiver should be held to the same standard for both asynchronous and synchronous deployment. Issue 1-1-2: SSB configuration We are ok with option 1 and compromise proposed by Ericsson |
| Ericsson | Issue 1-1-1: Network type Option 3 first. We’re also fine to include further applicability rule, but at least UE may test some async. cases. Issue 1-1-2: SSB configuration Option 1. As we mentioned in our tdoc, to avoid the severe SSB interference from neighbour cell, RAN4 can define test case with a higher SINR test point. |
| Apple | Issue 1-1-1: Network type We support option 1. If we assume baseline MMSE-IRC processing without any advanced processing for UE to take into account the uneven interference on 2 DMRS symbols then we could expect degradation in performance. But, companies results are showing that performance is comparable to sync assumption, without advanced processing, so we don’t understand why further testcases need to be introduced. Since the results also depend on what assumptions are used for interference cells overall it doesn’t seem very critical to introduce requirements with async network assumption. Issue 1-1-2: SSB configuration We support option 2. We would like more clarification on Ericsson’s proposal if based on the agreed simulation assumptions the operating SINR would see impact due to SSB interference , is the recommendation to add additional margin to the requirement or choose a different simulation assumption that results in increased operating SINR/SNR? Since we are defining PDSCH demod requirements here we should ensure that there is no issue with UE accessing the network, decoding PDCCH etc. |
| Qualcomm | Issue 1-1-1: Network type Prefer Option 1 because UE implementation of MMSE-IRC is agnostic to sync or async scenario. Issue 1-1-2: SSB configuration No strong preference. |
| Docomo | Issue 1-1-2: SSB configuration We think that Option 1 is typical NW deployment for both HomNet and HetNet. However, for the sake of progress, Option 3A is also acceptable for us as a compromise solution |

#### Sub-topic 1-2: Target PDSCH parameters for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-2-1: MCS for synchronous network OK with option 1 |
| Intel | Issue 1-2-1: MCS for synchronous network Support option 1 |
| MediaTek | Issue 1-2-1: MCS for synchronous network OK with Option 1. |
| China Telecom | Issue 1-2-1: MCS for synchronous network Support the recommended WF. |
| Nokia, Nokia Shanghai Bell | Issue 1-2-1: MCS for synchronous network We are ok with defining requirements based on MCS13 as seems to be consensus. |
| Ericsson | Issue 1-2-1: MCS for synchronous network Support the recommended WF. |
| Apple | Issue 1-2-1: MCS for synchronous network Support the recommended WF. |
| Qualcomm | Issue 1-2-1: MCS for synchronous network Ok with Option 1. |
| Docomo | Issue 1-2-1: MCS for synchronous network We support the recommended WF. |

#### Sub-topic 1-3: Interference model for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-3-1: INR values for Homogeneous deployment assumptions Option 1. The INRs in option 1 shows higher performance gain for MMSE-IRC over baseline from our simulation results Issue 1-3-2: INR values for HetNet deployment assumptions We prefer option 1 with 2 interference cell Issue 1-3-3: Number of explicitly modeled interference cells We prefer to use 2 interference cells to get higher performance gain. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS Since basic MMSE-IRC is used, UE is agnostic to time/frequency offset, we support to use option 2 since it has been used in Rel-16 Multi-TRP scenario. i.e. Option 1 Issue 1-3-5: INR values for asynchronous network (if introduced) We prefer to reuse the LTE value for asynchronous network (if introduced) to reduce work effort Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) We prefer to reuse the LTE value for asynchronous network (if introduced) to reduce work effort Issue 1-3-7: Details of interference modelling We don’t think interference should be simulated in PDCCH location in interference cell since our purpose is to verify the PDSCH performance so other channel’s performance should be guaranteed. |
| Intel | Issue 1-3-1: INR values for Homogeneous deployment assumptions Support Option 1, because it allows to better verify MMSE-IRC processing and can be observe in the NR deployment based on our system level analysis in comparisons to Option 2. Issue 1-3-2: INR values for HetNet deployment assumptions We suggest to consider Option 2. We think that INR value for scenario with 1 interference cell for Option 2 is more aligned with INR definition in TR 36.866, which was used as reference for considered values. Issue 1-3-3: Number of explicitly modeled interference cells We are fine to consider Option 3A as compromise solution. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS Support Option 1. Option 2 for eMIMO is considered because intra-cell multi-TRP scenario was assumed, and it was important to ensure that signals from different TRPs are within CP to avoid any issue with serving signal reception. Here we consider scenario with inter-cell interference. Therefore, we don’t have similar restrictions as for eMIMO multi-TRP case and Option 1 can be considered. Issue 1-3-5: INR values for asynchronous network (if introduced) We think that more analysis is needed. At least, we checked the INR values used for synchronous case. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) We are fine to consider Option 1. Issue 1-3-7: Details of interference modelling Generic beamforming model is defined in Section B.4.1 of TS 38.101-4 for serving signal. We can use this model also for interference signal. Methodology with multiple UEs modelling for interference cells was defined for LTE NAICS requirements to consider more practical conditions. Same time, it is not used for LTE MMSE-IRC requirements. We are open to discuss interference modelling procedure from LTE NAICS requirements. We support the third bullet, it is aligned with LTE requirements for scenarios with inter-cell interference. |
| CMCC | Issue 1-3-2: INR values for HetNet deployment assumptions We propose Option 1 INR 4.84dB in last meeting. This proposal considers 2 interference cells (INRs 11.39 and 5.45 dB) in scenario while UE can only mitigate the interference from the first dominant cell. While Option 2 seems also reasonable, which give larger INR in case of 1 interference cell in HetNet scenario. We would like to do more investigation based on Option 2. Issue 1-3-3: Number of explicitly modeled interference cells We support Option 3A, which not only considers real network deployment, but also considers test coverage. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS Option 1. Issue 1-3-5: INR values for asynchronous network (if introduced) If Option 1 in Issue 1-3-1 is used in sync network, then Option 2 can be used in async network. Vise versa. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) Ok with Option 1 |
| MediaTek | Issue 1-3-1: INR values for Homogeneous deployment assumptions We are OK with Option 1. Also, we slightly prefer to consider only 4Rx with one interference cell case in Option 1. Issue 1-3-2: INR values for HetNet deployment assumptions We are OK with Option 1. Also, we slightly prefer to consider only 2Rx with two interference cell case in Option 1. Issue 1-3-3: Number of explicitly modeled interference cells We are OK with different explicitly modeled interference cells for homogeneous and heterogeneous network. Also, we slightly prefer Option 3B. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS We support Option 2 which was modelled in the mTRP scenario. |
| China Telecom | Issue 1-3-1: INR values for Homogeneous deployment assumptions We support option 2 because we have already observed enough IRC gain under this configuration. If majority companies would prefer larger INR values for better verify the IRC performance, we are also ok with using option 1. Issue 1-3-2: INR values for HetNet deployment assumptions It seems we are using INR 5.49dB for 1 interference cell case for HomNet, it will be strange we have an even smaller INR value for HetNet (4.84 dB for 1 interference cell). Thus we suggest to have more investigation and check whether option 2 can be a reasonable INR value. Issue 1-3-3: Number of explicitly modeled interference cells We support option 3A. Issue 1-3-5: INR values for asynchronous network (if introduced) We think there is no big difference between sync and async scenarios in terms of INR values. We support to reuse the same INR value we use for sync scenarios. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) We are fine with Option 1 which is same with LTE test assumption. |
| Nokia, Nokia Shanghai Bell | Issue 1-3-1: INR values for Homogeneous deployment assumptions We are ok with proposed option 1 as it gives higher gain with the IRC receiver Issue 1-3-2: INR values for HetNet deployment assumptions We are ok with proposed option 1 as we did not observe any issues with this option Issue 1-3-3: Number of explicitly modeled interference cells We are ok with compromise solution of option 3A as it mirrors the real world deployments Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS The cell phase synchronization minimum accuracy requirement in 38.133 section 7.4 is set to be 3 us. So we believe this value should be used to define the minimum performance requirement. Issue 1-3-5: INR values for asynchronous network (if introduced) In case asynchronous is introduced we propose to reuse the INR values from synchronous case. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) We are ok with proposed value for initial simulations Issue 1-3-7: Details of interference modelling We are ok with Intel’s view about adding reference to section B.4.1 in 38.101-4 regarding beamforming when defining interference model. As multiple-UE configuration was used only for LTE NAICS and not for LTE MMSE-IRC requirements, we propose to go ahead with a single UE per interference cell with full CBW PRB allocation for NR MMSE-IRC requirements. Lastly, interference model for unallocated region of interfering cell PDCCH need to be defined. We propose QPSK randomly modulated symbols with random precoding for the number of antennas configured in the requirement |
| Ericsson | Issue 1-3-1: INR values for Homogeneous deployment assumptions We’re fine with both option 1 and 2. Issue 1-3-2: INR values for HetNet deployment assumptions We’re fine with option 1. Issue 1-3-3: Number of explicitly modeled interference cells We are fine to consider Option 3A as a compromise solution. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS Option 1. The definition for sync. is based on 3us in TS38.133 which should be reused to define the sync. test case for Inter-cell Demod. Issue 1-3-5: INR values for asynchronous network (if introduced) We support to reuse the same INR value as sync scenarios. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) Ok with Option 1 Issue 1-3-7: Details of interference modelling Ok with Option 1 |
| Apple | Issue 1-3-1: INR values for Homogeneous deployment assumptions We support option 1. Based on simulation results we don’t see much difference between the 2 interference levels. Issue 1-3-2: INR values for HetNet deployment assumptions We support option 1. For option 2, since not all companies presented results, we could have it as an option For option 3, the INR for 1 cell is much higher than agreed in RAN4#101, we would like to understand the justification. Issue 1-3-3: Number of explicitly modeled interference cells We support option 1. We still don’t see the benefit of introducing 2 interference cells as it doesn’t impact UE processing and only complicates test set-up. The same performance is reached with lower INR with one cell compared to 2 cells. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS We support option 2. With 3us time offset for 30KHz SCS the sync assumption is no longer valid in our understanding. Issue 1-3-5: INR values for asynchronous network (if introduced) We don’t support introducing requirements in async. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) We don’t support introducing requirements in async. Issue 1-3-7: Details of interference modelling For the 2nd bullet we aren’t sure why we need to model interfering UEs. For bullet 3 it might be more practical to assume PDCCH transmission from interference cells rather than whole control region filled with randomly modulated QPSK symbols. |
| Qualcomm | Issue 1-3-1: INR values for Homogeneous deployment assumptions We prefer Option 2 for 2 cell, Option 1 for 1 cell. Issue 1-3-2: INR values for HetNet deployment assumptions We can compromise to Option 1. Issue 1-3-3: Number of explicitly modeled interference cells We prefer Option 1 because adding 2nd cell distorts Rnn structure towards white noise compared to single cell and that is also reflected in simulation results in our paper showing higher gain of IRC over MRC with 1 cell compared to 2 cells. Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS Prefer Option 2. The definition for sync. is based on 3us in TS38.133 which is violated in Option 1 because difference between 2 interfering cells is 4us. Issue 1-3-5: INR values for asynchronous network (if introduced) Prefer to discuss after decision on whether to define requirements for async scenario. Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) Prefer to discuss after decision on whether to define requirements for async scenario. Issue 1-3-7: Details of interference modelling Prefer to discuss after decision on whether to define requirements for async scenario. |
| Docomo | Issue 1-3-1: INR values for Homogeneous deployment assumptions We support Option 1. Issue 1-3-3: Number of explicitly modeled interference cells We are fine with Option 3A as a compromise solution. Issue 1-3-5: INR values for asynchronous network (if introduced) We think that more analysis is needed. If there is no big difference between sync and async scenarios in terms of INR values, we prefer to reuse the same INR value for sync scenarios. |

#### Sub-topic 1-4: Test cases for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-4-1: Test cases for FDD synchronous network We prefer to keep align with UE supporting different antennas |
| Intel | Issue 1-4-1: Test cases for FDD synchronous network Based on agreement from previous meeting, we will have the following test scope for different type of UEs: 2 Rx UE, FDD only – HomoNet 2 Rx FDD test and HetNet 2 Rx FDD test 2 Rx UE, TDD only – HomoNet 2 Rx TDD test and HetNet 2 Rx TDD test 2 Rx UE, FDD and TDD – HomoNet 2 Rx FDD test and HetNet 2 Rx TDD test 4 Rx UE, FDD only – HomoNet 4 Rx FDD test and HetNet 4 Rx FDD test 4 Rx UE, TDD only – HomoNet 4 Rx TDD test and HetNet 4 Rx TDD test 4 Rx UE, FDD and TDD – HomoNet 4 Rx FDD test and HetNet 4 Rx TDD test The more clarification on Option 1 proposal is needed, to understand the changes which are suggested in comparison to previous meeting agreement. |
| CMCC | Follow the agreements in last meeting and consider following test cases: Test case for HomoNet, FDD, 2Rx Test case for HomoNet, FDD, 4Rx Test case for HomoNet, TDD, 2Rx Test case for HomoNet, TDD, 4Rx Test case for HetNet, FDD, 2Rx Test case for HetNet, FDD, 4Rx Test case for HetNet, TDD, 2Rx Test case for HetNet, TDD, 4Rx |
| MediaTek | Issue 1-4-1: Test cases for FDD synchronous network Our intention is to further minimize the test cases for UE under the scenario of inter-cell interference. We think the difference in Homo and HetNet is the difference in INR values. We try to cover both HomoNet and HetNet INR values but not to define HetNet/HomoNet for both 2Rx and 4Rx. For example, we can have: FDD, 2Rx, HetNet FDD, 4Rx, HomoNet TDD, 2Rx, HetNet (HomoNet) TDD, 4Rx, HetNet (or HomoNet) If there are concerns that UEs only support 2Rx cannot test for HomoNet, we can follow the previous agreements. |
| China Telecom | Issue 1-4-1: Test cases for FDD synchronous network Based on the discussion in the last meeting, we prefer to keep last meeting agreements to have the same test assumptions for 2Rx and 4Rx cases. |
| Nokia, Nokia Shanghai Bell | Issue 1-4-1: Test cases for FDD synchronous network We are ok with proposed channel models and INR values for each test case. We would though like to have both 2Rx and 4 Rx tested for each test. |
| Ericsson | Issue 1-4-1: Test cases for FDD synchronous network Keep the agreements as last meeting. |
| Apple | Issue 1-4-1: Test cases for FDD synchronous network Based on agreements in last meeting we define the same test for 2RX and 4RX, but we already have applicability rule that UE would be only tested for 4RX if it supports both 2RX and 4RX. |
| Qualcomm | Issue 1-4-1: Test cases for FDD synchronous network Prefer to keep the agreements as last meeting. |
| Docomo | Issue 1-4-1: Test cases for FDD synchronous network We prefer to keep last meeting agreements. |

#### Sub-topic 1-5: Receiver assumptions for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-5-1: TRS-IC/IM Option 1 As per our evaluation, there is up to 3.2dB performance gain for 64QAM with CR=0.45 with interference cell TRS, it means there is obvious impact on PDSCH performance due to poor accuracy of time-frequency tracking .Although RAN4 agreed to configure colliding TRS in previous meeting, we are not sure if it is emulated in companies’ initial simulation results? As RAN 4 has agreed to define the requirements with TRS colliding which may cause poor performance of time and frequency tracking, also with the consideration of not mixing the IRC processing with TRS-IM processing, minimal requirements with no TRS-IC/IM assumptions should be clarified in the simulation assumptions and TR. The specific TRS interference impact is under discussion in Rel-18 and can be further analysed. |
| Intel | Issue 1-5-1: TRS-IC/IM Based on our analysis from RAN4 #99 meeting (R4-2109198), there is limited performance impact of TRS-based parameters estimation on PDSCH performance in scenarios with serving TRS interfered by TRS and scenario with interference free serving TRS (i.e. ideal IC) for scenario with MCS 4 and 13. Therefore, we don’t see a strong need to define such restriction for agreed assumptions. Same time, we don’t have technical concern to include it in the TR. |
| MediaTek | Issue 1-5-1: TRS-IC/IM We think TRS interference cancellation/mitigation can be left to UE implementation and it is not necessary to add clarification in the simulation assumption and/or TR. |
| China Telecom | Issue 1-5-1: TRS-IC/IM To address HW’s concern, we are fine to add such clarification in the simulation assumption only. |
| Nokia, Nokia Shanghai Bell | Issue 1-5-1: TRS-IC/IM TRS interference cancellation/mitigation should be left to UE implementation |
| Ericsson | Issue 1-5-1: TRS-IC/IM Not necessary to add clarification in the simulation assumption and TR. It’s up to UE implementation. |
| Apple | Issue 1-5-1: TRS-IC/IM We don’t see the need for such clarification. Based on simulation results a few meetings ago we didn’t see any impact with overlapping vs non-overlapping TRS. Also, we don’t understand where such clarification is to be added. We don’t have a study phase or TR for MMSE-IRC for ICI in our understanding. |
| Qualcomm | Issue 1-5-1: TRS-IC/IM Same view as other companies. It’s up to UE implementation and it’s not needed to add to simulation assumptions. |

#### Sub-topic 1-6: Release independency of requirements for scenario 1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-5-1: Release independency of Demodulation requirements The issue is related to that with UE capability signaling. |
| Intel | Issue 1-5-1: Release independency of Demodulation requirements We support to define MMSE-IRC requirements for Scenario 1 realease independent from Rel-15. @Huawei: We think that decision on capability signalling has dependance on this issue. In case requirements are defined in release independent manner, definition of UE capability is not needed. |
| MediaTek | Issue 1-5-1: Release independency of Demodulation requirements For scenario 1, we support to define MMSE-IRC requirements to be release independent from Rel-15. |
| China Telecom | Issue 1-5-1: Release independency of Demodulation requirements We support to define MMSE-IRC requirements for Scenario 1 release independent from Rel-15. |
| Ericsson | Issue 1-5-1: Release independency of Demodulation requirements Support to define MMSE-IRC requirements for Scenario 1 release independent from Rel-15. |
| Apple | Issue 1-5-1: Release independency of Demodulation requirements We don’t see any issue with defining these requirements as release independent from Rel-15, but we would like to clarify and confirm that its only for Scenario 1 and sync case. |
| Qualcomm | Issue 1-5-1: Release independency of Demodulation requirements Ok with Option 1 for scenario 1 since UE is not using any Rel-16 or Rel-17 features for these requirements. |
| Docomo | Issue 1-5-1: Release independency of Demodulation requirements We support to define MMSE-IRC requirements for Scenario 1 release independent from Rel-15 |

#### Sub-topic 1-7: Common test parameters for scenario 2

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-7-1: Common test parameters We should discuss whether to introduce the performance requirements for scenario 2 in Rel-17 firstly. @Intel: We wonder that how UE have the knowledge of interference distribution and perform different scheme to handle that? Does NWA needed? |
| Intel | Issue 1-7-1: Common test parameters @Huawei: This scenario is included in the WI. Therefore, discussion on introduction for such requirements is not needed. Taking into account that for scenario 1, we have limited set of open issues, we think that we can start discussion on Scenario 2. As receiver assumption, we suggest to consider that UE uses per half slot processing. UE can estimate the interference level on first and second DMRS symbols separatly, make the comparison and decide whether to make the averaging of covariance matrix within slot or use per half slot processing. |
| CMCC | Issue 1-7-1: Common test parameters We are ok with discuss Scenario 2 in R17. For the receiver assumption from Intel’s comment, we wonder does UE should do the same processing even it is in Scenario 1? Since the scenario is transparent to UE (i.e. without NWA). |
| China Telecom | Issue 1-7-1: Common test parameters Firstly, we would like to thank Intel for the detailed investigation and proposals. Secondly, in our understanding, PDSCH mapping type B allows very flexible resource mapping to cope with the smaller time delay requirement for URLLC. We think more scenarios as shown below, should also be considered to verify the feasibility of the baseline IRC receiver for this scenario 2: - In the RANP discussion, there was company pointed out in RP-210317 that in the situation where the serving cell DMRS cannot observe the ICI (as shown below), UE will still apply MMSE receiving process. Cell1, UE1 Cell2, UE2 - Shorter symbol length compared to Intel’s proposal Due to flexible mapping in time domain, there exists situation that only DMRS symbol(s) and a few or no data symbol is interfered by ICI. In that case MMSE-MRC receiver may have better or similar performance with MMSE-IRC receiver. - We should also consider non-slot based PDSCH transmission for the serving cell, where UE is also very likely to be allocated only 1 DMRS symbol due to the shorter time length. |
| Nokia, Nokia Shanghai Bell | Issue 1-7-1: Common test parameters We are ok with reusing the parameters from scenario 1. SSB configuration should also be reused from scenario 1 since the SSB discussion is not impacted by the scenario choice. |
| Ericsson | Focus on scenario 1 and deprioritize the scenario 2 at least in this meeting. |
| Apple | Issue 1-7-1: Common test parameters For scenario 2 we would like to first understand the baseline assumption. 1. Is it MMSE-IRC processing or more advanced receiver? 2. Is UE made aware of such interference to handle it appropriately? 3. How is it different from what is proposed for R18 uneven / time-selective interference? We would first like to agree on baseline assumptions before starting to discuss simulation parameters. |
| Qualcomm | We should first discuss whether to define requirements for scenario 2. After that decision, we can start discussing the parameters. Based on Intel’s proposal, UE is supposed to do half-slot based DMRS processing which is an enhanced receiver and has never been considered in RAN4 before. Usually, RAN4 first studies any new receiver to evaluate its gains and then decides to define such requirements based on that study. This will also need new UE capability since it is an enhanced receiver. All signalling related aspects need to be finalized by February RAN4 meeting. Therefore, we suggest not to consider this scenario in Rel-17. |
| Docomo | Issue 1-7-1: Common test parameters We are fine with using Option 1 as baseline for Scenario 2. On the other hand, we think that we should also consider test case that non-slot based PDSCH transmission for the serving cell. |

#### Sub-topic 1-8: Target PDSCH parameters for scenario 2

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 1-8-1: Target PDSCH parameters We are ok with reusing PDSCH parameters from scenario 1 |
| Apple | We would first like to agree on baseline assumptions in Issue 1-7-1 before starting to discuss simulation parameters. |

#### Sub-topic 1-9: Interference model for scenario 2

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 1-9-1: Interference model Both scenario 2-1 and 2-2 are biased towards good performance with half slot based reference receiver. It should be discussed if the minimum performance requirement in such a biased scenario is acceptable or more challenging scenario should be discussed. |
| Apple | We would first like to agree on baseline assumptions in Issue 1-7-1 before starting to discuss simulation parameters. |

#### Sub-topic 1-10: Receiver assumptions for scenario 2

| Company | Comments |
| --- | --- |
| Apple | We would first like to agree on baseline assumptions in Issue 1-7-1 . |
| XXX |  |

## Summary for 1st round

### Open issues

#### Sub-topic 1-1: Common test parameters for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Network type | Tentative agreements: N/A Candidate options: - Option 1 (MediaTek, Apple, Qualcomm): Only consider synchronized network - Option 2 (Intel): Define requirements for asynchronous scenario with fixed or periodic interference precoder assumptions in Rel-17 - Option 3 (Intel, CMCC, [Nokia], Ericsson): Include FDD asynchronized network type in FR1 - Intel: In case scenario with testable MMSE-IRC and per-Slot precoder change will be observe - Option 4 (Huawei, China Telecom): Include FDD asynchronized network type with applicability rule - Option 4A (Huawei): if a UE has passed test for synchronized scenario, there is no need to test the asynchronized scenario - Option 4B (China Telecom): - For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for aync scenario and 1 HetNet test for sync scenario. - For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively. Recommendations for 2nd round: - Continue discussion on the following options - Option 1: Only consider synchronized network - Option 2: Include FDD asynchronized network type with applicability rule: - Option 2A: if a UE has passed test for synchronized scenario, there is no need to test the asynchronized scenario - Option 2B: - For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for aync scenario and 1 HetNet test for sync scenario. - For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively. |
| Issue 1-1-2: SSB configuration | Tentative agreements: N/A Candidate options: - Option 1 (Intel, Nokia, Ericsson, Qualcomm, Docomo): Use SSB Option 1 (All SSBs are in the same time/frequency resources) for all test - Ericsson: to avoid the severe SSB interference from neighbour cell, RAN4 can define test case with a higher SINR test point. - Option 2 (Huawei, Intel, [China Telecom], Apple, Qualcomm): Use SSB Option 2 (Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources) for all test - Option 3 (Intel, CMCC, MediaTek, China Telecom, Qualcomm, Docomo): Use different assumptions for different deployment scenarios: - Option 3A (CMCC, China Telecom, Docomo): SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions Recommendations for 2nd round: - Check whether Option 3A can be considered as compromise solution |

#### Sub-topic 1-2: Target PDSCH parameters for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-2-1: MCS for synchronous network | Tentative agreements: Use MCS 13 for requirements definition with homogeneous and heterogeneous deployment assumptions |

#### Sub-topic 1-3: Interference model for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-3-1: INR values for Homogeneous deployment assumptions | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, Intel, MediaTek, China Telecom, Nokia, Ericsson, Apple, Docomo): INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell - MediaTek: we slightly prefer to consider only 4Rx with one interference cell case in Option 1. - Option 2 (China Telecom, Ericsson): INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell - Option 3 (Qualcomm): Option 2 for 2 cell, Option 1 for 1 cell. Recommendations for 2nd round: - Check whether Option 1 can be agreed |
| Issue 1-3-2: INR values for HetNet deployment assumptions | Tentative agreements: N/A Candidate options: - 2 interference cells - Option 1 (Huawei, Intel, CMCC, MediaTek, Nokia, Ericsson, Apple, Qualcomm): INRs 11.39 and 5.45 dB - MediaTek: We slightly prefer to consider only 2Rx with two interference cell case in Option 1. - 1 interference cell - Option 1 (CMCC, Nokia, Ericsson, Apple, Qualcomm): INR 4.84 dB. - Option 2 (Intel): INR 7.58 dB - CMCC: We would like to do more investigation based on Option 2. - China Telecom: We suggest to have more investigation and check whether option 2 can be a reasonable INR value. - Apple: We could have it as an option Recommendations for 2nd round: - Stop discuss for this meeting. Continue discussion in the next meeting for the following options - Use INRs 11.39 and 5.45 dB in case of 2 interference cells are modelled - Select one of the following options for scenario with 1 interference cell - Option 1: INR 4.84 dB. - Option 2: INR 7.58 dB |
| Issue 1-3-3: Number of explicitly modeled interference cells | Tentative agreements: N/A Candidate options: - Option 1 (Apple, Qualcomm): 1 interference cell for all tests - Option 2 (Huawei): 2 interference cells for all tests - Option 3 (Intel, CMCC, MediaTek, China Telecom, Nokia, Ericsson, Docomo): Use different assumptions for different deployment scenarios: - Option 3A (Intel, CMCC, China Telecom, Nokia, Ericsson, Docomo): 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions - Option 3B (MediaTek): 1 interference cell for homogeneous deployment assumptions and 2 interference cells for heterogeneous deployment assumptions Recommendations for 2nd round: - Check whether Option 3A can be agreed |
| Issue 1-3-4: Time and frequency offsets for synchronized network with 30 kHz SCS | Tentative agreements: N/A Candidate options: - Option 1 (Intel, CMCC, Nokia, Ericsson): The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled) - Option 2 (Huawei, MediaTek, Apple, Qualcomm): The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled) Recommendations for 2nd round: - Continue discussion on two options |
| Issue 1-3-5: INR values for asynchronous network (if introduced) | Tentative agreements: N/A Candidate options: - Option 1 (Huawei): DIP1= -2.23dB and DIP2= -8.06dB (INR1=3.87dB and INR2= -1.96dB) - Option 2 (Intel, Docomo): More analysis is needed - Docomo: If there is no big difference between sync and async scenarios in terms of INR values, we prefer to reuse the same INR value for sync scenarios. - Option 3 (CMCC): If Option 1 in Issue 1-3-1 is used in sync network, then Option 2 can be used in async network. Vise versa. - Option 4 (China Telecom, Nokia, Ericsson): Reuse the same INR value we use for sync scenarios Recommendations for 2nd round: - Stop discussion for this meeting. Discuss the following options in case requirements will be introduced - Option 1: DIP1= -2.23dB and DIP2= -8.06dB (INR1=3.87dB and INR2= -1.96dB) - Option 2: Reuse INR values options from sync scenario - Other options are not precluded |
| Issue 1-3-6: Time and frequency offsets for asynchronous network (if introduced) | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, Intel, CMCC, China Telecom, Nokia, Ericsson): 0.33ms for interference cell 1 and 0.67ms for interference 2 Recommendations for 2nd round: - Stop discussion for this meeting. Use 0.33ms for interference cell 1 and 0.67ms for interference 2 as baseline in case requirements will be introduced |
| Issue 1-3-7: Details of interference modelling | Tentative agreements: N/A Candidate options: - Option 1 (Nokia, Intel, Ericsson): - Discuss the usage of a generic beamforming model. - Intel: Generic beamforming model is defined in Section B.4.1 of TS 38.101-4 for serving signal. We can use this model also for interference signal. - Nokia: We are ok with Intel’s view about adding reference to section B.4.1 in 38.101-4 regarding beamforming when defining interference model. - Discuss the usage of separate interfering UEs along with PRB allocation in interference modelling. We assumed a single UE with full CBW PRB allocation in drafting the TP. - Intel: We are open to discuss interference modelling procedure from LTE NAICS requirements. - Nokia: Go ahead with a single UE per interference cell with full CBW PRB allocation for NR MMSE-IRC requirements. - Apple: we aren’t sure why we need to model interfering UEs. - NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario. The codebook is TBD. - Huawei: Interference should be simulated in PDCCH location in interference cell since our purpose is to verify the PDSCH performance so other channel’s performance should be guaranteed. - Option 2 (Huawei): We don’t think interference should be simulated in PDCCH location in interference cell since our purpose is to verify the PDSCH performance so other channel’s performance should be guaranteed. - Option 3 (Apple): Assume PDCCH transmission from interference cells rather than whole control region filled with randomly modulated QPSK symbols. Recommendations for 2nd round: - Further discuss the interference modelling in PDCCH region: - Option 1: NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario. - Option 2: No interference signal in PDCCH region - Option 3: Assume PDCCH transmission from interference cells |

#### Sub-topic 1-4: Test cases for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-4-1: Test cases for FDD synchronous network | Tentative agreements: Keep previous meeting agreement - Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases: - One test case applied for Homogenous (TDLC300-100) for each duplex mode and 2Rx/4Rx - One test case applied for HetNet (TDLA30-10) for each duplex mode and 2Rx/4Rx - If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode |

#### Sub-topic 1-5: Receiver assumptions for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-5-1: TRS-IC/IM | Tentative agreements: N/A Candidate options: - Option 1 (Huawei): Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition - Intel: we don’t have technical concern to include it in the TR. - China Telecom: we are fine to add such clarification in the simulation assumption only. - Option 2 (MediaTek, Ericsson, Apple, Qualcomm): It is not necessary to add clarification in the simulation assumption and/or TR. Recommendations for 2nd round: - Check whether Option 2 can be agreed |

#### Sub-topic 1-6: Release independency of requirements for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-5-1: Release independency of Demodulation requirements | Tentative agreements: N/A Candidate options: - Option 1 (Intel, MediaTek, Ericsson, Apple, Qualcomm, Docomo): Inter-cell interference tests can be release independent from Rel-15 for scenario 1 - Option 2 (Huawei): The issue is related to that with UE capability signaling. Recommendations for 2nd round: - Continue discussion in the Sub-topic 4-1 |

#### Sub-topic 1-7: Common test parameters for scenario 2

|  | Status summary |
| --- | --- |
| Issue 1-7-0 (New): Whether to define requirements for Scenario 2 in Rel-17. | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, [China Telecom], Apple): FFS - Option 2 (Intel, CMCC, [Nokia], [Docomo]): Yes - Option 3 (Qualcomm): No - Option 4 (Ericsson): Focus on scenario 1 and deprioritize the scenario 2 at least in this meeting. Recommendations for 2nd round: - Stop discussion for this meeting. Encourage more companies inputs for the next RAN4 meeting on questions and scenarios provided by companies in this meeting. |
| Issue 1-7-1: Common test parameters | Tentative agreements: N/A Candidate options: - Option 1 (Intel): Reuse the following Scenario 1 assumptions for further performance analysis for Scenario 2: - Synchronized network - SCS/CBW: FDD 15kHz/10MHz, TDD 30kHz/40MHz - PDCCH allocation in time domain: symbols #0 and #1 of each slot - PDSCH allocation in frequency domain for all cells: Full PRB - TRS/CSI-RS configuration: Colliding among the cells - Propagation conditions: TDLC300-100 or TDLA30-10 - Antenna configuration: 2x2 and 2x4 ULA Low - Option 2 (China Telecom): Additional scenarios: - Scenario 1: Situation where the serving cell DMRS cannot observe the ICI - Scenario 2: Shorter symbol length - Scenario 3: Non-slot based PDSCH transmission for the serving cell Recommendations for 2nd round: - Stop discussion for this meeting. Capture companies comments and question in WF. Continue discussion next meeting. |

#### Sub-topic 1-8: Target PDSCH parameters for scenario 2

|  | Status summary |
| --- | --- |
| Issue 1-8-1: Target PDSCH parameters | Tentative agreements: N/A Candidate options: - Option 1 (Intel): Reuse the following Scenario 1 assumptions for further performance analysis for Scenario 2: - PDSCH FRC for serving cell: Rank 1, MCS 13 - PDSCH allocation in time domain: Mapping type A, Start symbol 2, Duration 12 - DMRS configuration: DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data - PRB bundle size: Set PRB bundle size as 2 for target PDSCH - HARQ process number: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline - Precoding model: Random precoder with Type I SP codebook Recommendations for 2nd round: - Stop discussion for this meeting. Capture companies comments and question in WF. Continue discussion next meeting |

#### Sub-topic 1-9: Interference model for scenario 2

|  | Status summary |
| --- | --- |
| Issue 1-9-1: Interference model | Tentative agreements: N/A Candidate options: - Option 1 (Intel): Consider the following interference modelling assumptions for Scenario 2: - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) - Precoding: Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. - Modulation order: 16QAM randomly modulated symbols. - PDSCH mapping in time domain and DMRS configuration: - Scenario 2-1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both interference cells, Single symbol front-loaded DMRS and no additional DMRS. - Scenario 2-2: Type A PDSCH mapping with starting symbol 2 and duration 5 for interference cell #1 and Type B PDSCH mapping with starting symbol 7 and duration 7 for interference cell #2, Single symbol front-loaded DMRS and no additional DMRS. Recommendations for 2nd round: - Stop discussion for this meeting. Capture companies comments and question in WF. Continue discussion next meeting |

#### Sub-topic 1-10: Receiver assumptions for scenario 2

|  | Status summary |
| --- | --- |
| Issue 1-10-1: Receiver assumptions | Tentative agreements: N/A Candidate options: - Option 1 (Intel): Consider the following reference receiver assumptions for Scenario 2: - MMSE-IRC with DMRS based covariance matrix estimation - Use covariance matrix from the first DMRS for demodulation of resource elements in the first half of slot and covariance matrix from the second DMRS for demodulation of resource elements in the second half of slot. Recommendations for 2nd round: - Stop discussion for this meeting. Capture companies comments and question in WF. Continue discussion next meeting |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2203008 |  |

### Open issues

Issue 1-1-1: Network type

- Proposals

  - Option 1: Only consider synchronized network

  - Option 2: Include FDD asynchronized network type with applicability rule:

    - Option 2A: if a UE has passed test for synchronized scenario, there is no need to test the asynchronized scenario

    - Option 2B:

      - For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for async scenario and 1 HetNet test for sync scenario.

      - For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively.

- Way forward

  - Check whether companies can compromise to Option 1 or Option 2.

| Company | Comments |
| --- | --- |
| Huawei | Option 1 or option 2B Based on our understanding, UE is agnostic to time offset and according to our evaluations there is no performance difference between async and sync. Option 2B seems replace all HomNet sync cases to HetNet async cases. We can comprise to option 2B. Hence, we have following applicability rules: 2 Rx UE, FDD only – HomoNet 2 Rx FDD async test and HetNet 2 Rx FDD sync test 2 Rx UE, TDD only – HomoNet 2 Rx TDD sync test and HetNet 2 Rx TDD sync test 2 Rx UE, FDD and TDD – HomoNet 2 Rx FDD async test and HetNet 2 Rx sync TDD test 4 Rx UE, FDD only – HomoNet 4 Rx FDD async test and HetNet 4 Rx FDD sync test 4 Rx UE, TDD only – HomoNet 4 Rx TDD sync test and HetNet 4 Rx TDD sync test 4 Rx UE, FDD and TDD – HomoNet 4 Rx FDD async test and HetNet 4 Rx TDD sync test |
| China Telecom | We support option 2B. |
| Intel | Option 2B looks fine for us |
| Ericsson | Option 2B |
| MediaTek | As Huawei mentioned, the IRC processing is agnostic to time offset. If we can verify MMSE-IRC processing through the synchronized scenario. We think we can save our efforts to define simulation assumptions and run lots of simulations. Hence, we support Option 1. |
| Qualcomm | Same comment as MediaTek. Support Option 1. |
| Apple | Support option 1. Same comment as MediaTek and Qualcomm |
| CMCC | Option 2B is preferred |
| Nokia | Results provided by Huawei seem un-intuitive to us, hard to follow how IRC performance could be completely agnostic to large TOs. Furthermore Intel has also provided results, which shows significant impact using the same random precoder per slot setup. We hope more results on this issue will be made available for next meeting to answer the question decisively if there is an impact or not. |
| Intel2 | We just realized that in case Option 2B will be considered, HomoNet FDD sync test will not be tested. Based on this, does it mean that, in case Option 2B will be considered, we need to define the following set of requirements? - FDD HomoNet Async - FDD HetNet Sync - TDD HomoNet Sync - TDD HetNet Sync It is fine for us in case we define testable settings for HomoNet Async. Based on our analysis for this meeting, we see the issue with feasibility of MMSE-IRC testing for both considered INR options for HomoNet in case per-slot precoder change is considered. Therefore, HetNet INR values are more preferable for us for testing for async case. |
| Huawei | @Nokia: In our results LTE INR values in async cases were reused and these INRs are too small, therefore it is reasonable no performance difference is observed. We may reconsider the INR values in case async scenario will be introduced. |
| China Telecom 2 | To Intel: We understand that with option 2B, for UEs supporting both FDD and TDD, sync FDD for HomNet will not be tested. Our thinking was that: 1) For each test scenario, we expect that if UE pass test under async, IRC processing under sync scenario can be ensured. 2) Moreover, sync scenario will always be tested under TDD. In that case, we agree we need to define requirements for FDD HomNet sync and async. We observed performance degradation from sync to async under HomNet scenarios in our simulation. Since we have agreed to have HomoNet for FDD and HetNet for TDD in the last meeting. Option 2B is the simplest way. |
| Intel3 | To China Telecom: Based on our understanding, FDD HomoNet Sync will not be used for testing of UE with FDD only support also. In first comment, Huawei have provided the list of the tested scenarios for Option 2B and there is no FDD HomoNet Sync case. Therefore, we assume that definition of requirements for FDD HomoNet Sync will be not required in case Option 2B is used. |
| China Telecom 3 | Thanks Intel for the comment. As we further checked, there is truly no FDD HomNet sync tests if we go with option 2B. We agree with you that in that case, no FDD HomNet sync requirement needs to be defined. |
| Docomo | We support Option 1. |

Issue 1-1-2: SSB configuration

- Proposals

  - Option 1: Use SSB Option 1 (All SSBs are in the same time/frequency resources) for all test

  - Option 2: Use SSB Option 2 (Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources) for all test

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

    - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

- Way forward

  - Check whether companies can compromise and Option 3A can be used

| Company | Comments |
| --- | --- |
| Huawei | We support option 2 1. Based on the summary of simulation results from companies, our concern is that: in some cases, the target SNR is lower than INR (For example, case with TDLA,4RX, INRs value equals to 11.39dB and 5.45dB ), how to always guarantee UE access the serving cell. i.e. SNR with lower value rather than interference cell. i.e. INR with higher value if SSB resources from two cells are colliding ? 2. From our simulation results, the SNR @ 1 % BLER of PBCH is higher than SNR @ 70% of max TP pf PDSCH for HetNet scenario. 3. PBCH is also used for time/frequency tracking. SSB colliding will cause poor accuracy of time/frequency tracking, which will degrade the performance of PDSCH. Based on our initial evaluations, we observed up to 1.8dB for 64QAM and 2.1dB for 256QAM performance gain with SSB-IM under the low network load compared to without IM receiver, it means the interference from the SSB of neighbour cells in the real network due to the colliding SSB configuration has serious impact on PDSCH performance, specific study needs to be conducted in Rel-18 to improve UE performance under network with colliding SSB configuration. Also in order to avoid the case that UE fails the test due to the poor performance of PBCH or poor time/frequency tracking accuracy rather than improper interference handling, it is better to configure no overlapping SSB resources for serving cell and interference cells. |
| China Telecom | We support option 1 and we can accept option 3A as compromise. |
| Ericsson | We support option 1. To Huawei, - In TS38.133, it clearly defined the cell identification requirement based on side condition SNR = -6dB which means UE can access the network even with serving cell power is lower than total interference power by 6dB. - In RAN1, the PBCH performance design is based on multiple trials to guarantee the system robust. MIB information is a semi-static information. No any requirement mandates UE to decode PBCH by one shot. Any requirement based on one shot simulation is a system over design. At the same time, in RAN4 RRM session, when RAN4 defines requirement for CGI reading, the requirement for MIB decoding is also 6 shots. - This issue is related to network deployment other than a UE receiver design. Compared different receiver for the performance gain is a wrong direction on this discussion. - To solve the concern for companies, the test case can be defined with a higher SNR for serving cell. Again, we want to further emphasis that SSB overlapping is a real network deployment. The defined requirement should base on the real network deployment, otherwise, the test case is meaningless. |
| MediaTek | We can support Option 3A to move forward. |
| Apple | We support option 2, but are fine with option 3A as compromise to move forward. |
| CMCC | We support option3A, fine with option 1 as compromise. |
| Nokia | We support option 1, but can compromise to Option 3A |
| Docomo | We support Option 1, but Option 3A is also acceptable for us as a compromise solution. |

Issue 1-3-1: INR values for Homogeneous deployment assumptions

- Proposals

  - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

  - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

  - Option 3: Option 2 for 2 cell, Option 1 for 1 cell.

- Way forward

  - Check whether Option 1 can be agreed based on majority support

| Company | Comments |
| --- | --- |
| Huawei | Option 1 |
| China Telecom | Fine with the recommended WF. |
| Ericsson | Option 1 |
| MediaTek | OK with Option1. |
| Qualcomm | Still prefer Option 2 or Option 3. Option 2 still has IRC gain > 1dB based on simulation results and RAN4 is defining minimum requirements. |
| Apple | We are fine with option 1. |
| CMCC | Option 1 is fine for us. |
| Nokia | We support option 1. |
| Docomo | We support Option 1. |
| Qualcomm2 | For Issue 1-3-1, we can compromise to Option 1. |

Issue 1-3-3: Number of explicitly modeled interference cells

- Proposals

  - Option 1: 1interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference cells for heterogeneous deployment assumptions

- Way forward

  - Check whether Option 3A can be agreed based on majority support

| Company | Comments |
| --- | --- |
| Huawei | Option 2. We concern that interference for 1 cell is too low and gain is not enough to verify MMSE-IRC processing |
| China Telecom | Support option 3A. |
| Ericsson | Option 3A |
| MediaTek | According the collected simulation results for heterogeneous deployment, the difference of average SNR between IRC and MRC is 2dB and 2.5dB for 2Rx and 4Rx respectively. We think it can be used to differentiate the behavior of receiver. We can support Option 3A to move forward. |
| Qualcomm | Still prefer Option 1 since IRC gains are > 1dB for 1 interfering cell. Also, 1 interfering cell will have more structured interference than 2 cell. |
| Apple | We still prefer option 1. We don’t think we see low gains with 1 cell compared to 2 cells. In fact we see better performance with 1 cell and more interference rejection since interference is more structured as Qualcomm also commented. |
| CMCC | Option 3A. |
| Nokia | We can agree to Option 3A |
| Docomo | We are fine with Option 3A. |

# Topic #2: MMSE-IRC receiver for inter-cell interference – CSI reporting requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200271 | Apple | Observation #1: With 2 RX the TP gain with 1 TX is higher than 2 TX. With 4 RX the TP gain with 2 TX is higher than that with 1 TX. Proposal #1: Further decide between 1TX and 2TX configuration for target cell based on results and observed TP gain. Proposal #2: Define requirements with BLER metric (X) with follow CQI in ICI as 2% Proposal #3: Define SINR and TP Gain (γ) based on simulation results from all companies. |
| R4-2200378 | MediaTek inc. | Proposal 1: Consider SINR = -2dB, γ =2 and X% = 2% to define CQI requirements. Proposal 2: Consider 1T2R and 1T4R to define CQI requirements. |
| R4-2200501 | Nokia, Nokia Shanghai Bell | Observation 1: Both antenna configurations have SINR regions with γ higher than 1.5. SINR values with option 2 are lower and arguably closer to cell edge conditions. Furthermore, option 2 is similar to what was used in LTE. Proposal 1: Prefer option 2 to define the requirements. Observation 2: SINR of ~0 dB has a ratio γ of ~1.5 for UEs with 2 Rx antenna configurations. Proposal 2: SINR of -2 dB and ratio γ of 1.5 can be used for 2 Rx antenna configuration. |
| R4-2200514 | Intel Corporation | Proposal 1: Consider the following setup for CQI MMSE-IRC requirements in scenario with inter-cell interference: - 2 Tx antenna for serving cell - Requirements - 2 Rx: SINR – 0 or 2 dB, T-put gain lower bound – 2 or 1.6, BLER lower bound – 0.02 - 4 Rx: SINR – 2 or 4 dB, T-put gain lower bound – 2.2 or 2, BLER lower bound – 0.02 |
| R4-2200799 | CMCC | Proposal 1: 2Tx shouldn’t be precluded due to the agreement about NZP CSI-RS relation between target and interference cells. Proposal 2: If modification for NZP CSI-RS agreement is needed, we propose to revisit as follows: - If the Tx antenna of serving cell is 2Tx - One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference. - If the Tx antenna of serving cell is 1Tx - NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference. |
| R4-2200901 | China Telecom | Proposal 1: Reuse the same Tx number we used for all the existing CQI tests in NR FR1, i.e., 2T2R and 2T4R. Proposal 2: Configure the target NZP CSI-RS with 2 ports to observe neighbor NZP CSI-RS interference on one RE and to observe interference data on the other RE. At the same time, PDSCH should be scheduled on the slots containing NZP CSI-RS on the neighbor cell. |
| R4-2200987 | Huawei, HiSilicon | Proposal 1: Use 2T2R and 2T4R for IRC CQI test Proposal 1: Assume that UE use IRC for demodulation and set the test metric that UE with different schemes for CQI calculation can be distinguished. Proposal 2: Use following test metric for CQI testing: - SINR=-2dB, γ is 2 for 2RX and 3 for 4RX, X=0.02 |
| R4-2201217 | Ericsson | Observation 1: The PDSCH Tput performance is the same for different Tx configuration of serving cell. Proposal 1: RAN4 to define CQI reporting test case based on 2T2R and 2T4R configurations. Proposal 2: RAN4 to define CQI reporting test metric with SINR=-2dB. Proposal 3: RAN4 to define CQI reporting test metric with γ=2. Proposal 4: RAN4 to introduce the BLER metric such as BLER > X% for CQI reporting test. |
| R4-2201218 | Ericsson | Simulation results on CSI reporting for inter-cell interference |
| R4-2201221 | Ericsson | Draft CR on CSI reporting test case (TDD) |
| R4-2201975 | Qualcomm Incorporated | Proposal 1: Define CQI reporting tests for only 1Tx or only 2Tx based on simulation alignment. Proposal 2: Use following parameters for defining FDD CQI reporting tests. - SINR = -1dB for 2Rx, -4dB for 4Rx - Throughput Ratio = 1.5 for 2Rx, 3.0 for 4Rx - BLER Threshold X = 1% |

Moderator’s note: Based on guidelines for this meeting, all Draft CR related discussion will be postponed to the next RAN4 meeting (i.e. R4-2201221 will be postponed)

## Open issues summary

### Sub-topic 2-1: Simulation assumptions

Issue 2-1-1: Antenna configuration

- Background

  - Option 1: 2T2R, 2T4R, ULA Low

  - Option 2: 1x2/1x4 ULA Low

- Proposals

  - Option 1 (Intel, China Telecom, Huawei, Ericsson, Qualcomm): 2x2, 2x4, ULA Low

  - Option 2 (MediaTek, Nokia, Qualcomm): 1x2, 1x4, ULA Low

  - Option 3 (Apple): Further decide between 1TX and 2TX configuration for target cell based on results and observed TP gain.

- Recommended WF

  - Check views on options above

  - Check whether Option 1 (2x2, 2x4, ULA Low) can be used

Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx

- Proposals

  - Option 1 (CMCC, China Telecom): One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference.

- Recommended WF

  - Check views on option above

### Sub-topic 2-2: Requirements setup

Issue 2-2-1: SINR for 2 Rx

- Proposals

  - Option 1 (MediaTek, Nokia, Huawei, Ericsson): -2 dB

  - Option 2 (Intel): 0 or 2 dB

  - Option 3 (Qualcomm): -1 dB

- Recommended WF

  - Check views on option above

Issue 2-2-2: T-put gain for 2 Rx

- Proposals

  - Option 1 (MediaTek, Huawei, Ericsson): 2

  - Option 2 (Nokia, Qualcomm): 1.5

  - Option 3 (Intel): 2 or 1.6

- Recommended WF

  - Decide based on simulation results after agreement on SINR point

Issue 2-2-3: BLER for 2 Rx

- Proposals

  - Option 1 (MediaTek, Intel, Huawei): 0.02

  - Option 2 (Qualcomm): 0.01

- Recommended WF

  - Decide based on simulation results after agreement on SINR point

Issue 2-2-4: SINR for 4 Rx

- Proposals

  - Option 1 (MediaTek, Huawei, Ericsson): -2 dB

  - Option 2 (Intel): 2 or 4 dB

  - Option 3 (Qualcomm): -4 dB

- Recommended WF

  - Check views on option above

Issue 2-2-5: T-put gain for 4 Rx

- Proposals

  - Option 1 (MediaTek, Ericsson): 2

  - Option 2 (Intel): 2.2 or 2

  - Option 3 (Huawei, Qualcomm): 3

- Recommended WF

  - Decide based on simulation results after agreement on SINR point

Issue 2-2-6: BLER for 4 Rx

- Proposals

  - Option 1 (MediaTek, Intel, Huawei): 0.02

  - Option 2 (Qualcomm): 0.01

- Recommended WF

  - Decide based on simulation results after agreement on SINR point

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 2-1: Simulation assumptions

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-1-1: Antenna configuration Option 1, since 2Tx is more practical in real world and IRC is only sensitive to number of RXs |
| Intel | Issue 2-1-1: Antenna configuration Support Option 1. This is the typical configuration used for existing CQI requirements. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx Support Option 1 |
| CMCC | Issue 2-1-1: Antenna configuration Option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx If 2Tx is adopted, then we prefer Option 1, the further clarification can avoid ambiguation. |
| MediaTek | Issue 2-1-1: Antenna configuration We just wonder why to define different Tx number for serving and interference cell. However, according to the simulation results, there are also throughput gains for the case of 2Tx. Hence, we are OK to define 2Tx for serving cell following the existing CQI requirements. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx If 2Tx for the serving cell is agreed, we are OK with Option 1. |
| China Telecom | Issue 2-1-1: Antenna configuration Support option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx Support Option 1 |
| Ericsson | Issue 2-1-1: Antenna configuration Option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 2-1-1: Antenna configuration Can also support option 1 as it also has SINR range with acceptable throughput gain. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx We see option 1 as necessary if 2TX is chosen for serving cell. |
| Apple | Issue 2-1-1: Antenna configuration We are fine with option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx We support option 1. |
| Qualcomm | Issue 2-1-1: Antenna configuration We have no strong preference. Ok with Option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx Ok with Option 1. |
| Docomo | Issue 2-1-1: Antenna configuration We support Option 1. Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx We support Option 1. |

#### Sub-topic 2-2: Requirements setup

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-2-1: SINR for 2 Rx Option 1. From our simulation results, the TP ratio difference for CQI reporting by using IRC and MRC is big at SINR=-2dB Issue 2-2-2: T-put gain for 2 Rx According to our simulation results, option 3 is more feasible Issue 2-2-3: BLER for 2 Rx Option 1 Issue 2-2-1: SINR for 4 Rx Option 1. From our simulation results, the TP ratio difference for CQI reporting by using IRC and MRC is big at SINR=-2dB Issue 2-2-2: T-put gain for 4 Rx According to our simulation results, option 3 is more feasible Issue 2-2-3: BLER for 4 Rx Option 1 |
| Intel | Issue 2-2-1: SINR for 2 Rx and Issue 2-2-1: SINR for 4 Rx We suggest to consider different SINR values for 2 Rx and 4 Rx requirements (i.e. similar to existing CQI requirements). Proposal from QC is fine for us. For other parameters, we can decide based on simulation results for agreed SINR values. |
| China Telecom | Issue 2-2-1: SINR for 2 Rx We support option 1 which is same with LTE requirement definition. Since we are using INR approach for the interference modeling, we wonder whether SINR should be converted to SNR value for the requirement definition. Issue 2-2-3: BLER for 2 Rx Option 1. Issue 2-2-1: SINR for 4 Rx Option 1. Same with issue 2-2-1. Issue 2-2-3: BLER for 4 Rx Option 1 |
| Ericsson | Issue 2-2-1: SINR for 2 Rx Option 1. Issue 2-2-2: T-put gain for 2 Rx Option 1. Issue 2-2-3: BLER for 2 Rx Option 1. Issue 2-2-1: SINR for 4 Rx Option 1. Issue 2-2-2: T-put gain for 4 Rx Option 1. Issue 2-2-3: BLER for 4 Rx Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 2-2-1: SINR for 2 Rx Option 1. Issue 2-2-2: T-put gain for 2 Rx Option 2. |
| Apple | Issue 2-2-1: SINR for 2 Rx We support option 2. But we could also discuss based on sim results. We didn’t evaluate for < 0dB SINR. Hence, we suggest discussing a SINR range for companies to bring results in next meeting. SINR: -2 to 10 dB for 2RX; -4 to 8 dB for 4RX Issue 2-2-2: T-put gain for 2 Rx Fine with recommended WF. Issue 2-2-3: BLER for 2 Rx Fine with recommended WF. Issue 2-2-1: SINR for 4 Rx Same comments as Issue 2-2-1. Issue 2-2-2: T-put gain for 4 Rx Fine with recommended WF. Issue 2-2-3: BLER for 4 Rx Fine with recommended WF. |
| Qualcomm | Issue 2-2-1: SINR for 2 Rx and Issue 2-2-1: SINR for 4 Rx Based on the simulation results provided from different companies, each company provided results for different set of SINRs and most companies provided results only for FDD. Therefore, we suggest to collect simulation results from all companies for the same range of SINRs (for example, -6 to 0 dB) for both FDD and TDD in the next meeting. Based on the alignment, we can decide the SINR point. We would prefer to use SINR <= 0dB because serving cell will be in general stronger than interfering cell. For reference, LTE requirements were also defined for SINR = -2dB For other parameters, we can decide based on simulation results. |
| Docomo | Issue 2-2-2: T-put gain for 2 Rx We support the recommended WF. Issue 2-2-3: BLER for 2 Rx We support the recommended WF. Issue 2-2-2: T-put gain for 4 Rx We support the recommended WF. Issue 2-2-3: BLER for 4 Rx We support the recommended WF. |

## Summary for 1st round

### Open issues

#### Sub-topic 2-1: Simulation assumptions

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Antenna configuration | Tentative agreements: 2x2, 2x4, ULA Low |
| Issue 2-1-2: NZP CSI-RS configuration for scenario in case of 2 Tx | Tentative agreements: One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference. |

#### Sub-topic 2-2: Requirements setup

|  | Status summary |
| --- | --- |
| Issue 2-2-1: SINR for 2 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, China Telecom, Ericsson, Nokia): -2 dB - Option 2 (Apple): 0 or 2 dB - Option 3 (Intel): -1 dB - Option 4 (Apple, Qualcomm): FFS and use same SINR range for simulation results in the next meeting. - Apple: SINR range -2 to 10 dB - Qualcomm: SINR range -6 to 0 dB Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results - Check whether the following can be captured in the WF: - Companies are encouraged to bring FDD/TDD and 2Rx/4Rx results for the SINR range [-6 to 10] dB in the next RAN4 meeting |
| Issue 2-2-2: T-put gain for 2 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Ericsson): 2 - Option 2 (Nokia): 1.5 - Option 3 (Huawei): 2 or 1.6 - Option 4 (Intel, Apple, Qualcomm, Docomo): FFS based on results Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results |
| Issue 2-2-3: BLER for 2 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, China Telecom, Ericsson): 0.02 - Option 2: 0.01 - Option 3 (Intel, Apple, Qualcomm, Docomo): FFS based on results Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results |
| Issue 2-2-4: SINR for 4 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, China Telecom, Ericsson): -2 dB - Option 2: 2 or 4 dB - Option 3 (Intel): -4 dB - Option 4 (Apple, Qualcomm): FFS and use same SINR range for simulation results in the next meeting. - Apple: SINR range -4 to 8 dB - Qualcomm: SINR range -6 to 0 dB Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results |
| Issue 2-2-5: T-put gain for 4 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Ericsson): 2 - Option 2: 2.2 or 2 - Option 3 (Huawei): 3 - Option 4 (Intel, Apple, Qualcomm, Docomo): FFS based on results Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results |
| Issue 2-2-6: BLER for 4 Rx | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, China Telecom, Ericsson): 0.02 - Option 2: 0.01 - Option 3 (Intel, Apple, Qualcomm, Docomo): FFS based on results Recommendations for 2nd round: - Stop discussion this meeting. Continue in the next meeting based on simulation results |
| Issue 2-2-7 (New): Test setup methodology for signal power | Tentative agreements: N/A Candidate options: - Option 1 (China Telecom): Since we are using INR approach for the interference modeling, we wonder whether SINR should be converted to SNR value for the requirement definition. Recommendations for 2nd round: - Check Option 1 in the second round |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2203009 | Intel: we also suggest to add the SINR step (1 or 2 dB) for results collection for the next meeting. From our side, we slightly prefer to consider 1 dB SINR step (i.e. SINR range [-6:1:10] dB). Such granularity will allow to check whether SINR point with 3 dB difference can be selected for 2 Rx and 4 Rx test (i.e. similar to existing CQI requirements). |
|  | Apple: In the WF do we need to specify the options for SINR and TP Gain? If we do, we should also add other options not precluded since we will decide based on simulation results. We prefer 1dB SINR step for results alignment as well. |

### Open issues

Issue 2-2-1: Test setup methodology for signal power

- Option 1: Define test based on SINR

- Option 2: Define test based on SNR

| Company | Comments |
| --- | --- |
| Nokia | We support Option 1 to reuse same methodology as in LTE. |
| Intel | At current stage, we support Option 2 to align with Demod test setup methodology. Same time, we need some time to check what will be the best option from requirements description in the specification. |
| Qualcomm | We slightly favor Option 2 to align with demod tests. But we are ok to decide in the next meeting. |
| MediaTek | We are fine with Option 2. |
| Huawei | Prefer option 1. Considering we have two cases that case 1 has interference and noise and case 2 has only noise with the condition that power of interference plus noise of case 1 equals to noise of case 2. SNR methodology will make a confusion that only power of noise is same. |
| Ericsson | Prefer option 1 to follow the LTE methodology, but no strong view. |
| Apple | We prefer option 2 to align with demod, but option 1 might be better to capture in requirements definition. We can decide in next meeting. |

# Topic #3: MMSE-IRC receiver for intra-cell inter-user interference

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200272 | Apple | Observation #1: With random precoder for co-scheduled UE, the performance degradation is 1.7 to 2 dB for 2TX and 3.8 dB for 4TX. Proposal #1: Define requirements for MU-MIMO with orthogonal precoder for co-scheduled UEs. Observation #2: For 2TX with 1 layer per co-scheduled UE there is no significant performance delta between various DMRS port mapping between co-scheduled UEs. Proposal #2: For 2TX configure co-scheduled UEs on same CDM group. Proposal #3: For requirements with MU-MIMO configure same DMRS scrambling ID for target and co-scheduled UEs. |
| R4-2200379 | MediaTek inc. | Proposal 1: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. Proposal 2: For the test cases with rank 1+1, configure DMRS port 0 for target UE and DMRS port 1 for the interference UE, i.e., same CDM group. Proposal 3: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. |
| R4-2200380 | MediaTek inc. | TP to TR 38.833: Scenario for inter-user interference suppression for MU-MIMO |
| R4-2200515 | Intel Corporation | Observation #1: For PMI matrix selection for co-scheduled UE Option 1 (Select the precoder to ensure orthogonality) provides better performance in comparison to Option 2 (Random) for all considered scenarios and different receiver assumptions. Observation #2: For DMRS ports mapping for scenario with Rank 1+1 configuration Option 2 (different CDM groups mapping) and Option 3 (variable CDM groups mapping) allows to verify correct interfere-plus-noise covariance matrix estimation for 2 Rx UE for scenarios with different CDM groups for target and interference UEs. Observation #3: Test setup for Option 3 (variable CDM groups mapping) is more complicated in comparison to Option 2 (different CDM groups mapping) for DMRS ports mapping for scenario with Rank 1+1 configuration. Proposal 1: Consider the following assumptions for MU-MIMO modelling for requirements definition: - Precoder selection for interference UE: Option 1 (Select the precoder to ensure orthogonality) - DMRS ports mapping DMRS ports for case with rank 1+1: - First priority: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Second priority: Variable DMRS port mapping - Same DMRS scrambling ID for target UE and co-scheduled UE |
| R4-2200516 | Intel Corporation | TP to TR 38.833: Link level simulation results for Inter-user interference suppression for MU-MIMO |
| R4-2200800 | CMCC | Proposal 1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE |
| R4-2200807 | CMCC | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO |
| R4-2200903 | China Telecom | Observation 1: With orthogonal PMI selection for the co-scheduled UE, UE can achieve 70% max throughput with lower SNR point than that of random PMI selection. Observation 2: Both random and orthogonal PMI selection can achieve reasonable performance gain between MMSE-IRC and the baseline MMSE. Random PMI selection can achieve higher MMSE-IRC performance gain over the baseline MMSE receiver. Proposal 1: Fine to use either random or orthogonal PMI selection for the co-scheduled UE. Use only one PMI selection method for the co-scheduled UE for all rank configurations. Observation 3: For the variable DMRS port mapping, since we are using the FRC in the real test, we have to always configure the DMRS group number as 2. Observation 4: Such configuration may impact the Rnn accuracy: UE may always perform Rnn estimation with all the DMRS REs, since the DMRS group number is configured 2. Proposal 2: Need feedback on whether it is practical to use variable DMRS port config with DMRS CDM group number is always 2. If so, companies need to do more investigation on whether UE Rnn estimation process and IRC performance will be impacted. Proposal 3: Use same CDM group to test the Rnn estimation accuracy. Proposal 4: Fine with either same or different scrambling ID when paired UEs are in different CDM groups |
| R4-2200904 | China Telecom | TP to TR 38.833: Updated conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario |
| R4-2200989 | Huawei, HiSilicon | TP to TR 38.833: Introduction of simulation assumptions of intra cell inter user MMSE-IRC receiver |
| R4-2200990 | Huawei, HiSilicon | Observation 1: Selecting orthogonal PMI matrix for paired UEs during the test is not near to the real network behaviour without emulating the network’s real precoding process. Observation 2: The performance difference for different PMI selection is only about 1dB for case with rank 1+1, the performance is not bottleneck for random PMI. Observation 3: the performance gain for MMSE-IRC over MMSE-MRC is higher for case with random PMI selection compared to that with orthogonal PMI selection and hence the processing of MMSE-IRC will be better verified. Observation 4: TE should perform PMI orthogonalization processing 26 times per TTI based on the assumption of 52RBs with 10MHz bandwidth and 15 kHz SCS which may cause high complexity and long test time. Observation 5: Scrambling ID configuration has negligible effect on performance. Proposal: Use following parameters for intra cell inter user MMSE-IRC requirements definition: - Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE - Same CDM group for both UEs for rank 1+1 - Same scrambling ID for all cases |
| R4-2200991 | Huawei, HiSilicon | Observation 1: Performance for different CDM group is slightly better than same CDM group. Observation 2: There is only about 1 dB performance lost for random PMI selection compared to that for orthogonal PMI selection. Observation 3: The performance gain for MMSE-IRC over MMSE-MRC is lager for case with random PMI selection compared to that for orthogonal PMI selection. Observation 4: There is only about 3.6 dB performance lost for random PMI selection compared to that for orthogonal PMI selection. Observation 5: The performance gain for MMSE-IRC over MMSE-MRC is lager for case with random PMI selection compared to that for orthogonal PMI selection. |
| R4-2200992 | Huawei, HiSilicon | Draft CR: Introduction of MU-MIMO Beamforming model in TS 38.101-4 |
| R4-2200993 | Huawei, HiSilicon | Summary of simulation results for intra cell inter user MMSE receiver requirements |
| R4-2201219 | Ericsson | Observation 1: No performance difference for configuring the interference UE between the same CDM group and different CDM groups for rank(1,1) for 70% maximum Tput point. Observation 2: No typical performance difference between same and different scrambling ID when paired UEs are in the same CDM group for 70% maximum Tput point. Proposal 1: RAN4 to define the MMSE-IRC performance based on random PMI selection for both target and interference UE in intra-cell inter-users to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE. Proposal 2: RAN4 to define the MU-MIMO rank 1 test case with variable DMRS port mapping. - The number of CDM groups shall be 2 for same CDM group - Example: the modulation symbols of the signal under test are mapped to port 0, and the modulation symbols of the signal for interfering UE are mapped randomly onto antenna port 1, 2, or 3. Proposal 3: RAN4 to define the MU-MIMO rank 1 test case with variable scrambling ID configurations. |
| R4-2201220 | Ericsson | Observation 1: Compared with MRC, IRC receiver can have obvious gain in 70% of maximum throughput SNR point |
| R4-2201966 | Qualcomm Incorporated | Proposal 1: Use random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality. Proposal 2: Use same DMRS scrambling ID for all co-scheduled UEs. |

Moderator’s note: Based on guidelines for this meeting, all Draft CR related discussion will be postponed to the next RAN4 meeting (i.e. R4-2200992 will be postponed)

## Open issues summary

### Sub-topic 3-1: Inter-user interference modeling

Issue 3-1-1: Precoder selection for interference UE

- Background

  - Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

  - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE

  - Option 3: Use option 2 for rank 1+1 and option 1 for rank 2+2.

- Proposals

  - Option 1 (Apple, MediaTek, Intel, China Telecom, Qualcomm)

  - Option 2 (CMCC, China Telecom, Huawei, Ericsson)

- Recommended WF

  - Check whether Option 3 (option 2 for rank 1+1 and option 1 for rank 2+2) can be considered as compromise solution to cover both PMI selection options or whether there is any other compromise solution

Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration

- Background

  - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

  - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

  - Option 3: Variable DMRS port mapping during the test.

- Proposals

  - Option 1 (Apple, MediaTek, China Telecom, Huawei)

  - Option 2 (Intel – First priority)

  - Option 3 (Intel – Second priority, Ericsson)

    - China Telecom: Need feedback on whether it is practical to use variable DMRS port config with DMRS CDM group number is always 2. If so, companies need to do more investigation on whether UE Rnn estimation process and IRC performance will be impacted.

    - Ericsson: The number of CDM groups shall be 2 for same CDM group

      - Example: the modulation symbols of the signal under test are mapped to port 0, and the modulation symbols of the signal for interfering UE are mapped randomly onto antenna port 1, 2, or 3.

- Recommended WF

  - Check whether Option 1 can be considered based on majority companies views

Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE

- Background

  - Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

  - Option 2: Same scrambling ID for all cases

  - Option 3: Configure variable scrambling ID during the test.

- Proposals

  - Option 1 (MediaTek, China Telecom)

  - Option 2 (Apple, Intel, China Telecom, Huawei, Qualcomm)

  - Option 3 (Ericsson)

- Recommended WF

  - Check whether Option 2 can be considered based on majority companies views

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 3-1: Inter-user interference modeling

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-1-1: Precoder selection for interference UE We support option 2 and we have following reasons. - Based on our understanding, network always use orthogonization technique based on channel reciprocity by using SRS based channel estimation. In this test, PMI selection is random without any prior information so network’s real precoding process is not simulated at all even we use orthogonal PMI matrix for paired UEs. There are still large interference between the paired Ues at the receiving side even orthogonal precoding is simply used without channel detection .Therefore, we think it is more typical to use random PMI selection for co-scheduled UE. - From our simulation results, the performance difference for different PMI selection is only about 1dB for case with rank 1+1, we don’t think the performance is the bottleneck for random PMI. - What’s more, random PMI selection will cause larger lager interference compared to orthogonal PMI selection and according to our simulation results, the performance gain for MMSE-IRC over MMSE-MRC is higher for case with random PMI selection compared to that with orthogonal PMI selection and hence the processing of MMSE-IRC will be better verified. - From the perspective of complexity, PMI orthogonalization processing should be performed per PRB bundling size which means TE should perform PMI orthogonalization processing 26 times per TTI based on the assumption of 52RBs with 10MHz bandwidth and 15kHz SCS. This may cause high complexity and long test time. If some companies have strong views to introduce random PMI selection, we can comprise to option 3. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Option 1 Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE Option 2 |
| Intel | Issue 3-1-1: Precoder selection for interference UE We support Option 1 as first priority, because we think that typical network will try to reduce the inter-UE interference in the practical MU-MIMO conditions to achieve good DL performance. For this purpose, RAN1 designed Type II codebooks with multiple enhancements. However, we consider Type I codebooks. Therefore, we can not ideally mitigate the inter-user interference using these codebooks and using of orthogonal Type I precoders is not the ideal scenario for MU case. As compromise, Option 3 is fine for us. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration To move forward, we are fine to consider Option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE Support Option 2 |
| CMCC | Issue 3-1-1: Precoder selection for interference UE Option 2 is our first choice. We can compromise to Option 3 to move forward. |
| MediaTek | Issue 3-1-1: Precoder selection for interference UE We think both Option 1 and Option 2 cannot reflect the realistic precoder selection. However, NW will try to reduce interference between co-scheduled UE for downlink throughput. We support Option 1 and can compromise to Option 3. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Support Option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE It is possible for gNB to configure different scrambling ID to co-scheduled UE. Hence, we support Option 1 to consider both same and different scrambling ID in the test cases. We can also compromise to Option 3 to move forward. |
| China Telecom | Issue 3-1-1: Precoder selection for interference UE Either option 1 or option 2 is fine for us. Since precoding method does not impact IRC processing, we expect use only one option for all tests. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We support Option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE The recommended WF is fine for us. |
| Ericsson | Issue 3-1-1: Precoder selection for interference UE Option 2. As we explained in several meetings, the network is unlikely only based on the target UE’s PMI feedback to decide the precoder. Meanwhile, from TE vendors’ feedback, option 2 is more feasible than option 1. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration To move forward, we can compromise to option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE We can compromise to option 1. Whether the co-scheduled UEs are configured with same scrambling ID or not is fully up to network’s configuration. To consider test coverage, we propose to use option 3. As a compromised solution, we’re fine with option 1. |
| Apple | Issue 3-1-1: Precoder selection for interference UE We support option 1 as first priority. We shouldn’t define requirements with unrealistic assumptions. When MU-MIMO is used the gNB would use the PMI from each user to determine a suitable precoder to minimize the inter-user interference, just not randomly choose precoders that are not identical. Based on TE vendor feedback we don’t think option 1 is not feasible or less feasible than option 2. With option 3 we need to have 2 different setups for 2x2 and 4x4. Would that be acceptable to all including TE vendors? Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We are fine with the recommended WF - option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE We are fine with the recommended WF - option 2. |
| Qualcomm | Issue 3-1-1: Precoder selection for interference UE Prefer Option 1 since NW will try its best to orthogonalize precoders. Option 2 is very pessimistic. Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Ok with option 1. Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE Ok with option 2. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200380 | Company A |
|  | Company B |
| R4-2200516 | Company A |
|  | Company B |
| R4-2200807 | Company A |
|  | Company B |
| R4-2200904 | Company A |
|  | Company B |
| R4-2200989 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

#### Sub-topic 3-1: Inter-user interference modeling

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Precoder selection for interference UE | Tentative agreements: N/A Candidate options: - Option 1 (Intel, MediaTek, China Telecom, Apple, Qualcomm): Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. - Option 2 (Huawei, CMCC, China Telecom, Ericsson): Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE - Option 3 (Huawei, Intel, CMCC, MediaTek): Use option 2 for rank 1+1 and option 1 for rank 2+2. Recommendations for 2nd round: - Continue discussion on all options. Check whether Option 3 can be used. |
| Issue 3-1-2: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration | Tentative agreements: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group |
| Issue 3-1-3: DMRS scrambling ID for target UE and co-scheduled UE | Tentative agreements: N/A Candidate options: - Option 1 (MediaTek, Ericsson): Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2 (Huawei, Intel, China Telecom, Apple, Qualcomm): Same scrambling ID for all cases - Option 3 (MediaTek): Configure variable scrambling ID during the test. Recommendations for 2nd round: - Continue discussion on options 1 and 2. |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2203011 | Intel: I think probably it is better to keep all options for “Issue 2: DMRS scrambling ID for target UE and co-scheduled UE”, because Option 1 was accepted by more companies in comparison to Option 3. |
|  | Moderator: Could you please update the feature list issue based on outcome of discussion in another e-mail thread? Sub-topic 4-2: UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15 - Option 2: Mandatory with or without UE capability signaling - Option 3: Optional with or without UE capability and applicable from Rel-17 |

### Open issues

Issue 1: PMI matrix selection for Co-scheduled UE for 2TX and 4TX

- Proposals:

  - Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

  - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE

  - Option 3: Use option 2 for rank 1+1 and option 1 for rank 2+2.

- Recommendations for 2nd round:

  - Continue discussion on all options. Check whether Option 3 can be used.

| Company | Comments |
| --- | --- |
| Huawei | We support option 2 as first priority and can compromise to option 3. We agree both option 1 and option 2 are not practical for scheduling of gNB but our purpose is to verify the UE’s capability on mitigate inter-user’s interference, so, from the test purpose point, we should try to select the configuration to maximum the gain of MMSR-IRC. Moreover, if we use random PMI selection in Rel-17 case, advanced receiver in Rel-18 such as E-MMSE-IRC/RML will achieve more gain in such configurations and we can simply reuse these test cases for performance comparison. Same time, we also think random PMI selection is more easily for TE’s implementation. |
| Intel | From our side, Option 1 is preferred. Same time, we are also fine to consider Option 3 to move forward and cover different precoder assumptions in the tests. As for actual testing, we understand that Option 2 is slightly easier in comparison to Option 1. But based TE feedback from one of the previous discussions, there is no any issue to consider Option 1. |
| CMCC | Share similar views with HW, Option 2 is our first choice. To move forward, Option 3 is ok for us. |
| Apple | We support option 1. We don’t understand what maximum gain from MMSE-IRC is observed with random precoder means when orthogonal precoder gives better performance. If its referring to gain over MMSE, its Inf in most cases. |
| Ericsson | We support option 2 as first priority and can compromise to option 3. |
| MediaTek | Share the same view as Intel that there is no issues for orthogonal precoder from the feedback of TE vender in the previous meeting. We support Option 1 but we can go for Option 3 to move forward. |
| China Telecom | We preferred to have one test setup for all tests since precoding method does not impact IRC performance. Considering the majority’s view, to move forward, we can also compromise to option 3. |
| Huawei | @Apple : For rank 1+1 with 4RX, the requirements for MMSE-MRC is not inf and random PMI selection will cause large interference which will bring more gain for IRC. |
| Qualcomm | We support Option 1. In our opinion, option 2 is very pessimistic and sets a bad precedent for gNB implementation. |
| Qualcomm2 | For Issue 1, we still think that it’s a bad precedent for gNB implementation to choose random precoder. However, to make progress, we can compromise to Option 2. |
| Apple2 | We are not in favor of having different options for different testcases, especially when it doesn't impact UE processing and doesn’t even provide additional test coverage in anyway. In order to move forward we can compromise to option 2. |

Issue 2: DMRS scrambling ID for target UE and co-scheduled UE

- Proposals:

  - Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

  - Option 2: Same scrambling ID for all cases

  - Option 3: Configure variable scrambling ID during the test.

- Recommendations for 2nd round:

  - Continue discussion on all options. Check whether Option 3 can be used.

| Company | Comments |
| --- | --- |
| Huawei | Option 2. We think option 2 is the simplest way for implementation and there is no performance impact |
| Intel | In first round we support Option 2, but we are also fine to consider Option 1, because from test design we don’t see the big difference whether to configure Scrambling ID to 0 for all UEs or 0 for UE 1 and 1 for UE 2 (for example). |
| Apple | We support option 2 to have same configuration in all tests. |
| Ericsson | We support option 3 as first priority and can compromise to option 1. Whether the co-scheduled UEs are configured with same scrambling ID or not is fully up to network’s configuration. To consider test coverage, either option 3 or option 1 is fine for us. |
| MediaTek | It is possible for gNB to configure different scrambling ID to co-scheduled UE. Hence, we support Option 1 to consider both same and different scrambling ID in the test cases. |
| China Telecom | We are fine with option 2. We do not agree option 3. In our understanding, DMRS scrambling ID should be not changed so frequently in practical at least for the target UE. |
| Qualcomm | We support Option 2. In our opinion, option 3 is very artificial and won’t happen in practice and it will unnecessarily complicate the test setup. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203007 (Revision of R4-2200516) | [Intel] For preparation of this TP, the following simulation results summary was used: R4-2200993 Summary of simulation results for intra cell inter user MMSE-IRC receiver requirements v02_Eric_Mod.xlsx In this meeting, most of the companies have provided the results for scenarios which were agreed for requirements definition. Therefore, we can update simulation results only for these scenarios. For all other results we suggest to remove [] and put “No results” for scenarios with TBA. For scenarios, which captured in the summary document, we can keep [] and probably remove [] in the next RAN4 meeting. For some scenarios we have rather big span, we have two options how to capture these results in TP in this meeting: - Option 1: Keep results with big span - Option 2: Capture results based on subset of results with acceptable span Option 1 currently is used, but we can update TP with Option 2 in case it is needed. |
|  | [Apple] Thanks for the comment explaining the updates. We updated results in the R4-2200993 Summary of simulation results for intra cell inter user MMSE-IRC receiver requirements v03_Mod_Apple.xlsx No strong preference for options above. Option 1 is fine to save time and effort. |
| R4-2200807 | China Telecom: Performance gain is also observed for Rank 2+2 with MCS 13 Orthogonal. Thus should be captured in the result summary. |
|  | CMCC: CTC's comment has been captured in our TP version 2. |
| R4-2200904 | Company A |
|  | Company B |
| R4-2200989 | [Intel] Based on review of R4-2200989 TP Introduction of simulation assumptions on intra cell inter user MMSE-IRC receiver.docx, it is not clear what is the suggested changes in comparison to simulation assumptions agreed in the previous meeting? Based on our understanding, simulation assumptions for Phase I evaluation was already correctly captured in the previous RAN4 meeting. |
|  | [Apple] We have same observation and comments as Intel. We prefer to remove options for scrambling ID as they were no results captured separately for these options. |
|  | [Huawei]: Sorry for the mistake, we uploaded a wrong version. Please check the lasted version: draft R4-2200989 TP Simulation assumptions of intra cell inter user MMSE-IRC receiver_v2.docx In this TP, we removed all the parameters related to TDD, since there were no simulation results for TDD in phase 1. |

# Topic #4: UE feature list and capability signaling

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201603 | Intel Corporation | Proposal 1: Define mandatory without capability signaling feature to indicate that UE supports MMSE-IRC requirements for scenarios with inter-cell interference and slot-based transmission in case requirements will be defined in a non-release independent manner. Proposal 2: Define mandatory without capability signaling feature to indicate that UE supports MMSE-IRC requirements for scenarios with intra-cell inter-user interference in case requirements will be defined in a non-release independent manner. |

## Open issues summary

### Sub-topic 4-1: UE feature list MMSE-IRC requirements for scenarios with inter-cell interference

Issue 4-1-1: UE feature list

- Proposals

  - Option 1 (Intel): Define mandatory without capability signaling feature to indicate that UE supports MMSE-IRC requirements for scenarios with inter-cell interference and slot-based transmission in case requirements will be defined in a non-release independent manner.

- Recommended WF

  - Check views on option above

### Sub-topic 4-2: UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

Issue 4-2-1: UE feature list

- Proposals

  - Option 1 (Intel): Define mandatory without capability signaling feature to indicate that UE supports MMSE-IRC requirements for scenarios with intra-cell inter-user interference in case requirements will be defined in a non-release independent manner.

- Recommended WF

  - Check views on option above

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 4-1: UE feature list MMSE-IRC requirements for scenarios with inter-cell interference

| Company | Comments |
| --- | --- |
| Huawei | Issue 4-1-1: UE feature list We prefer to follow LTE to define UE capability signaling |
| Intel | Issue 4-1-1: UE feature list In LTE, MMSE-IRC receiver is considered as Enhanced processing from Rel-11 and MMSE-MRC receiver was considered as baseline for LTE Rel-8. Same time, MMSE-IRC receiver is considered as baseline from Rel-15 NR. Therefore, we cannot follow LTE procedure. |
| CMCC | Issue 4-1-1: UE feature list Option 1 is fine for us. |
| China Telecom | Issue 4-1-1: UE feature list We support the MMSE-IRC for both ICI and MU-MIMO scenarios to be mandatory without capability signaling. Same time, we think that whether to define the MMSE-IRC capability, does not conflict with the test requirement to be release independent from Rel-15. For the MMSE-IRC for ICI scenario, since both CQI reporting and PDSCH demodulation test requirements are defined, no signaling is needed. For the MMSE-IRC for MU-MIMO scenario, consider network UEs not pass test for Rel-17, we are also fine to discuss the UE MMSE-IRC capability under MU-MIMO to be either mandatory with or without signaling. |
| Ericsson | Option 1 is fine is us. We have the same understanding as Intel, RAN4 assumed MMSE-IRC receiver was considered as the baseline receiver for Rel-15 NR UE demodulation requirements. So We support the MMSE-IRC for both ICI and MU-MIMO scenarios to be mandatory without capability signalling. |
| Apple | We are not very sure how defining this UE capability is even helpful since MMSE-IRC is baseline receiver since Rel-15. In LTE it was an advanced receiver. |
| Qualcomm | We prefer this to be release independent. We are not sure why this needs to be defined as a separate feature since MMSE-IRC receiver has always been the baseline since Rel-15. |
| Docomo | Issue 4-1-1: UE feature list We don’t understanding the reason why need to define this UE capability. In our understanding, MMSE-IRC receiver is considered as baseline from Rel-15 NR. |

\

#### Sub-topic 4-2: UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

| Company | Comments |
| --- | --- |
| Huawei | Issue 4-2-1: UE feature list We prefer to follow LTE to define UE capability signaling |
| Intel | Issue 4-2-1: UE feature list Same comment as for Issue 4-1-1 |
| CMCC | Issue 4-1-1: UE feature list Option 1 is fine for us. |
| China Telecom | Issue 4-2-1: UE feature list Commented in issue 4-1-1. |
| Ericsson | Issue 4-2-1: UE feature list Same comment as for Issue 4-1-1 |
| Apple | Issue 4-2-1: UE feature list Same comment as for Issue 4-1-1 |
| Qualcomm | Issue 4-2-1: UE feature list Same comment as for Issue 4-1-1 |
| Docomo | Issue 4-2-1: UE feature list Same comment as for Issue 4-1-1 |

## Summary for 1st round

### Open issues

#### Sub-topic 4-1: UE feature list MMSE-IRC requirements for scenarios with inter-cell interference

|  | Status summary |
| --- | --- |
| Issue 4-1-1: UE feature list | Tentative agreements: N/A Candidate options based on GTW: - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15 (majority supporting) - Option 2: [Mandatory/optional] with UE capability signaling with granularity per UE Recommendations for 2nd round: - Continue discussion on two options in the second round |

#### Sub-topic 4-2: UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

|  | Status summary |
| --- | --- |
| Issue 4-2-1: UE feature list | Tentative agreements: N/A Candidate options based on GTW: - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15 (majority supporting) - Option 2: [Mandatory/optional] with UE capability signaling with granularity per UE Recommendations for 2nd round: - Continue discussion on two options in the second round |

## Discussion on 2nd round

This issue will be discussed separately, and agreements will be captured in “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC” and “WF on MMSE-IRC receiver for intra-cell inter-user interference”.

### Open issues

Sub-topic 4-1: UE feature list MMSE-IRC requirements for scenarios with inter-cell interference

- Proposals for discussion

  - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

  - Option 2: [Mandatory/optional] with UE capability signaling with granularity per UE

  - Option 3: Optional without UE capability signalling and applicable from Rel-17

  - Option 4: Optional without UE capability signalling and applicable from Rel-15

- Recommended WF

  - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

  - Option 2: Optional without UE capability signalling and applicable from Rel-17

  - Option 3: Optional without UE capability signalling and applicable from Rel-15

| Company | Comment |
| --- | --- |
| Intel | 1) Based on 36.306 Section 6.6.1 and 6.6.2, MMSE-IRC is optional without capability signalling LTE feature 2) Based on WF R4-1809392 we have the following agreement for NR for all considered scenarios: MMSE-IRC/MMSE as baseline Based on this we think that UE capability signalling should not be considered. As for optional or mandatory, taking into account that in LTE this feature is optional from Rel-11 and that we agreed to assume MMSE-IRC as one of baseline receivers for NR from Rel-15, we assume that it should be mandatory. Therefore, we assume that Rel-15 NR UE can pass the requirements for scenario with inter-cell interference and Option 1 can be considered. |
| Huawei | We support option 2 and define optional with capability signaling that is same as LTE. In Rel-15, there aren't any interferences considered in all cases so there is no difference between MMSE-IRC and MMSE-MRC at all in essential. That's the reason that we didn't insist on change to MMSE-MRC during Rel-15 discussion although we ever raised this concern (we think all interested companies ever involved in Rel-15 discussion should know this very clearly). Maybe some companies submitted simulation results by using MMSE-IRC for Rel-15, but this can't preclude that some companies ever submitted results by using MRC. Because there is no differernce at all, the results can be aligned well and also the UEs with MMSE-MRC can also pass those Rel-15 tests. That is also one important motivation for this WI proposal to define performance requirements with inter-cell or intra-cell interference considered to verify if UE can correctly cancellation the interference and ensure good performance in Rel-17 that are not taken into account in Rel-15. It is very unreasonable and unacceptable to mandate Rel-15 UE to pass the performance requirements defined in Rel-17 with totally different assumptions from Rel-15 requirements. This cause unnecessary challenage and testing cost for Rel-15/16 UEs. |
| Apple | We support option 1. Our understanding is that MMSE-IRC is baseline receiver from Rel-15. We don’t see necessity for new UE capability in Rel-17 for ICI as with receiver assumption from Rel-15 the PDSCH requirements in ICI can be met. In case capability has to be introduced then it should be mandatory. |
| CMCC | Option 1. MMSE-IRC is the baseline receiver for NR. No need to introduce new UE feature due to introducing the requirements for new scenario. |
| MediaTek | According to the comments from Intel, LTE use “without capability signaling”. If we do follow LTE, it should be “without capability signaling”. Could Huawei clarify why to use “with capability”? |
| Ericsson | We support option 1. It’s clearly captured the reason is due to limited time, we didn’t finished these requirements in Rel-15 other than RAN4 anticipate some performance enhancement. These Rel-17 demodulation performance requirements should be considered as the continue work from Rel-15. Therefore, we don’t think HW’s arguments are valid. NR Rel-15 demodulation performance requirements was started from April 2018 and completed by December 2019. Several NR features specified in Rel-15 core part are not covered when developing the Rel-15 performance requirements. A Rel-16 WI on Rel-15 demod leftovers was approved in June 2019, which specifies: - UE demod/CSI requirements including CA normal demod, CA CQI reporting, CA power imbalance demod, EN-DC power imbalance demod, PMI reporting for Tx ports larger than 8 and up to 32, TDD LTE-NR co-existence demod requirements; - BS demod requirements including 30% throughput PUSCH, testable PUSCH 2T2R 16QAM requirements for FR2. However, due to the limited time unit, the performance requirements for some important features are still not covered in Rel-16. Therefore, it is proposed to open a new Rel-17 WI to define performance requirements for the selected NR features such as: PDSCH advanced receivers for handling inter-cell and inter-user interference, PUSCH 256QAM requirements for FR1.. |
| China Telecom | We support option 1. |
| Qualcomm | Support Option 1. Similar comments as Ericsson. MMSE-IRC receiver was baseline receiver in Rel-15 and we don’t see any reason to define new capability. |
| Huawei | Thank Intel for information about LTE. Yes, LTE define performance requirements for IRC for inter-cell interference is optional without UE capability without release independence, so we would like to follow LTE and add Option 3. Option 3 is fine for us. - For receiver assumption for NR Rel-15, as we clarified that RAN4 had common understanding that MRC=IRC without interference assumption, it means there is no simulation results difference by using IRC or MRC from companies. So we strongly can’t accept such arguments. - We can understand the UEs from proponents of Option 1 can support IRC from Release 15, but it is up to UE implementation and can’t preclude other possible implementations. - @Ericsson, follow your logic, I am wondering if we can assume that features introduced in following NR releases than Rel-15 should be supported by UE, because all work are not covered in Rel-16 due to limited time unit? - It is very unreasonable and unacceptable to mandate Rel-15 UE to pass the performance requirements defined in Rel-17 with totally different assumptions from Rel-15 requirements. |
| MediaTek | We think it should be applicable from Rel-15. Therefore, we provide option 4. |
| Docomo | We support Option 1. |

Sub-topic 4-2: UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

- Proposals for discussion

  - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

  - Option 2: [Mandatory/optional] with UE capability signaling with granularity per UE

  - Option 3: Optional without UE capability and applicable from Rel-17

- Recommended WF

  - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

  - Option 2: Mandatory with or without UE capability signaling

  - Option 3: Optional with or without UE capability and applicable from Rel-17

| Company | Comment |
| --- | --- |
| Intel | Additional information in comparison to Sub-topic 4-2: Based on our review of 36.101, FDD MU-MIMO test is defined in Section 8.3.1.1 Test 2. Same time, there is no any applicability for this test. Therefore, this test is mandatory for LTE UE. Based on all discussion for this and previous issue, we also support Option 1 here. |
| Huawei | We support option 2 and define optional with capability signaling. Same views with topic 4-1 |
| Apple | We support option 1. Same comments as 4-1. |
| CMCC | Option 1. Same comments as topic 4-1. |
| Ericsson | Option 1. Same comments as topic 4-1. |
| China Telecom | Consider the exist network NR UEs not pass test for Rel-17, we are fine to discuss the UE MMSE-IRC capability under MU-MIMO to be either mandatory with or without signaling. In our understanding, IRC receiving under MU-MIMO is different from that of under ICI in the following aspects: - From UE point of view, due to flexible DMRS port mapping configured by the BS, UE needs to read the DCI and to detect whether there exists co-scheduled UE interference on the same or different CDM group. We are not sure whether all Ues with IRC are able to do this. From BS point of view, , UE’s reported CQI cannot predict its performance under real MU-MIMO, because there is no IUI on the CSI-RS, which is very different from the ICI situation. Therefore, it may be better for UE to inform the BS the support of IRC for MU-MIMO, to better assist the UE pairing algorithm. |
| Qualcomm | Support Option 1. Same comments as 4-1. |
| Huawei | Same logic as 4-1. But based on analysis from China Telecom, if the UE capability of IRC for intra-cell inter-user interference is helpful for gNB to schedule MU-MIMO, it is fine for us to define optional UE capability. So Option 2 and Option 3 are fine for us. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel Corporation |  |
| WF on CSI requirements for inter-cell interference MMSE-IRC | Ericsson |  |
| Summary of simulation results for Inter-cell MMSE-IRC CQI reporting | Ericsson |  |
| WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200380 | TP to TR 38.833: Scenario for inter-user interference suppression for MU-MIMO | MediaTek inc. | Agreeable |  |
| R4-2200516 | TP to TR 38.833: Link level simulation results for Inter-user interference suppression for MU-MIMO | Intel Corporation | Revised |  |
| R4-2200807 | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO | CMCC | Return to |  |
| R4-2200904 | TP to TR 38.833: Updated conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario | China Telecom | Return to |  |
| R4-2200989 | TP to TR 38.833: Introduction of simulation assumptions of intra cell inter user MMSE-IRC receiver | Huawei, HiSilicon | Return to |  |

All other tdocs can be Noted.

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203008 | WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel | Agreeable |  |
| R4-2203009 | WF on CSI requirements for inter-cell interference MMSE-IRC | Ericsson | Agreeable |  |
| R4-2203010 | Summary of simulation results for Inter-cell MMSE-IRC CQI reporting | Ericsson | Noted |  |
| R4-2203011 | WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei | Agreeable |  |
| R4-2203007 | TP to TR 38.833: Link level simulation results for Inter-user interference suppression for MU-MIMO | Intel Corporation | Agreeable |  |
| R4-2200807 | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO | CMCC | Agreeable |  |
| R4-2200904 | TP to TR 38.833: Updated conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario | China Telecom | Agreeable |  |
| R4-2200989 | TP to TR 38.833: Introduction of simulation assumptions of intra cell inter user MMSE-IRC receiver | Huawei, HiSilicon | Agreeable |  |

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)