3GPP TSG-RAN WG4 Meeting # 101-bis-e													R4-2203114

Electronic Meeting, January 17-25, 2022

Agenda item:			6.13.6.1, 6.13.6.2, 6.13.6.3

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [101-bis-e][322] NR_NTN_Demod

Document for:	Information

# Introduction

The summary covers the contributions submitted under the following agendas

- 6.13.6.1 - General requirements

- 6.13.6.1 - Satellite Access Node demodulation requirements

- 6.13.6.3 - UE demodulation requirements

Note: Companies’ comments submitted in NWM  for the 1st round has been captured in R4-2202972.

# Topic #1: General aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary and Companies views’ collection for 1st round

### Issue 1-1: General assumptions on demodulation requirements

| ### T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| ### R4-2200475 | Ericsson | Observation 1: If baseline architecture introduced by RAN3 is considered and feeder link is assumed as ideal transmission, there would be no difference between transparent and non-transparent architecture from demodulation perspective. Observation 2: In reality, there should be remaining frequency, timing and channel impact after feeder link transmission which might impact on demodulation performance of transparent architecture. Observation 3: It is not clear about the satellite transmission power assumption, “fixed gain” or “fixed PSD”. Proposal 1: Satellite industry deliver a proper error and power model for satellite + feeder link. Companies could further study the impact on demodulation based on the model. Observation 4: GEO and LEO have quite different deployment which might need different demodulation requirements. Observation 5: Only FR1 band is considered for Rel-17 NTN requirement discussion. Observation 6: There should be no difference between earth fixed beam and moving beam from demodulation perspective. Observation 7: The delay spread in feeder link could be ignored due to high direction antenna and LOS propagation. The Doppler shift, frequency error could be pre-compensated by GW and satellite, but it would be good to have a model for the remaining error. Proposal 2: Channel model for service link demodulation requirement could base on NTN-TDL_A/B/C/D. Observation 8: In service UL transmission, the Doppler shift and delay spread at satellite side could be small due to frequency pre-compensation and typical LOS scenario. Observation 9: In service DL transmission, the Doppler shift and delay spread at UE side could be high due to no frequency pre-compensation and typical NLOS scenario. Proposal 4: Consider normal speed UE in Rel-15 (up to 120km/h) as start point for further discussion on NTN NR demodulation. |
| ### R4- 2201785 | Intel Corporation | Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition. |
| ### R4-2201420 | Ericsson | Proposal 1: Follow the agreement to only consider transparent architecture for RAN4 demodulation part discussion Proposal 2: Only consider <6GHz frequency band for Rel-17 Proposal 3: Only consider FR1 for UE demodulation requirement |
| ### R4- 2201786 | Intel Corporation | Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition |

Issue 1-1-1: Architecture

- Proposals

  - Option 1: (Ericsson)

    - The architecture defined by RAN3 as baseline. Follow the agreement to only consider transparent architecture for RAN4 demodulation part discussion.

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

Issue 1-1-2: Band and frequency

- Proposals

  - Option 1: (Ericsson)

    - Only consider FR1 demodulation requirements in Rel-17

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

Issue 1-1-3: Frequency/time error and power model.

- Proposals

  - Option 1: (Ericsson)

    - Satellite industry deliver a proper error and power model for satellite + feeder link. Companies could further study the impact on demodulation based on the model.

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

Issue 1-1-4: Earth fixed beam and moving beam.

- Proposals

  - Option 1: (Ericsson)

    - No difference between earth fixed beam and moving beam from demodulation perspective.

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

Issue 1-1-5: General assumption for service link

- Proposals

  - Option 1: (Ericsson)

    - For UL demodulation, Small Doppler shift and delay spread for both GEO and LEO deployment

    - For UE demodulation, NTN-TDL plus Doppler shift  for channel model assumptions.

  - Option 2: (Intel)

    - RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

Issue 1-1-6: UE speed on NTN demodulation

- Proposals

  - Option 1: (Ericsson)

    - Consider normal speed UE in Rel-15 (up to 120km/h) as start point.

- Recommended WF

  - Companies are encouraged to provide their views on this issue.

| Company | Comments |
| --- | --- |

## Summary for 1st round

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Issue 1-1: General impact on demodulation requirements

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Architecture | Tentative agreements: The architecture defined by RAN3 as baseline. Follow the agreement to only consider transparent architecture for RAN4 demodulation part discussion. Recommendations for 2nd round: None |
| Issue 1-1-2: Band and frequency | Tentative agreements: Only consider FR1 demodulation requirements in Rel-17 Recommendations for 2nd round: None |
| Issue 1-1-3: Frequency/time error and power model. | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the frequency/timing error and power model in the 2nd round For frequency/timing error, the following options can be discussed: Option 1: For service link, ±0.1ppm of carrier frequency is consider. For feeder link, the frequency error is negligible. Option 2: Satellite companies to deliver a practical frequency/timing model for service link and feeder link. Option 3: Consider the floating boundary model due to the satellite motion For power model, the following options can be discussed: Option 1: Satellite companies to deliver a practical power model, e.g., fixed gain or fixed PSD Option 2: Fixed SNR at UE and SAN side |
| Issue 1-1-4: Earth fixed beam and moving beam. | Tentative agreements: No difference between earth fixed beam and moving beam from demodulation perspective. RAN4 to only consider one set of cases to cover both earth fixed beam and moving beam. Recommendations for 2nd round: None |
| Issue 1-1-5: General assumptions for service link | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the doppler shift and delay spread for service link in the 2nd round: Option 1: For UL, Small Doppler shift and delay spread is assumed. For DL, Doppler shift should be considered. Option 2: For UL and DL, Small Doppler shift and delay spread is assumed after UE time and frequency compensation Option 3: For UL and DL, RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account. |
| Issue 1-1-6: UE speed on NTN demodulation | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the UE speed assumptions in the 2nd round: Option 1: Up to 120km/h is the start point for UE speed Option 2: No need to define the UE speed since the residual time and frequency offset can be very small regardless the UE speed |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 1-1-3a: Frequency/time error model.

Proposals:

For frequency/timing error, the following options can be discussed:

- Option 1: For service link, ±0.1ppm of carrier frequency is consider. For feeder link, the frequency error is negligible.

- Option 2: Satellite companies to deliver a practical frequency/timing model for service link and feeder link.

- Option 3: Consider the floating boundary model due to the satellite motion

Recommended WF

- Satellite companies are encouraged to deliver a practical frequency/timing model for service link and feeder link

- FFS on whether to consider the floating boundary model due to the satellite motion

| Company | Comments |
| --- | --- |

Issue 1-1-3b: Power model.

Proposals:

For power model, the following options can be discussed:

- Option 1: Satellite companies to deliver a practical power model, e.g., fixed gain or fixed PSD

- Option 2: Fixed SNR at UE and SAN side

- Option 1: Satellite companies to deliver a practical power model, e.g., fixed gain or fixed PSD

- Option 2: Fixed SNR at UE and SAN side

Recommended WF

Satellite companies are encouraged to deliver a practical power model, e.g., fixed gain or fixed PSD

| Company | Comments |
| --- | --- |

Issue 1-1-5: General assumptions for service link and feeder link

Proposals:

Further discuss the doppler shift and delay spread for service link:

- Option 1: For UL, Small Doppler shift and delay spread is assumed. For DL, Doppler shift should be considered.

- Option 2: For UL and DL, Small Doppler shift and delay spread is assumed after UE time and frequency compensation

- Option 3: For UL and DL, RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account.

Recommended WF

For UL and DL, RAN4 to further discuss the Doppler shift and Delay spread after UE time and frequency compensation for service link and feeder link.

| Company | Comments |
| --- | --- |

Issue 1-1-6: UE speed on NTN demodulation

Proposals:

Further discuss the UE speed assumptions

- Option 1: Up to 120km/h is the start point for UE speed

- Option 2: No need to define the UE speed since the residual time and frequency offset can be very small regardless the UE speed

Recommended WF

- Up to 120km/h is the start point for UE speed. Further discuss whether need to define the UE speed for NTN demodulation.

| Company | Comments |
| --- | --- |

# Topic #2: Satellite Access Node demodulation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary and Companies views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 2-1: Scope of requirements

| R4-2201016 | Huawei, HiSilicon | Proposal 1: Do not specify K_offset and K_mac parameters for satellite performance requirements definition. Proposal 2: Do not define satellite performance requirements with HARQ Processes 32. Proposal 3: For satellite performance requirements derived from legacy BS requirements, only consider Rel-15 and Rel-16 features. Case by case to decide whether to reuse or modify some test parameters if needed. Most of test parameters should be reused as much as possible to reduce simulation workload. |
| --- | --- | --- |
| R4- 2201785 | Intel Corporation | Proposal 3: NTN Satellite Access Node is not required to pass legacy Rel-15/Rel-16 tests |
| R4-2200476 | Ericsson | Observation 5: It is not clear whether the principle of applicability method in Rel-15/16 TN gNB demodulation requirements could be the same for satellite access node demodulation requirement. |

Issue 2-1-1: Legacy BS Rel-15/Rel-16 demodulation requirements apply to NTN Satellite Access Node or not?

- Proposals

  - Option 1: (Huawei)

    - Yes. Most of test parameters from legacy BS should be reused as much as possible to reduce simulation workload.

  - Option 2: (Intel)

    - No

  - Option 3: (Ericsson)

    - FFS

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-1-2: Enhancement on time relationship

- Proposals

  - Option 1: (Huawei)

    - Do not specify K_offset and K_mac parameters for satellite performance requirements definition.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-1-3: Enhancement on HARQ

- Proposals

  - Option 1: (Huawei)

    - Do not define satellite performance requirements with HARQ Processes 32.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 2-2: General assumptions

| R4-2200171 | CATT | Proposal 1: To discuss channel model for NTN PUSCH demodulation and its parameters such as maximum DS and maximum Doppler. |
| --- | --- | --- |
| R4-2200476 | Ericsson | Observation 1: Small Doppler shift and delay spread in UL demodulation for both GEO and LEO deployment because pre-compensation for Doppler is mandatory and satellites are LOS condition. Observation 2: Same channel model might be applied for UL demodulation requirement for both GEO and LEO deployment if satellite companies confirm the remaining Doppler error is similar in both deployments after pre-compensation. Proposal 1: Start with FR1 FDD band SCS 15kHz for UL demodulation requirements. The selection of bandwidth could depend on the simulation results. Observation 3: Similar UE assumption as Rel-15/16 could be reused for NTN demodulation requirement. Proposal 2: 1Tx2Rx could be the start point for NTN UL demodulation discussion. |
| R4-2201016 | Huawei, HiSilicon | - Proposal 4: To simplify the test setup, the synchronization method during the test can be reused from the legacy BS testing for satellite performance requirements definition. Proposal 5: For NTN satellite performance requirements, introduce NTN-TDL-A/B/C/D for satellite performance requirements definition and select 100ns and 300ns as delay spread. Permutation and combination can be used such as: NTN-TDLA100, NTN-TDLB300, NTN-TDLC100, NTN-TDLD300. Proposal 6: NTN-TDL fading channel model should be selected to replace the legacy TDL fading channel model for NTN requirements definition when applicable. Proposal 7: Doppler can be select as 200Hz that is 0.1 ppm of carrier frequency corresponding to n256. Proposal 8: For NTN satellite performance requirements, select 5MHz, 10MHz and 20MHz bandwidth for 15kHz SCS while 10MHz and 20MHz bandwidth for 30kHz SCS. |
| R4- 2201785 | Intel Corporation | Proposal 1: RAN4 to define the requirements for PUSCH, PUCCH and PRACH for NTN-TDL channel models |

Issue 2-2-1: Channel model

- Proposals

  - Option 1: (Ericsson, Huawei, Intel)

    - NTN-TDL-A/B/C/D.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-2-2: Delay spread

- Proposals

  - Option 1: (Huawei)

    - Select 100ms and 300ms delay spread, i.e., NTN-TDLA100, NTN-TDLB300, NTN-TDLC100, NTN-TDLD300.

  - Option 2: (CATT)

    - To further discuss the maximum delay spread

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-2-3: Doppler shift

- Proposals

  - Option 1: (Huawei)

    - 200Hz for n256

  - Option 2: (CATT)

    - To further discuss the maximum Doppler shift

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-2-4: Tx and Rx assumptions for UL demodulation

- Proposals

  - Option 1: (Ericsson)

    - 1Tx, 2Rx as the start point.

  - Option 2: (Huawei)

    - 1/2Tx, 2/4/8Rx

  - Option 3: (Intel)

    - 1Tx, 2/4/8Rx

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-2-5: UL time and frequency synchronization

- Proposals

  - Option 1: (Huawei)

    - Reuse the synchronization method from legacy BS testing.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 2-3: PUSCH assumptions

| R4-2200171 | CATT | Proposal 1: To discuss channel model for NTN PUSCH demodulation and its parameters such as maximum DS and maximum Doppler. Proposal 2: To discuss the channel model with frequency offset for NTN PRACH demodulation. Proposal 3: To discuss timing offset value for NTN PRACH demodulation. |
| --- | --- | --- |
| R4-2200476 | Ericsson | Observation 4: New demodulation requirements for PUSCH, PUCCH, PRACH and UL TA could be considered for NTN due to new channel model and new scenarios. The detailed assumptions need further discussion. Observation 5: It is not clear whether the principle of applicability method in Rel-15/16 TN gNB demodulation requirements could be the same for satellite access node demodulation requirement. |
| R4-2201016 | Huawei,HiSilicon | Proposal 9: Do not consider high speed train scenario, URLLC scenario and interlaced PUSCH scenario for NTN PUSCH requirements definition. Do not consider UCI multiplexed on PUSCH cases for NTN PUSCH requirements definition. Other PUSCH requirements can be reused. Proposal 10: For NTN satellite performance requirements, consider QPSK and 16QAM with high priority and follow outcome from RF side to decide whether 64QAM is considered. Proposal 11: Only consider QPSK for NTN PUSCH requirements for NTN PUSCH requirements definition. |
| R4-2201785 | Intel Corporation | Requirement; Parameter; Values. Requirements for PUSCH with transform precoding disabled; Fraction of maximum throughput 70%; Number of TX antennas; 1. Number of RX antennas; 2, 4, 8. Modulation order; QPSK, 16QAM 64QAM can be added based on the agreement in RF room. CBW / SCS; 15kHz SCS: 5MHz, 10MHz, 15MHz, 20MHz 30kHz SCS: 10MHz, 15MHz, 20MHz. Propagation conditions; Different combinations of NTN-TDL channels and FRC to be tested. PUSCH mapping type; A, B. Fraction of maximum throughput 30%; Number of TX antennas; 1. Number of RX antennas; 2. Modulation order; QPSK. CBW / SCS; 15 kHz / 5MHz 30 kHz / 10MHz. Propagation conditions; One NTN TDL channel to be chosen. PUSCH mapping type; A, B. Requirements for PUSCH with transform precoding enabled; Number of TX antennas; 1. Number of RX antennas; 2, 4, 8. Modulation order; QPSK. CBW / SCS; 15 kHz / 5MHz 30 kHz / 10MHz. Propagation conditions; One NTN TDL channel to be chosen. PUSCH mapping type; A, B. Requirements for UCI multiplexed on PUSCH; Number of TX antennas; 1. Number of RX antennas; 2. Modulation order; 16QAM. CBW / SCS; 30 kHz / 10MHz. Propagation conditions; One NTN TDL channel to be chosen. PUSCH mapping type; A, B. Requirements for UL timing adjustment; Number of TX antennas; 1. Number of RX antennas; 2. Modulation order; 16QAM. CBW / SCS; 15 kHz / 5MHz 30 kHz / 10MHz. Propagation conditions; FFS. PUSCH mapping type; A, B. Proposal 4: RAN4 to define PUSCH requirements for NR NTN according to the scope shown in Table 2-1. |

Issue 2-3-1: PUSCH requirements

- Proposals

  - Option 1: (Huawei)

    - To reuse the following Rel-15/16 requirements except SCS/CBW set and modulation order

      - Transform precoding disabled, rank 1/2, 2/4/8 Rx

      - Transform precoding enabled, rank 1, 2/4/8 Rx

      - UL timing adjustment, rank 1, 2 Rx

      - repetition Type A, rank 1, 2 Rx

      - mapping Type B with non-slot transmission, rank 1, 2 Rx

      - 2-step RA type, rank 1, 2 Rx

  - Option 2: (Intel)

    - To reuse the following Rel-15/16 requirements except SCS/CBW set  and modulation order

      - Transform precoding disabled, rank 1, 2/4/8 Rx

      - Transform precoding enabled, rank 1, 2/4/8 Rx

      - UCI multiplexed on PUSCH, rank 1, 2 Rx

      - UL timing adjustment, rank 1, 2 Rx

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-3-2: SCS/CBW set for PUSCH requirements

- Proposals

  - Option 1: (Huawei)

    - 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

  - Option 2: (Intel)

    - 15kHz SCS: 5/10/15/20MHz, 30kHz SCS: 10/15/20MHz for fraction of maximum throughput 70% for requirements for PUSCH with transform precoding disabled

    - 15 kHz SCS: 5MHz, 30 kHz SCS: 10MHz for fraction of maximum throughput 30% for requirements for PUSCH with transform precoding disabled

    - 15 kHz SCS: 5MHz, 30 kHz SCS: 10MHz for requirements for PUSCH with transform precoding enabled

    - 30 kHz SCS: 10MHz for requirements for UCI multiplexed on PUSCH

    - 15 kHz SCS: 5MHz, 30 kHz SCS: 10MHz for requirements for UL timing adjustment

  - Option 3 (Ericsson)

    - 15kHz SCS: CBW is FFS

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-3-3: Modulation order for PUSCH requirements

- Proposals

  - Option 1: (Huawei)

    - QPSK for all cases

  - Option 2: (Intel)

    - QPSK, 16QAM for fraction of maximum throughput 70% for requirements for PUSCH with transform precoding disabled

    - QPSK for fraction of maximum throughput 30% for requirements for PUSCH with transform precoding disabled

    - QPSK for requirements for PUSCH with transform precoding enabled

    - 16QAM for requirements for UCI multiplexed on PUSCH

    - 16QAM for requirements for UL timing adjustment

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 2-4: PUCCH assumptions

| R4-2201016 | Huawei,HiSilicon | Proposal 12: Do not consider interlaced PUCCH for NTN PUCCH requirements definition. Other PUCCH requirements can be reused. |
| --- | --- | --- |
| R4-2201785 | Intel Corporation | Proposal 5: RAN4 to define requirements for PUCCH Formats 0-4 and for the same test parameters as for Rel-15 PUCCH requirements except propagation conditions (NTN TDL should be used) and CBW set. The CBW set for requirement definition should follow Table 2-2 Table 2-2. NTN CBW set to for requirement definition SCS kHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz. 15; Yes; Yes; Yes; Yes. 30; Yes; Yes; Yes. |

Issue 2-4-1: PUCCH requirements

- Proposals

  - Option 1: (Huawei, Intel)

    - To reuse the following legacy TN BS requirements except SCS/CBW set

      - PUCCH format 0/1/2/3/4, 2/4/8 Rx

      - Multi-slot PUCCH format 1, 2 Rx

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 2-4-2: SCS/CBW set  for PUCCH requirements

- Proposals

  - Option 1: (Huawei)

    - 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

  - Option 2: (Intel)

    - 15kHz SCS: 5/10/15/20MHz, 30kHz SCS: 10/15/20MHz

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

### Issue 2-5: PRACH assumptions

| R4-2201016 | Huawei,HiSilicon | Proposal 13: Do not consider AWGN channel and do not consider high speed train scenario for NTN PRACH requirements definition. Other PRACH requirements can be reused. Proposal 14: Only consider typical B4/C2 preamble format for NTN satellite requirements. |
| --- | --- | --- |
| R4-2201785 | Intel Corporation | Proposal 6: For NTN PRACH, RAN4 to reuse Rel-15 requirements for false alarm probability and missed detection in AWGN channel Proposal 7: RAN4 to define requirements for PRACH time error tolerance and missed detection for NTN TDL channel considering the same set of test parameters as in Rel-15 except the set of PRACH preambles to be tested. Proposal 8: RAN4 to define the set of PRACH preambles to be tested for NR NTN. |
| R4-2200171 | CATT | Proposal 2: To discuss the channel model with frequency offset for NTN PRACH demodulation. Proposal 3: To discuss timing offset value for NTN PRACH demodulation. |

Issue 2-5-1: PRACH requirements

- Proposals

  - Option 1: (Huawei)

    - Do not consider AWGN channel and do not consider high speed train scenario for NTN PRACH requirements definition. Other PRACH requirements can be reused.

    - Only consider typical B4/C2 preamble format for NTN satellite requirements.

  - Option 2: (Intel)

    - For NTN PRACH, RAN4 to reuse Rel-15 requirements for false alarm probability and missed detection in AWGN channel

    - RAN4 to define requirements for PRACH time error tolerance and missed detection for NTN TDL channel considering the same set of test parameters as in Rel-15 except the set of PRACH preambles to be tested.

    - RAN4 to define the set of PRACH preambles to be tested for NR NTN.

  - Option 3: (CATT)

    - To discuss the channel model with frequency offset for NTN PRACH demodulation

    - To discuss timing offset value for NTN PRACH demodulation

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

## Summary for 1st round

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Issue 2-1: Scope of requirements

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Legacy BS Rel-15/Rel-16 demodulation requirements apply to NTN Satellite Access Node or not? | Tentative agreements: N/A Recommendations for 2nd round: Further discuss if the following option is acceptable for companies in the 2nd round: Option 1: For SAN, RAN4 to define the new demodulation requirements. Whether the legacy test parameters can be reused is FFS. |
| Issue 2-1-2: Enhancement on time relationship | Tentative agreements: Do not specify K_offset and K_mac parameters for SAN performance requirements definition. Recommendations for 2nd round: None |
| Issue 2-1-3: Enhancement on HARQ | Tentative agreements: Do not define satellite performance requirements with HARQ Processes Recommendations for 2nd round: None |

### Issue 2-2: General assumptions

|  | Status summary |
| --- | --- |
| Issue 2-2-1: Channel model | Tentative agreements: NTN-TDL-A/B/C/D.as the starting point. Recommendations for 2nd round: Further discuss whether down-selection is needed or not. Option 1: Yes, specify the down-selection channel model Option 2: No |
| Issue 2-2-2: Delay spread | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the Delay spread based on TR38.811. Companies can provide the proposals in the 2nd round if any. |
| Issue 2-2-3: Doppler shift | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the residual error after UE pre-compensation in the 2nd round: Option 1: 200Hz for n256 Option 2: FFS |
| Issue 2-2-4: Tx and Rx assumptions for UL demodulation | Tentative agreements: N/A Recommendations for 2nd round: Further discuss the Tx and Rx assumptions for SAN demodulation in the 2nd round: Option 1: 1Tx, 2Rx as the starting point Option 2: 1Tx/2Tx, 2/4/8Rx |
| Issue 2-2-5: UL time and frequency synchronization | Tentative agreements: Reuse the synchronization method from legacy BS testing Recommendations for 2nd round: N/A |

### Issue 2-3: PUSCH assumptions

| Issue 2-3-1: PUSCH requirements | Tentative agreements: To define the following requirements and further discuss on Tx/Rx configurations - Transform precoding disabled - Transform precoding enabled - UL timing adjustment - repetition Type A FFS on other requirements. Recommendations for 2nd round: To further discuss whether to define the following requirements Option 1: To further consider 2-step RA type requirement Option 2: To further consider mapping Type B with non-slot transmission requirements |
| --- | --- |
| Issue 2-3-2: SCS/CBW set for PUSCH requirements | Tentative agreements: To consider both 15khz and 30khz SCS. FFS on CBW and test metric. Recommendations for 2nd round: Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz Option 2: FFS based on the link budget |
| Issue 2-3-3: Modulation order for PUSCH requirements | Tentative agreements: N/A Recommendations for 2nd round: Option 1: QPSK for all cases Option 2: QPSK, 16QAM Moderator’s note: Agreement from RF session: - Include 64QAM as optional with manufacture declaration basis for SAN. - Include 64QAM (DL and UL ) for NTN satellite UE as optional feature with granularity [per UE] |

### Issue 2-4: PUCCH assumptions

| Issue 2-4-1: PUCCH requirements | Tentative agreements: To define the requirements for PUCCH formant 01/2/3/4. The Rx configuration is FFS. Recommendations for 2nd round: Whether to define Multi-slot PUCCH format 1, 2 Rx requirement |
| --- | --- |
| Issue 2-4-2: SCS/CBW set for PUCCH requirements | Tentative agreements: N/A Recommendations for 2nd round: Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz Option 2: FFS based on the link budget |

### Issue 2-5: PRACH assumptions

| Issue 2-5-1: PRACH requirements | Tentative agreements: Do not consider high speed train scenario for NTN PRACH requirements definition Recommendations for 2nd round: To further discuss whether to consider the AWGN and preambles in the 2nd round Option 1: Do not consider the AWGN and only consider typical B4/C2 preamble format Option 2: FFS |
| --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 2-1-1: Legacy BS Rel-15/Rel-16 demodulation requirements apply to NTN Satellite Access Node

or not?

Proposals:

Further discuss if the following option is acceptable for companies or not:

- Option 1: For SAN, RAN4 to define the new demodulation requirements. Whether the legacy test parameters can be reused is FFS.

| Company | Comments |
| --- | --- |
| Ericsson | We don’t think “Whether the legacy test parameters can be reused is FFS.” is necessary. The test parameters need analysis case by case anyway. |

Issue 2-2-1: Channel model

Proposals:

NTN-TDL-A/B/C/D as the starting point. Further discuss whether down-selection is needed or not.

- Option 1: Yes, specify how to down-select channel model

- Option 2:  No

| Company | Comments |
| --- | --- |

Issue 2-2-2: Delay spread

Proposals:

Further discuss the Delay spread based on TR38.811. Companies can provide the proposals:

- Option 1: Companies can provide the proposals if any.

| Company | Comments |
| --- | --- |

Issue 2-2-3: Doppler shift

Proposals:

Further discuss the residual error after UE pre-compensation:

- Option 1: 200Hz for n256

- Option 2: FFS

| Company | Comments |
| --- | --- |

Issue 2-2-4: Tx and Rx assumptions for UL demodulation

Proposals:

Further discuss the Tx and Rx assumptions for SAN demodulation:

- Option 1: 1Tx, 2Rx as the starting point

- Option 2: 1Tx/2Tx, 2/4/8Rx

| Company | Comments |
| --- | --- |
| Ericsson | Checking LLS assumption in TR38.821 Table 6.1.2-2 and Table 6.1.2-4, 1Tx1Rx is assumed and 2Rx is optional. In that case, we propose to consider Option 3: 1Tx, 1/2Rx as the start point, FFS for 2Tx, 4/8Rx. |

Issue 2-3-1: PUSCH requirements

Proposals:

To further discuss whether to define the following requirements

- Option 1: To further consider 2-step RA type requirement

- Option 2: To further consider mapping Type B with non-slot transmission requirements

| Company | Comments |
| --- | --- |

Issue 2-3-2: SCS/CBW set for PUSCH requirements

Proposals:

- Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

- Option 2: FFS based on the link budget

| Company | Comments |
| --- | --- |

Issue 2-3-3: Modulation order for PUSCH requirements

Proposals:

- Option 1: QPSK for all cases

- Option 2: QPSK, 16QAM

| Company | Comments |
| --- | --- |
| Huawei | We can agree to introduce QPSK for PUSCH requirements firstly and FFS whether to introduce 16QAM for PUSCH requirements and do not introduce 64QAM for PUSCH requirements. |
| Ericsson | We think 64QAM is hard to be fulfilled in UL transmission, but it could be included in FFS with 16QAM. Companies can further check if it is feasible for requirement. |

Issue 2-4-1: PUCCH requirements

Whether to define Multi-slot PUCCH format 1, 2 Rx requirement

- Option 1: Yes

- Option 2: No

| Company | Comments |
| --- | --- |

Issue 2-4-2: SCS/CBW set for PUCCH requirements

- Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

- Option 2: FFS based on the link budget

| Company | Comments |
| --- | --- |

Issue 2-5-1: PRACH requirements

Proposals:

To further discuss whether to consider the AWGN and preambles in the 2nd round

- Option 1: Do not consider the AWGN and only consider typical B4/C2 preamble format

- Option 2: FFS

| Company | Comments |
| --- | --- |

# Topic #3: NTN UE demodulation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary and Companies views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 3-1: Scope of requirements

| R4-2201015 | Huawei, HiSilicon | Proposal 1: Only define incremental performance requirements for NTN. Proposal 2: Consider satellite-based NTN scenario as high priority. Proposal 3: Time relationship between uplink and downlink should be modified for the following cases if new requirements are defined for NTN scenario, selecting K_offset equal to the propagation delay defined in the channel model. - PDSCH cases: Type 2 HARQ codebook based HARQ-ACK/NACK feedback via PUCCH - PDCCH cases: Type 2 HARQ codebook based HARQ-ACK/NACK feedback via PUCCH (Same as PDSCH cases, it is RAN5 test setup design to collecting PDCCH BLER statistics by counting HARQ-ACK/NACK feedback for PDCCH scheduling PDSCH) - CSI reporting case: Periodic CSI reporting via PUCCH or aperiodic CSI reporting via PUSCH Proposal 5: Define PDSCH performance requirements with HARQ Processes 32. Proposal 6: Define PDSCH performance requirements with the condition that half HARQ process is enabled while another half HARQ process is disabled. Proposal 7: Study a new test method for PDSCH test with disabled HARQ feedback. |
| --- | --- | --- |
| R4-2201420 | Ericsson | Proposal 5: Evaluate all three satellite deployments and down select before defining test cases and requirement, e.g. define requirement for GEO and LEO 600 |
| R4- 2201786 | Intel Corporation | Proposal 3: NTN UE is required to pass legacy Rel-15/Rel-16 tests Proposal 4: RAN4 to define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes. Proposal 5: There is no need to repeat all the tests from the legacy set for new NTN channel models. Limited set of tests can be defined for NTN PDSCH and PDCCH |

Issue 3-1-1: Legacy TN UE Rel-15/Rel-16 demodulation requirements apply to NTN UE or not?

- Proposals

  - Option 1: (Huawei, Intel)

    - Yes. Only consider the incremental performance requirements for NTN.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-1-2: Satellite-based NTN and HAPS scenarios

- Proposals

  - Option 1: (Huawei)

    - Consider satellite-based NTN scenario as high priority.

  - Option 2: (Ericsson)

    - Option 2: Evaluate all three satellite deployments and down select before defining test cases and requirement, e.g. define requirement for GEO and LEO 600

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-1-3: Enhancement on time relationship

- Proposals

  - Option 1: (Huawei)

    - Time relationship between uplink and downlink should be modified for the following cases if new requirements are defined for NTN scenario, selecting K_offset equal to the propagation delay defined in the channel model.

      - PDSCH cases: Type 2 HARQ codebook based HARQ-ACK/NACK feedback via PUCCH

      - PDCCH cases: Type 2 HARQ codebook based HARQ-ACK/NACK feedback via PUCCH (Same as PDSCH cases, it is RAN5 test setup design to collecting PDCCH BLER statistics by counting HARQ-ACK/NACK feedback for PDCCH scheduling PDSCH)

      - CSI reporting case: Periodic CSI reporting via PUCCH or aperiodic CSI reporting via PUSCH

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-1-4: Enhancement on HARQ

- Proposals

  - Option 1: (Huawei)

    - Define PDSCH performance requirements with HARQ Processes 32.

    - Define PDSCH performance requirements with the condition that half HARQ process is enabled while another half HARQ process is disabled

    - Study a new test method for PDSCH test with disabled HARQ feedback.

  - Option 2: (Intel)

    - Define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 3-2: General assumptions

| R4-2201015 | Huawei, HiSilicon | Proposal 8: For NTN UE performance requirements, select one NLOS conditions channel model from NTN-TDL-A/B and one LOS conditions channel model NTN-TDL-C/D. Proposal 9: For NTN UE performance requirements, select 300ns as delay spread. |
| --- | --- | --- |
| R4-2201420 | Ericsson | Proposal 6: Consider only NTN-TDL-A/B/C/D plus Doppler shift for channel model assumption. |
| R4- 2201786 | Intel Corporation | Proposal 1: RAN4 to define the requirements for PDSCH and PDCCH for NTN-TDL channel models |

Issue 3-2-1: Channel model

- Proposals

  - Option 1: (Ericsson, Intel)

    - NTN-TDL-A/B/C/D

  - Option 2: (Huawei)

    - Select one NLOS conditions channel model from NTN-TDL-A/B and one LOS conditions channel model NTN-TDL-C/D

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-2-2: Delay spread

- Proposals

  - Option 1: (Ericsson)

    - Discusses the time and frequency offset for UE demodulation test cases, and takes UE speed into account

  - Option 2: (Huawei)

    - Select 300ns as delay spread

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-2-3: Doppler shift

- Proposals

  - Option 1: (Ericsson)

    - Discusses the time and frequency offset for UE demodulation test cases, and takes UE speed into account

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 3-3: PDSCH/PDCCH/PBCH assumptions

| R4-2201015 | Huawei, HiSilicon | Proposal 10: For NTN UE performance requirements, select 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS. Proposal 11: For NTN UE performance requirements, consider QPSK and 16QAM with high priority and follow outcome from RF side to decide whether 64QAM is considered. |
| --- | --- | --- |
| R4-2201420 | Ericsson | Proposal 4: Suggest considering FDD 10MHz bandwidth and 15kHz SCS for initial evaluation Proposal 8: New demodulation requirement for PBCH, PDSCH and PDCCH can be considered for NTN. The detailed assumptions need further discussion |
| R4- 2201786 | Intel Corporation | Proposal 1: RAN4 to define the requirements for PDSCH and PDCCH for NTN-TDL channel models |

Issue 3-3-1: PDCSH/PDCCH/PBCH requirements

- Proposals

  - Option 1: (Ericsson)

    - Define new demodulation requirements for PDCSH/PDCCH/PBCH

  - Option 2: (Intel)

    - Define new demodulation requirements for PDSCH/PDCCH

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-3-2: SCS/CBW set for PDSCH requirements

- Proposals

  - Option 1: (Huawei)

    - 15kHz SCS: 10MHz, 30kHz SCS: 20MHz

  - Option 2: (Ericsson)

    - 15kHz SCS: 10MHz as the starting point

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

Issue 3-3-3: Modulation order

- Proposals

  - Option 1: (Huawei)

    - Consider QPSK and 16QAM with high priority and follow outcome from RF side to decide whether 64QAM is considered

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

### Issue 3-4: CSI reporting assumptions

| R4-2201015 | Huawei,HiSilicon | Proposal 4: Do not consider any CSI reporting requirements for NTN scenario. |
| --- | --- | --- |
| R4-2201420 | Ericsson | Observation 3: The time shift could be large due to the long distance between UE and the NTN-payload and the quickness of the NTN-payload Proposal 9: Evaluate the time shift of CSI reporting before having any decision on whether to have CSI reporting requirement for NTN |

Issue 3-4-1: CSI reporting requirements

- Proposals

  - Option 1: (Huawei)

    - Do not consider any CSI reporting requirements for NTN scenario.

  - Option 2: (Ericsson)

    - Evaluate the time shift of CSI reporting before having any decision on whether to have CSI reporting requirement for NTN

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |

## Summary for 1st round

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Issue 3-1: Scope of requirements

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Legacy TN UE Rel-15/Rel-16 demodulation requirements apply to NTN UE or not? | Tentative agreements: Legacy TN UE Rel-15/Rel-16 demodulation requirements shall also apply for NTN UE. Recommendations for 2nd round: None. The new requirements for NTN UE will be handled in issue 3-3. |
| Issue 3-1-2: Satellite-based NTN and HAPS scenarios | Tentative agreements: Satellite-based requirements is with high priority. Recommendations for 2nd round: Further discuss whether to define the sperate requirements for GEO and LEO Option 1: To define the separate requirements for GEO and LEO1200/600 Option 2: Only to define the requirements for LEO600 |
| Issue 3-1-3: Enhancement on time relationship | Tentative agreements: N/A Recommendations for 2nd round: To further discuss the enhancement on time relationship Option 1: selecting K_offset equal to the propagation delay defined Option 2: selecting K_offset large enough to ensure UL/DL timeline is met |
| Issue 3-1-4: Enhancement on HARQ | Tentative agreements: N/A Recommendations for 2nd round: o Option 1:  Define PDSCH performance requirements with HARQ Processes 32. FFS on optional or mandatory testing  Define PDSCH performance requirements with the condition that half HARQ process is enabled while another half HARQ process is disabled  FFS on test method for PDSCH test with disabled HARQ feedback. o Option 2:  Define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes (To clarify what is the number of HARQ progress) . |

### Issue 3-2: General assumptions

|  | Status summary |
| --- | --- |
| Issue 3-2-1: Channel model | Tentative agreements: N/A Recommendations for 2nd round: To further discuss the possible down-selectin from NTN-TDL-A/B/C/D Option 1: one NLOS conditions channel model from NTN-TDL-A/B and one LOS conditions channel model NTN-TDL-C/D Option 2: FFS |
| Issue 3-2-2: Delay spread | Tentative agreements: N/A Recommendations for 2nd round: Option 1: 250ns Option 2: FFS |
| Issue 3-2-3: Doppler shift | Tentative agreements: N/A Recommendations for 2nd round: Option 1: To discuss the residual time and frequency offset for UE demodulation, and takes UE speed into account. Option 2: Not to consider the residual time and frequency offset for UE demodulation |

### Issue 3-3: PDSCH/PDCCH/PBCH assumptions

| Issue 3-3-1: PDCSH/PDCCH/PBCH requirements | Tentative agreements: N/A Recommendations for 2nd round: Option 1: Define new demodulation requirements for PDCSH/PDCCH/PBCH Option 2: Define new demodulation requirements for PDCSH |
| --- | --- |
| Issue 3-3-2: SCS/CBW set for PDSCH requirements | Tentative agreements: N/A Recommendations for 2nd round: Option 1: 15kHz SCS: 10MHz as the starting point Option 2: 15kHz SCS: 10MHz, 30kHz SCS: 20MHz |
| Issue 3-3-3: Modulation order | Tentative agreements: N/A Recommendations for 2nd round: Option 1: consider QPSK and 16QAM for PDSCH Option 2: consider QSPK, 16QAM and 64QAM for PDSCH Moderator’s note: Agreement from RF session: - Include 64QAM as optional with manufacture declaration basis for SAN. - Include 64QAM (DL and UL ) for NTN satellite UE as optional feature with granularity [per UE] |

### Issue 3-4: CSI reporting assumptions

| Issue 3-4-1: CSI reporting requirements | Tentative agreements: N/A Recommendations for 2nd round: Option 1: Do not consider any CSI reporting requirements Option 2: Do not consider PMI and RI reporting requirements, FFS for CQI reporting for LEO. |
| --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 3-1-2: Satellite-based NTN and HAPS scenarios

Proposals:

Further discuss whether to define the sperate requirements for GEO and LEO

- Option 1: To define the separate requirements for GEO and LEO1200/600

- Option 2: Only to define the requirements for LEO600

Recommended WF

- Further discuss whether RAN4 to define the sperate requirements for GEO and LEO

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1-2: Satellite-based NTN and HAPS scenarios We suggest adding “lower priority for HAPS”. |

Issue 3-1-3: Enhancement on time relationship

Proposals：

To further discuss the enhancement on time relationship

- Option 1: selecting K_offset equal to the propagation delay defined

- Option 2: selecting K_offset large enough to ensure UL/DL timeline is met

Recommended WF

- RAN4 to discuss how to select K_offset to ensure UL/DL timeline is met

| Company | Comments |
| --- | --- |

Issue 3-1-4: Enhancement on HARQ

Proposals:

- Option 1:

  - Define PDSCH performance requirements with HARQ Processes 32. FFS on optional or mandatory testing.

  - Define PDSCH performance requirements with the condition that half HARQ process is enabled while another half HARQ process is disabled

  - FFS on test method for PDSCH test with disabled HARQ feedback.

- Option 2:

  - Define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes (To clarify what is the number of HARQ progress).

Recommended WF

- Define PDSCH performance requirements with HARQ Processes 32. FFS on optional or mandatory testing 32 HARQ Processes

- Define PDSCH performance requirements with disabled HARQ. The disable HARQ processes number is FFS.

- FFS on test method for PDSCH test with disabled HARQ feedback

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 3-1-4 We think that 32 HARQ processes can be optionally tested as it is not a mandatory feature. Also, disabled HARQ can be tested with the number of re-Tx set to 1, without requiring a special test. |
| Huawei | From our understanding, 32 HARQ processes requirements should be tested only when UE supports “26-5 Increasing the number of HARQ processes” that is optional with capability signalling. For the test setup of the disabled HARQ requirement, we can further check until next meeting. |
| Huawei | For the applicability for the PDSCH performance requirements with HARQ Processes 32 and disabled HARQ feedback, we further clarify the wording to make it clearer. “– Define applicability rule that the PDSCH performance requirements with HARQ Processes 32 shall apply for UEs which support RAN1 UE feature “26-5 Increasing the number of HARQ processes” only. ” |

Issue 3-2-1: Channel model

Proposals:

To further discuss the possible down-selectin from NTN-TDL-A/B/C/D

- Option 1: one NLOS conditions channel model from NTN-TDL-A/B and one LOS conditions channel model NTN-TDL-C/D

- Option 2: FFS

Recommended WF

- RAN4 to select one NLOS channel model from NTN-TDL-A/B and one LOS channel model from NTN-TDL-C/D.

| Company | Comments |
| --- | --- |

Issue 3-2-2: Delay spread

Proposals:

- Option 1: RAN4 to further study the Delay spread after UE pre-compensation

Recommended WF:

- RAN4 to further study the Delay spread after UE pre-compensation

| Company | Comments |
| --- | --- |
| Qualcomm | Unlike the legacy static BS, due to the motion of SAN, e.g., in LEO case, both Doppler shift and timing drift could be time-varying. To this end, we encourage companies to explore models or ways to quantify the time-varying Doppler and timing drift. |

Issue 3-2-2: Doppler spread

Proposals:

- Option 1: RAN4 to further study the Doppler shift after UE pre-compensation

Recommended WF:

- RAN4 to further study the Doppler shift after UE pre-compensation

| Company | Comments |
| --- | --- |
| Qualcomm | Unlike the legacy static BS, due to the motion of SAN, e.g., in LEO case, both Doppler shift and timing drift could be time-varying. To this end, we encourage companies to explore models or ways to quantify the time-varying Doppler and timing drift. |

Issue 3-3-1: PDCSH/PDCCH/PBCH requirements

Proposals:

- Option 1: Define new demodulation requirements for PDCSH/PDCCH/PBCH

- Option 2: Define new demodulation requirements for PDCSH

Recommended WF:

- RAN4 to define new requirements for PDSCH

- FFS on whether to define new requirements for PBCH and PDCCH

| Company | Comments |
| --- | --- |

Issue 3-3-2: SCS/CBW set for PDSCH requirements

Proposals:

- Option 1: 15kHz SCS: 10MHz as the starting point

- Option 2: 15kHz SCS: 10MHz, 30kHz SCS: 20MHz

Recommended WF:

- To consider 15kHz SCS, 10MHz. FFS on 30kHz and CBW

| Company | Comments |
| --- | --- |

Issue 3-3-3: Modulation order

Proposals:

- Option 1: consider QPSK and 16QAM for PDSCH

- Option 2: consider QSPK, 16QAM and 64QAM for PDSCH

Recommended WF:

- Consider QPSK and 16QAM for PDSCH. FFS on 64QAM

| Company | Comments |
| --- | --- |

Issue 3-4-1: CSI reporting requirements

Proposals:

- Option 1: Do not consider any CSI reporting requirements

- Option 2: Do not consider PMI and RI reporting requirements, FFS for CQI reporting for LEO

Recommended WF:

- Do not consider PMI and RI reporting requirements, FFS for CQI reporting for LEO

| Company | Comments |
| --- | --- |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general and NTN UE demodulation requirements | Qualcomm Incorporated | Topic#1 and Topic#3 |
| WF on NTN SAN demodulation requirements | Huawei, HiSilicon | Topic#2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200171 | Discussion on Satellite Access Node demodulation requirements | CATT | Noted |  |
| R4-2200475 | Discussion on general issue for NTN NR | Ericsson | Noted |  |
| R4-2200476 | Discussion on satellite access node demodulation requirement for NTN NR | Ericsson | Noted |  |
| R4-2201015 | Discussion on UE NTN demod | Huawei,HiSilicon | Noted |  |
| R4-2201016 | Discussion on satellite NTN demod | Huawei,HiSilicon | Noted |  |
| R4-2201420 | Discussion on UE demodulation for NTN | Ericsson | Noted |  |
| R4-2201785 | Discussion on Satellite Access Node demodulation requirements for NR NTN | Intel Corporation | Noted |  |
| R4-2201786 | Discussion on UE demodulation requirements for NR NTN | Intel Corporation | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203042 | WF on general and NTN UE demodulation requirements | Qualcomm Incorporated | Agreeable |  |
| R4-2203043 | WF on NTN SAN demodulation requirements | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Nicholas, Pu | nicholas.pu@ericsson.com |
| Huawei, HiSilicon | Zehan Zhao | zhaozehan@hisilicon.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)