3GPP TSG-RAN WG4 Meeting # 101-bis-e 											R4-2202304

Electronic Meeting, 17th – 25th January 2022

Agenda item(s):			5.4.1, 5.4.2, 5.4.4 and 5.5.1, 5.5.2, 5.5.4

Source:	Moderator (UIC)

Title:	Email discussion summary for [101-bis-e][104] RAIL_900_1900MHz

Document for:	Information

# Introduction

This e-mail discussion addresses the issues in connection with the provision of 5G NR using the assigned RMR spectrum 900MHz and 1900MHz. The discussion is addressed according to the sub-items of the planned agenda items. Table x  summarizes the contributions submitted to the corresponding agenda items to be elaborated during 1st round and potentially 2nd round discussion:

Table 0-1: List of contributions applicable to thread 104

| RMR | AI | Tdoc | Title | Contributor |
| --- | --- | --- | --- | --- |
| 900 | 5.4.1 | R4-2201687 | Revised TR 38.853 version 0.2.0 | UIC |
| 900 | 5.4.2 | R4-2200702 | On RMR900 sensitivity | Nokia |
| 900 | 5.4.4 | R4-2201688 | TR_38.853_changes_clause 7.1 | UIC |
| 1900 | 5.5.1 | R4-2201328 | RMR 1900 MHz - TP to TR 38.852 on Regulatory background | Ericsson |
| 1900 | 5.5.1 | R4-2201682 | Revised TR 38.852 version 0.2.0 | UIC |
| 1900 | 5.5.2 | None |  |  |
| 1900 | 5.5.4 | R4-2201685 | TR_38.852_changes_clause 7.2 | UIC |
| 1900 | 5.5.4 | R4-2201683 | TR_38.852_changes_clause 7.1 | UIC |

# Topic #1: RMR 900MHz

## Companies’ contributions summary – subject Revision of 3GPP TR 38.853 version 0.2.0 (study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201687 | UIC | Proposal: The revision of 3GPP TR 38.853 v0.1.0 is for approval. Agreed TPs from RAN4#101 have been incorporated. |

## Companies’ contributions summary – subject UE RF parameters (study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200702 | Nokia | Proposal : n100 REFSENS is kept as -100 dBm Note: The proposal is based on the representation of the ratio between duplex gap size and absolute frequency and its resulting figure in the context with the filter design. |

## Companies’ contributions summary – pCR to 3GPP TR 38.853 version 0.2.0(study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201688 | UIC | The contribution proposes changes of the current wording related to clause 7.1. The proposed changes are intended to improve readability and further clarification. |

## Open issues summary

### Sub-topic 1-1 revision 3GPP TR 38.853 v0.2.0

The proposed revisions to clause 7.1 3GPP TR 38.853 is for approval based on RAN4#101-e technical proposals. Please indicate concerns if something is missing.

Issue 1-1:	Please provide comments if necessary.

Proposal(s)

- Option 1: R4-2201687

- Option 2: N/A

- Recommended WF

  - Approve revision on TR 38.853 v0.2.0

### Sub-topic 1-2 UE RF parameters

Issue 1-1:	The contribution addresses the subject of REFSENS applicable for the use by RMR 900. In RAN4#101-e meeting non consensus was reached about the figure of REFSENS. Proposed values in RAN4#101-e were either -97dBm or -100dBm. The present tdoc provides the necessary details to apply REFSENS of -100dBm applicable to RMR900.

Proposal(s)

- Option 1: Approve REFSENS proposal as indicated in R4-2200702

- Option 2: Others

### Sub-topic 1-3 Revisions to clause 7.1 in TR 38.853 v0.2.0

Issue 1-1:	 Version 0.2.0 of TR 38.853 has some issues with the proposed text in clause 7.1. The present tdoc provides revisions of the current text that eliminates such as “it was agreed”. Furthermore it need to be clarified if n1 is adjacent to n100 or it is a typo if n8 is meant here.

Please provide comments if something is missing or requires further necessary alignment.

Proposal(s)

- Option 1: Follow the proposal of R4-2201688 and agree on the proposed changes

- Option 2: Other

NOTE:	Further revisions of the proposed text, if necessary, are welcome.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 1-1 Revision of 3GPP TR 38.853 v0.2.0

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve revision on TR 38.853 v0.2.0 |
| Ericsson | Option 1 |
| UIC | Option 1 but revising the year on the frontpage |
| Moderator | R4-22001687 need to be revised to replace “2021” by “2022”. |
| Moderator 19.01.2022 | With the revision of R4-22001687 (replacing the year) the sub-topic can be closed. |

Sub-topic 1-2 UE RF parameter

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve REFSENS proposal as indicated in R4-2200702 |
| Ericsson | Option 1 |
| UIC | Option 1: R4-2200702 REFSENS was derived accordingly and the differences to band n8 are clearly visible. Approve REFSENS value in R4-2200702 for the use by band n100. |
| Moderator | The corresponding table 7.2.2-1 need to be updated. A corresponding TP need to provide the change. |
| Moderator 19.01.2022 | To the proposed REFSENS value in R4-2200702 applicable for RMR 900 no objections has been received. Therefore, the proposed value for REFSENS in R4-22000702 is agreed. The sub-topic can be closed. |

Sub-topic 1-3 Revisions to clause 7.1 in TR 38.853

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Follow the proposal of R4-2201688 and agree on the proposed changes |
| Ericsson | Actually, this TP is agreeable but there is a mistake in the TR subclause 7.1.1. It’s written “fixed antenna gain value of 18 dBi”, but it should be -17dBi for 900MHz. We would propose to revise this TP to fix this issue as well. |
| UIC | Agree with Ericsson that that the antenna gain applicable for band n100 is “17dB”. In this context it is proposed to change the wording of “fixed antenna gain” to “dedicated antenna gain”. |
| Ericsson | To UIC: not sure what “dedicated” would mean here. Can we change to “maximum antenna gain” instead? |
| Moderator | Revise R4-2201688.. |

## Summary for 1st round

### General

The overview provides the necessary action to be taken in the second round.

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 1-3 R4-2201688 | Update antenna gain value and wording as proposed by Ericsson. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201682 | Update year on the frontpage |
| R4-2201688 | Revise clause 7.1, antenna again and wording accordingly. |

## Discussion on 2nd round (if applicable)

### Sub-topic 1-1 Revisions to clause 7.1 in TR 38.853 v0.2.0

Issue 1-1:	 Version 0.2.0 of TR 38.853 has some issues with the proposed text in clause 7.1. The present tdoc provides revisions of the current text that eliminates such as “it was agreed”. Furthermore it need to be clarified if n1 is adjacent to n100 or it is a typo if n8 is meant here. The draft update revises the issues about the maximum antenna of 17dBi.

Please provide comments if something is missing or requires further necessary alignment.

Proposal(s)

- Option 1: Follow the proposal of “draft R4-220xxxx_was_R4-2201688” and agree on the proposed changes

- Option 2: Other

## Companies views’ collection for 2nd round

### Open issues

Sub-topic 1-1 Revisions to clause 7.1 in TR 38.853

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with the revision |

# Topic #2: RMR 1900

## Companies’ contributions summary – subject Revision of 3GPP TR 38.852 version 0.2.0 (study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201682 | UIC | Proposal: The revision of 3GPP TR 38.852 v0.2.0 is for approval. Agreed TPs from RAN4#101-e have been incorporated. |

## Companies’ contributions summary – subject RMR 1900 regulatory background (study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201328 | Ericsson | The document addresses the regulatory background in the context of RMR1900 to be considered in the definition of the relevant system and RF parameters. |

## Companies’ contributions summary – Proposed revisions to clauses 7.1 and 7.2 in TR 38.852 v0.2.0 (study)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201683 | UIC | The contribution proposes changes of the current wording related to clause 7.1. The proposed changes are intended to improve readability and further clarification. |
| R4-2201685 | UIC | The contribution proposes changes of the current wording related to clause 7.2The proposed changes are intended to improve readability and further clarification. |

## Open issues summary

### Sub-topic 2-1 revision 3GPP TR 38.852 v0.2.0 (study)

Issue 1-1:	The proposed revision of 3GPP TR 38.853 v0.2.0 is for approval based on RAN4#101-e technical proposals. Please indicate concerns if something is missing.

Proposal(s)

- Option 1: Approve R4-2201682;

- Option 2: N/A

- Recommended WF

  - Approve R4-2201682

### Sub-topic 2-2 – subject RMR 1900 regulatory background (study)

Issue 1-1:	Regulatory aspects have been discussed and agreed during RAN4#100-e meeting. This TP addresses now the necessary updates in TR 38.852 considering contribution R4-2201328

Proposal(s)

- Option 1: Consider R4-2201328 and approve R4-2201328

- Option 2: NA

- Recommended WF

  - Approve R4-2201328

### Sub-topic 2-3 Proposed revision to clause 7.1 in TR 38.852 (study)

Issue 1-1:	Version 0.2.0 of TR 38.852 has some issues with the proposed text in clause 7.1. The present tdoc provides revisions of the current text that eliminates such as “it was agreed”.

Please provide comments if something is missing or requires further necessary alignment.

Proposal(s)

- Option 1:Follow the proposal of R4-2201683;

- Option 2: NA

NOTE:	Further revisions of the proposed text, if necessary, are welcome.

### Sub-topic 2-4 Proposed revision to clause 7.2 in TR 38.852 (study)

Issue 1-1:	Some introductory text is missing for clause 7.2 version 0.2.0 of TR 38.852. Contribution R4-2201685 proposes some text.

Proposal(s)

- Option 1:Follow the proposal of R4-2201685;

- Option 2: NA

NOTE:	Further revisions of the proposed text, if necessary, are welcome.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 2-1 3GPP TR 38.852 revision version 0.2.0

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve R4-2201682; |
| Ericsson | Option 1 |
| UIC | Option 1 but revising the year on the frontpage |
| Huawei | Option 1 |
| Moderator | With the revision of R4-22001687 (replacing the year) the sub-topic can be closed. |

Sub-topic 2-2 RMR 1900 regulatory background (study)

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve R4-2201328 |
| UIC | Option 1: To approve R4-2201328 |
| Huawei | Option 1 |
| Moderator | R4-2201328 is approved. The sub-topic can be closed. |

Sub-topic 2-3 Proposed revisions to clause 7.1 in TR 38.852 v0.2.0 (study)

| Company | Comments |
| --- | --- |
| Nokia | Option 1 |
| Ericsson | We would propose the following change to in subclause 7.1.3 (which is a hanging subclause by the way, this should be fixed as well): RAN4 will not specify requirements related to the applicable for CEPT assumption for BS enhanced selectivity considering operating in adjacent band n1 and deployed close by the railway. |
| Moderator | Proposal: ### 7.1.3 Receiver characteristics [Add] 7.1.3.1 General [Change] RAN4 will not specify requirements related to the CEPT assumption for BS enhanced selectivity operating adjacent to band n1 and deployed close by the corresponding railway infrastructure. |
| Ericsson | We are fine with moderator’s proposal. |
| Moderator | Revise R4-2201683. |

Sub-topic 2-4 Proposed revisions to clause 7.2 in TR 38.852 v0.2.0 (study)

| Company | Comments |
| --- | --- |
| Nokia | Option 1:Follow the proposal of R4-2201685; |
| Ericsson | Option 1 |
| Huawei | Could you re-phrase the proposed text to “The UE requirements for RMR 1900 are summarized in the upcoming tables applicable for 3GPP TS 38.101-1 [2]. ” Minor Suggestion: The tdoc title is better to be in the form of “TP on TR 38.852: update section 7.2.1” |
| Moderator | Revise R4-2201685 as proposed by Huawei. |

## Summary for 1st round

### General

The present overview provide the necessary action to be taken in the second round.

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 2-3 | Wording update required. |
| Sub-topic 2-4 | Wording update required. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201683. | Revise R4-2201683 to adapt the wording |
| R4-2201685 | Revise R4-2201685 to adapt the wording. |

## Discussion on 2nd round

### Sub-topic 2-1 Proposed revision to clause 7.1 in TR 38.852 (study)

Issue 1-1:	Version 0.2.0 of TR 38.852 has some issues with the proposed text in clause 7.1. The present tdoc provides revisions of the current text that eliminates such as “it was agreed”. The revision of R4-2201683 fixes the hanging paragraph (7.3.1) and provides the necessary updates for the text in paragraph 7.3.1

Please provide comments if something is missing or requires further necessary alignment.

Proposal(s)

- Option 1:Follow the proposal of draft R4-220xxx_was_R4-2201683;

- Option 2: NA

### Sub-topic 2-2 Proposed revision to clause 7.2 in TR 38.852 (study)

Issue 1-1:	Some introductory text is missing for clause 7.2 version 0.2.0 of TR 38.852. Contribution R4-2201685 proposes some text. The revision of R4-2201685 follows the text proposal from Huawei proposed during 1st round

Proposal(s)

- Option 1:Follow the proposal of draft R4-220xxxx_was_R4-2201685;

- Option 2: NA

NOTE:	Further revisions of the proposed text, if necessary, are welcome.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 2-1 Proposed revisions to clause 7.1 in TR 38.852 v0.2.0 (study)

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with the revision |

Sub-topic 2-2 Proposed revisions to clause 7.2 in TR 38.852 v0.2.0 (study)

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with the revision |

# Recommendations for Tdocs

## 1st round

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2201687 | Revised TR 38.853 version 0.2.0 | UIC | Revised | Revise the year on the frontpage |
| R4-2200702 | On RMR900 sensitivity | Nokia | Agreeable |  |
| R4-2201688 | TR_38.853_changes_clause 7.1 | UIC | Revised |  |
| R4-2201328 | RMR 1900 MHz - TP to TR 38.852 on Regulatory background | Ericsson | Agreeable |  |
| R4-2201682 | Revised TR 38.852 version 0.2.0 | UIC | Revised | Revise the year on the frontpage |
| R4-2201683 | TR_38.852_changes_clause 7.1 | UIC | Revised |  |
| R4-2201685 | TR_38.852_changes_clause 7.2 | UIC | Revised |  |

## 2nd round

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202245 | Revised TR 38.853 version 0.2.0 | UIC | Agreeable | was R4-2201687 |
| R4-2202246 | TR_38.853_changes_clause 7.1 | UIC | Agreeable | was R4-2201688 |
| R4-2202242 | Revised TR 38.852 version 0.2.0 | UIC | Agreeable | was R4-2201682 |
| R4-2202243 | TR_38.852_changes_clause 7.1 | UIC | Agreeable | was R4-2201683 |
| R4-2202244 | TR_38.852_changes_clause 7.2 | UIC | Agreeable | was R4-2201685 |

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| UIC (Union Internationale des Chemins de fer) | Ingo Wendler | ingo.wendler@sbb.ch |
| Ericsson | Dominique Everaere | dominique.everaere@ericsson.com |
| Nokia | Iwajlo Angelow | iwajlo.angelow@NOKIA.COM |
| Huawei | Mohammed Abdi Abyaneh | mohammad.abdi.abyaneh@huawei.com |