TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2117059

Electronic Meeting, November 01-12, 2021

Agenda Item:	10.8.1

Source: 	Puloli, Baicells

Title: 	TP to TR 36.779: Frequency band arrangement

Document for:	Approval

# Introduction

This document provides text proposal for TR 36.779 on frequency band arrangement.

# 2. References

- Reallocation of TV Channels 60-69, the 746-806 MHz Band, ET Docket No. 97-157.

# 3	Text proposal

<<< start of TP>>>

## 5   Frequency band arrangement and regulatory background

Band number [100] was specified for the new FDD E-UTRA NB-IoT only operating band consisting of the 1 + 1 MHz of paired FDD spectrum (UL: 787-788 MHz; DL:757-758); defined as A Block in the Upper 700 MHz Band Plan [1].

The band plan and supported channel bandwidths for Band [100] are shown in Table 5-1 and Table 5-2 respectively.

Table 5-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [100] | 787 MHz | – | 788 MHz | 757 MHz | – | 758 MHz | FDD |

Table 5-2 E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| [100]1 |  |  |  |  |  |  |
| NOTE 1: 1 This band is an NB-IoT only band. None of E-UTRA standard channel bandwidths is applicable. |  |  |  |  |  |  |

Tx-Rx frequency separation is shown in Table 5-3.

Table 5-3: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [100] | 30 MHz |

<<< end of TP>>>