TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2117060

Electronic Meeting, November 01-12, 2021

Agenda Item:	10.8.1

Source: 	Puloli

Title: 	TP to TR 36.779: FCC regulatory requirements

Document for:	Approval

# Introduction

This document provides text proposal for section 5.4 of TR 36.779 on regulatory framework.

# 2. References

- PART 27—MISCELLANEOUS WIRELESS COMMUNICATIONS SERVICES, Electronic Code of Federal Regulations.

# 3.	Text proposal

<<< start of TP>>>

## 5.4 FCC regulation requirement

The FCC rules, relevant to RAN4, governing the Upper 700 MHz Band (746-806) are generally found in the 47 CFR Part 27 [1]. They are summarized as follows:

- Power Limits

The power limits are defined in the §27.50 - Power limits and duty cycle [1].

Fixed and base stations transmitting a signal must not exceed an effective radiated power (ERP) of 1000 watts and an antenna height of 305 m height above average terrain (HAAT), except that antenna heights greater than 305 m HAAT are permitted if power levels are reduced below 1000 watts ERP in accordance with Table 1 below:

| Antenna height meters (feet) | Effective radiated power (ERP) watts |
| --- | --- |
| Above 1372 (4500) | 65 |
| Above 1220 (4000) To 1372 (4500) | 70 |
| Above 1067 (3500) To 1220 (4000) | 75 |
| Above 915 (3000) To 1067 (3500) | 100 |
| Above 763 (2500) To 915 (3000) | 140 |
| Above 610 (2000) To 763 (2500) | 200 |
| Above 458 (1500) To 610 (2000) | 350 |
| Above 305 (1000) To 458 (1500) | 600 |
| Up to 305 (1000) | 1000 |

Control stations and mobile stations transmitting in the 776-788 MHz band and fixed stations transmitting in the 787-788 MHz band are limited to 30 watts ERP.

Portable stations (hand-held devices) transmitting in the band are limited to 3 watts ERP.

For transmissions in the 757-758, 775-776, 787-788, and 805-806 MHz bands, maximum composite transmit power shall be measured over any interval of continuous transmission using instrumentation calibrated in terms of RMS-equivalent voltage. The measurement results shall be properly adjusted for any instrument limitations, such as detector response times, limited resolution bandwidth capability when compared to the emission bandwidth, etc., so as to obtain a true maximum composite measurement for the emission in question over the full bandwidth of the channel.

2. Emission Limits

The emission limits are defined in the §27.53 - Power limits and duty cycle [1]

The power of any emission outside a licensee´s frequency band/bands of operations shall be attenuated below the transmitter power P, measured in watts, by at least 43+10log10(P) dB, considering a resolution bandwidth of 100 kHz or greater, except within 0-100 kHz immediately outside and adjacent a licensee´s frequency block where it is required to be 30 kHz.   That corresponds to a single absolute emission power spectral density limit of -13 dBm (Emission limit = PdBW-43-PdB= - 43dBW = -13dBm) over the specified resolution bandwidth for all transmitters (including base stations and portable devices) regardless of actual ERP or frequency offset.

The emission power for all frequencies between 763-775 MHz and 793-805 MHz shall be attenuated below the transmitter power P, measured in watts, by a factor not less than 76 + 10 log (P) dB in a 6.25 kHz band segment, for base and fixed stations; and by a factor not less than 65 + 10 log (P) dB in a 6.25 kHz band segment, for mobile and portable stations

The emissions in the band 1559-1610 MHz shall be limited to −70 dBW/MHz equivalent isotropically radiated power (EIRP) for wideband signals, and −80 dBW EIRP for discrete emissions of less than 700 Hz bandwidth. For the purpose of equipment authorization, a transmitter shall be tested with an antenna that is representative of the type that will be used with the equipment in normal operation.

<<< end of TP>>>