TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2117062

Electronic Meeting, November 01-12, 2021

Agenda Item:	10.8.1

Source: 	Puloli

Title: 	TP to TR 36.779: nearby non-3GPP services

Document for:	Approval

# Introduction

This document provides text proposal for section 5.2 of TR 36.779 on nearby non-3GPP services.

# 2	Text proposal

<<< start of TP>>>

## 5.2 Nearby non-3GPP services

FCC assigned a pair of 6+6 MHz FDD spectrum for public safety narrow band (PSNB) services. The downlink is in 769 – 775 MHz and uplink is in 799 – 805 MHz; as shown in Figure 1 and Figure 2.

In order to protect the PSNB services, as mentioned in FCC requires that the emission limits to this band be limited to -46 dBm / 6.25 kHz from base stations operated in the Upper 700 MHz A block and -35 dBm / 6.25 kHz from mobile devices; respectively.

<<< end of TP>>>