3GPP TSG-RAN WG4 Meeting # 101-e                                                      R4-2117082

Electronic Meeting, November 01-12, 2021

Source:	CMCC, ZTE Corporation

Title:	TP for TR 37.717-11-11: DC_n28A_34A

Agenda Item:	7.13.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of NR band n28 and LTE band 34 for TR 37.717-11-11.

- Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.5.0

- Text Proposal

----- Start of TP -----

    - 6.1.x	DC_n28_34

      - 6.1.x.1	Configuration for DC

Table 6.1.x.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_34A | DC_n28A_34A | No |

      - 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_34A | 23 | +2/-3 |

      - 6.1.x.3	Spurious emission band UE co-existence for DC

Table 6.1.x.3-1: Spurious emissions for inter-band DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_n28_34 | - UTRA Band 3, 7, 8, 18, 19 ,20, 26, 31, 38, 40, 41, 72 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

      - 6.1.x.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.x.4-1 lists up to 5th order harmonics and intermodulation products for DC_n28A_34A.

Table 6.1.x.4-1: Harmonic and IMD analysis

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 703 | 748 | 2010 | 2025 |
| 2nd order harmonics frequency range | 1406 | 1496 | 4020 | 4050 |
| 3rd order harmonics frequency range | 2109 | 2244 | 6030 | 6075 |
| 4th order harmonics frequency range | 2812 | 2992 | 8040 | 8100 |
| 5th order harmonics frequency range | 3515 | 3740 | 10050 | 10125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1262 | 1322 | 2713 | 2773 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 619 | 514 | 3272 | 3347 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3416 | 3521 | 4723 | 4798 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2644 | 2524 | 5426 | 5546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 84 | 234 | 5282 | 5372 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4119 | 4269 | 6733 | 6778 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7397 | 7292 | 982 | 787 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8743 | 8848 | 4822 | 5017 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4669 | 4534 | 1776 | 1941 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7436 | 7571 | 6129 | 6294 |

As wen can see from table Table 6.1.x.4-1 :

- No harmonic issue for DC_n28A_34A

- 5th intermodulation issue may falls into band n28 own Rx.

      - 6.1.x.5	∆TIB and ∆RIB values

For DC_n28_34, same TIB,c and RIB,c values are given in the tables below.

Table 6.1.x.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n28_34 | n28 | 0.3 |
|  | 34 | 0.3 |

Table 6.1.x.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n28_34 | n28 | 0 |
|  | 34 | 0 |

      - 6.1.x.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen that IMD5 could potentially desensitize band n28 DL carrier. However, after checking the test point, we see there are no suitable test points considering both symmetric UL/DL frequency point and the 5th intermodulation product of |4*fn28_ul-f34|. For example, DL Fc= 760.5MHz when UL Fc = 705.5MHz, however,  |4*fn28_ul-f34|  IMD5 product will not fall into  DL Fc range, which means no MSD are needed to be defined.

      - 6.1.x.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28A_34A.

----- End of TP -----