3GPP TSG-RAN WG4 Meeting # 101-e                                                      R4-2117087

Electronic Meeting, November 01-12, 2021

Source:	CMCC, ZTE Corporation

Title:	TP for TR 37.717-11-11: DC_n28A_40A

Agenda Item:	7.13.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of NR band n28 and LTE band 40 for TR 37.717-11-11.

- Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.5.0

- Text Proposal

----- Start of TP -----

    - 6.1.x	DC_n28_40

      - 6.1.x.1	Configuration for DC

Table 6.1.x.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_40C | DC_n28A_40A | No |

      - 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_40A | 23 | +2/-3 |

      - 6.1.x.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n28A_40C are the same with DC_n28A_40A, which have been already defined in the TS38.101-3 spec.

      - 6.1.x.4	MSD analysis for DC

The MSD analysis for DC_n28A_40C are the same with DC_n28A_40A, where there are no harmonic and IMD falling into own receive bands.

Moreover, there exist 3rd harmonic mixing issue for this combination.

      - 6.1.x.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n28A_40C are the same with DC_n28A_40A, which have already been defined in the TS38.101-3 spec.

      - 6.1.x.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n28A_40C configuration.

Due to the receiver harmonic mixing MSD requirement are defined for the channel bandwidths supported in the affected DL bands, regardless of the CA bandwidth class supported in that band,. Therefore, it is no need to define additional receiver harmonic mixing MSD requirements for the DC_28A_n40C configuration due to the current receiver harmonic mixing MSD  requirements defined in TS38.101-3 are for band 40 UL falls into 3rd harmonic of  band n28, which can apply to band combination of band 40 and band n28 with any supported configurations

      - 6.1.x.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28A_40C.

----- End of TP -----