# 3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2117108

# Electronic Meeting, 1– 12 November, 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.880 | CR | 0001 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 37.880: ACS of PSNB handset receivers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R-4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM-Core |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ACS for PSNB handsets are needed as per the WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds data for PSNB handsets based on the market available PSNB handsets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACS value of the PSNB handsets will be missing from the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<First Changed Section>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

ACLR	Adjacent Channel Leakage Ratio

A-MPR	Additional Maximum Power Reduction

ACS	Adjacent Channel Selectivity

BB	Base Band

BS	Base Station

BW	Bandwidth

CL	Coupling Loss

DL	Downlink

FWA	Fixed Wireless Access

HPUE	High Power User Equipment

IF	Intermediate Frequency

ITU	International Telecommunication Union

LTE	Long Term Evolution

MOP	Maximum Output Power

MPR	Allowed Maximum Power Reduction

MTC	Machine-Type Communications

NB-IoT	Narrowband – Internet of Things

NB	Narrowband

NR	New Radio

OOBE	Out Of Band Emissions

PA	Power Amplifier

PC	Power Class

PRB	Physical Resource Block

PSNB	Public Safety Narrowband

RAN4	Radio Access Network Working Group 4

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SW	Software

TR	Technical Report

Tx	Transmitter

UE	User Equipment

<Next Changed Section>

## 5.3	Coexistence with Public Safety

Satisfactory coexistence between two communication systems in the same geographical area normally depends on two main criteria:

1) The interfering transmitter out of band emission (OOBE) received by the affected receiver is sufficiently below this affected receiver noise floor.

2) The total interfering carrier power attenuated by the affected receiver radio frequency (RF), intermediate frequency (IF), and base band (BB) filters is sufficiently below the affected receiver noise floor.

System simulation is required for coexistence studies on the interference cases involving UE. This is because the UE positions are not fixed, and they are not expected to transmit with full power most of the time due to power control. Hence using deterministic analysis could result in findings that are too conservative, which in turn lead to unnecessary tight coexistence requirements. System simulation need to be performed based on agreed methodology and parameters, so that results from different companies can be compared directly to draw the conclusions from the studies.

The public safety narrowband (PSNB) system parameters used for Band 26 and PSNB coexistence studies in TR 37.806 [14] are shown in table 5.3-1, and some simulation results can be found in clause 7.2 of TR 37.806. These parameters can be considered together with the assumptions provided in clause 5.1 of this TR for the system simulation between HPUE and public safety operation.

Table 5.3-1: PSNB system parameters

|  | Base Station | Portable |
| --- | --- | --- |
| Carrier frequency | 850 MHz |  |
| Channel bandwidth | 6.25 kHz |  |
| Cell radius | 12 km |  |
| Antenna height | 100 m from ground | 1.5m |
| Lognormal fading | 10 dB |  |
| Antenna gain and antenna pattern | 11 dBi omni-directional | Antenna gain + body loss =-6 dBi |
| Noise figure | 5.7 dB | 9.75 dB |
| Transmit power | 45 dBm (after combiner loss) | 36 dBm |
| SNR Threshold | 16.5 dB | 16.5 dB |
| Effective Noise Bandwidth (ENBW) | 6.25 kHz | 6.25 kHz |
| Noise Floor | -130.3dBm / 6.25 kHz | -126.3dBm / 6.25 kHz |
| Sensitivity | -113.8dBm / 6.25 kHz | -109.8dBm / 6.25 kHz |
| ACS |  | 72 dB |

Based on datasheet of “APX™ 8000 ALL-BAND P25 PORTABLE RADIO” the ACS of PSNB handset can be considered as 72dB

Note that the target OOBE from a Band 5 HPUE to the public safety downlink and uplink frequency ranges (for criterion 1), as well as the achievable attenuation on the Band 5 HPUE carrier power at the public safety downlink and uplink receiver filters (for criterion 2), need to be agreed on to evaluate the impact from a Band 5 HPUE on the public safety downlink in 851-861 MHz and the public safety uplink in 806-816 MHz. The simulation methodology and parameters for the system simulation should be further discussed and agreed during the work item (WI) phase.

<End of changes>