3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117129

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Samsung, Ericsson

Title:			TP for TR 37.827 for DC_2-48_n77

Agenda item:		7.32.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include additional DC_2-48_n77 configuration according to the request in [1] and the approved text proposal [2].

Reference

[1] RP-211760: 	Revised WID: Rel-17 Power Class 2 for EN-DC with x LTE bands + y	NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2)

[2] R4-2112681: 	TP for TR 37.827 for DC_2-48_n77

# Text Proposal

<Start of Text Proposal>

## 5.24	DC_2-48_n77

### 5.24.1	Transmitter Characteristics

#### 5.24.1.1	Maximum Output Power

Table 5.24.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.24.1.2	Configurations for EN-DC

Table 5.24.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A_n77A DC_2A-48C_n77A DC_2A-48D_n77A DC_2A-48A_n77C DC_2A-48C_n77C DC_2A-48D_n77C | DC_2A_n77A |

#### 5.24.1.3	Co-existence study

According to the co-existence studies performed, no IMD issue is identified for this configuration.

### 5.24.2	Receiver Characteristics

#### 5.24.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

<End of Text Proposal>