3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117135

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Samsung, Ericsson

Title:			TP for TR 37.827 for DC_5_n5-n77

Agenda item:		7.32.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include additional DC_5_n5-n77 configuration according to the request in [1] and the approved text proposal [2].

Reference

[1] RP-211760: 	Revised WID: Rel-17 Power Class 2 for EN-DC with x LTE bands + y	NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2)

[2] R4-2114940:	TP for TR 37.827 for DC_5_n5-n77

# Text Proposal

<Start of Text Proposal>

## 5.21	DC_5A_n5A-n77A

### 5.21.1	Transmitter Characteristics

#### 5.21.1.1	Maximum Output Power

Table 5.21.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.21.1.2	Configurations for EN-DC

Table 5.21.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n5A-n77A DC_5A_n5A-n77C | DC_5A_n77A |

#### 5.21.1.3	Co-existence study

According to the co-existence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_5A_n77A configuration, IMD4 and IMD5 products fall into the band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference

### 5.21.2	Receiver Characteristics

#### 5.21.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.21.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.21.2.1.1-1 for this dual connectivity configuration.

Table 5.21.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR EN-DC Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n5A-n77A2 DC_5A_n5A-n77C2 | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 20.3 | FDD | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

#### 5.21.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.21.2.1.1-1.

<End of Text Proposal>