3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117140

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Samsung, Ericsson

Title:			TP for TR 37.827 for DC_5-66_n2-n77

Agenda item:		7.32.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include additional DC_5-66_n2-n77 configuration according to the request in [1] and the approved text proposals in [2].

- Reference

[1] RP-211760: 	Revised WID: Rel-17 Power Class 2 for EN-DC with x LTE bands + y	NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2)

[2] TPs for TR 37.827

- R4-2114941: TP for TR 37.827 for DC_5_n2-n77

- R4-2105357: TP for TR 37.827 for DC_66_n2-n77

# Text Proposal

<Start of Text Proposal>

## 5.x	DC_5-66_n2-n77

### 5.x.1	Maximum Output Power

Table 5.x.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.x.2		Configurations for EN-DC

Table 5.x.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n2A-n77A DC_5A-66A-66A_n2A-n77A | DC_5A_n77A DC_66A_n77A |

#### 5.x.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

<End of Text Proposal>