3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117150

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Samsung, Ericsson

Title:			TP for TR 37.827 for DC_13-66_n77

Agenda item:		7.32.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include additional DC_13-66_n77 configuration according to the request in [1] and the approved text proposal [2].

Reference

[1] RP-211760: 	Revised WID: Rel-17 Power Class 2 for EN-DC with x LTE bands + y	NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2)

[2] R4-2105355:	TP for TR 37.827 for DC_13-66_n77

# Text Proposal

<Start of Text Proposal>

## 5.5	DC_13-66_n77

### 5.5.1	Transmitter Characteristics

#### 5.5.1.1	Maximum Output Power

Table 5.5.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.4.1.2	Configurations for EN-DC

Table 5.21.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | DC_13A_n77A DC_66A_n77A |

#### 5.5.1.2	Co-existence study

According to the PC3 DC_13A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 66.

- 3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 13

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_13A-66A_n77A combination.

### 5.5.2	Receiver Characteristics

#### 5.5.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.5.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_13-66_n77 are defined in table 5.5.2.2.1-1.

Table 5.5.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 25.3 | IMD3 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 23.4 | IMD32 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.5.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_13-66_n77 are the same as the Case A defined in table 5.5.2.1.1-1.

<End of Text Proposal>