3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117172

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Nokia, Samsung

Title:			TP for TR 37.717-11-21 for DC_2-13-66_n2-n77, DC_2-13-66-66_n2-n77

Agenda item:		7.18.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_2-13-66_n2-n77 configuration according to the request in [1] and related text proposals [2].

- Reference

[1] RP-211750:	Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

[2] TP for TR 37.717-11-21

- R4-2117167: 	TP for TR 37.717-11-21 for DC_2-13_n2-n77

- R4-2117169: 	TP for TR 37.717-11-21 for DC_2-66_n2-n77

- R4-2016323:   TP for TR 37.717-11-21 to include DC_13-66_n2-n77

# Text Proposal

<Start of Text Proposal>

## 8.X	DC_2-13-66_n2-n77

### 8.X.1	Operating bands for DC

Table 8.X.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-13A-66A_n2A-n77A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.X.2	Configuration for DC

Table 8.x.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A | CA_n2A-n77A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A-66A | CA_n2A-n77A |

### 8.X.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.X.4	∆TIB and ∆RIB values

For DC_2-13-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.X.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77, DC_2-13-66-66_n2-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77 DC_2-13-66-66_n2-n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 8.X.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

<End of Text Proposal>