3GPP TSG-RAN WG4 Meeting #101-e 	R4-2117177 (rev. R4-2016315)

Electronic Meeting, 01st – 12th November2021

Source:	Verizon, LG Electronics, Samsung

Title:	A modified (R42016315) TP for TR 37.717-11-21 to include additional configurations

Agenda item:	7.18.2

Document for:	Approval

# Background

This text proposal is a revision of TP R4-2016315 which was approved for TR 37.717-11-21 to add DC_2_n66-n77 configurations as defined in WID [1].

This proposal introduces an uplink DC_2A_n66A configuration in addition of the approved DC_2A_n77A in [2].

- Reference

[1] RP-211750:	Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

[2] R4-2112735:	TR37.717-11-21-v060

# Text Proposal

##### ---Start of changes---

## 6.49	DC_2_n66-n77

### 6.49.1	Operating bands for DC

Table 6.49.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.49.2	Configuration for DC

Table 6.49.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n66A-n77A DC_2A_n66A-n77C | DC_2A_n66A DC_2A_ n77A | 2A | CA_n66A-n77A |
| DC_2A-2A_n66A-n77A DC_2A-2A_n66A-n77C | DC_2A_n66A DC_2A_ n77A | CA_2A-2A | CA_n66A-n77A |

### 6.49.3	Co-existence studies

Based on co-existence studies, 2nd, 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Based on co-existence studies, 2nd and 4th order IMD generated by dual uplink of 2_n66 may fall into own Rx of band n77

### 6.49.4	∆TIB and ∆RIB values

For DC_2_n66-n77, the TIB,c and RIB,c values are derived from CA_n25-n66-n78 and are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 6.49.5	MSD

As mentioned in 6.49.3, 2nd 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Table 6.49.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | N/A |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |
|  | n77 | 3566 | 10 | 50 | 3566 | 29.4 | IMD2 |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n77 | 3784 | 10 | 50 | 3784 | 8.9 | IMD4 |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

##### ---End of changes---