3GPP TSG-RAN WG4 Meeting # 101-e                                  																		R4-2117197

Electronic Meeting, November 1-12, 2021

Source:	Charter Communications

Title:	TP to TR 38.717.01-01 for NR intra band CA_n96

Agenda item:	7.6.2

Document for:	Approval

# Background

# This contribution provides text proposal on the NR intra band carrier aggregation for CA_n96 as defined in revised WID on NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y). RP-211757 [1].

# Text Proposal

##### ---Start of changes---

# 6	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.x	CA_n96 DL_n96UL

### 6.x.1	Channel bandwidths per operating band for CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink NR CA configurations | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
| CA_n96B | CA_n96B | 20 | 20, 40, 60, 80 |  |  |  | 100 | 0 |
| CA_n96C | CA_n96C | 60, 80 | 40, 60, 80 |  |  |  | 160 | 0 |
| CA_n96D | - | 80 | 80 | 60, 80 |  |  | 240 | 0 |
| CA_n96E | - | 80 | 80 | 80 | 80 |  | 320 | 0 |

### 6.x.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

Considering the receiver design for the higher operation frequency with for the band. And for CA_n96E, there may be gain ripple due to larger aggregation bandwidth configuration than ever, we suggest to allow [0.5]dB REFSENS relaxation as below:

Table 6.x.3-1: ΔRIB,c due to CA

| Inter-band CA combination | Operating Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n96E | n96 | 0.5 |

“Text Omitted”

# 6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.x	CA_2DL_n96(2A)_1UL_n96A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_n96(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(2A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 |  |  | 160 | 0 |

### 6.x.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.x	CA_2DL_n96(3A)_1UL_n96A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_n96(3A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(3A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 |  | 240 | 0 |

### 6.x.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.x	CA_2DL_n96(4A)_1UL_n96A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_n96(4A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(4A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 320 | 0 |

### 6.x.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

The DL NCCA with four carriers CA_n96(4A) was first required in RAN4. Considering design complexity of four non-contiguous carriers, REFSENS relaxation may be needed. One possible implantation is split receiver architecture to cover non-contiguous carriers within the whole pass band. Here we propose ΔRIBNC as below:

Table 6.x.3-1: Intra-band non-contiguous CA reference sensitivity for shared spectrum

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- |
| CA_n96(4A) | 15/30/60 | 320MHz | 0.5 | TDD |

##### ---End of changes---

# Reference

[1]	RP-211757, “Revised WID on Rel-17 revised WID on “NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)” Ericson Corporation