3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117250

Electronic Meeting, 1 – 12 November 2021

Agenda Item:	10.9.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS RF requirements for support of 16QAM in NB-IoT

Document for:	Approval

# 1.	Introduction

The work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e [1]. One of the objectives of this work item is to support 16QAM in NB-IoT unicast in UL and DL. BS RF requirements for support of 16QAM in NB-IoT were further discussed at TSG RAN4#101-e and a way forward was approved [2].

This contribution provides further proposals on BS RF requirements for the support of 16QAM in NB-IoT unicast in UL and DL according to the approved WF [2] and the agreements in RAN1 [3].

# 2.	Discussion

2.1	NB-IoT RB power dynamic range for in-band or guard band operation or NB-IoT operation in NR in-band

The following point was stated in [2]:

- Reuse the existing NB-IoT RB power dynamic range requirements for 16QAM for in-band or guard band operation or NB-IoT operation in NR in-band. And no changes to the specifications, nor new test cases for 16QAM RB power dynamic range are needed.

Hence no further discussion is required on this topic.

2.2	Error Vector Magnitude

The following point was stated in [2]:

- The majority view is to set the EVM limit for 16QAM to 12.5%. FFS whether to allow different rated output power for 16QAM.

Currently, different rated total output powers may be declared for BS configured for E-UTRA 256QAM downlink operation [4], i.e., not even E-UTRA 64QAM downlink operation is allowed different rated output power, hence we do not see a need to allow back-off for NB-IoT 16QAM support. Moreover, allowing different rated output power for NB-IoT 16QAM downlink operation will create different downlink link budget for NB-IoT QPSK and 16QAM operation, and thus make cell layout design more complicate. Therefore, we propose the same rated output power as declared for QPSK should be used for 16QAM, i.e., not to allow different rated output power for 16QAM.

Proposal 1: The same rated output power as declared for QPSK should be used for 16QAM, i.e., not to allow different rated output power for 16QAM.

2.3	Fixed Reference Channels for Receiver Dynamic Range

The following was stated in [2]:

- FFS whether to define new 16QAM FRCs for BS Rx dynamic range tests

For E-UTRA, uplink FRCs using 16QAM are used for the receiver dynamic range requirement [5] because the higher required SNR comparing to QPSK would present a more stringent condition for the receiver dynamic range. Moreover, as recorded in TR 36.804 [6], for E-UTRA this requirement measures the effects of receiver impairments such as receiver EVM, and in order to make the test sensitive to receiver EVM impairments, the wanted signal is a 16QAM signal. Therefore, new uplink FRC using 16QAM would need to be defined for receiver dynamic range requirement of NB-IoT BS supporting 16QAM to ensure satisfactory BS performance. Moreover, new uplink NB-IoT FRC using 16QAM will need to be defined for receiver demodulation performance requirement. We consider it beneficial in term of test set-up to use the same uplink NB-IoT FRC using 16QAM for both receiver dynamic range and receiver demodulation performance testing, especially for receiver dynamic range test with NB-IoT in-band or guard band operation where 16QAM can be used for both E-UTRA and NB-IoT operation in the test.

The UL TBS agreed in RAN1 for 16QAM in NB-IoT uplink is provided in table 1 below.

Table 1: TBS indices are introduced for 16QAM in NB-IoT uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

It has also been agreed in RAN1 that repetition is not used for 16QAM in NB-IoT uplink, where it can be used for 3, 6 and 12 subcarriers NPUSCH format 1. Note that 16QAM cannot be used for 1 subcarrier NPUSCH format 1. Considering the agreements in RAN1, and to ensure satisfactory BS receiver performance with 3, 6 and 12 subcarriers for 16QAM in NB-IoT uplink, it is proposed to define at least one of the new uplink FRCs for 16QAM as shown in table 2 below.

Table 2: FRC parameters for 16QAM in NB-IoT uplink

| Reference channel | Ax-y | Ax-y | Ax-y |
| --- | --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 15 | 15 |
| Number of tones | 3 | 6 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM |
| Diversity | No | No | No |
| Frequency offset | 0 | 0 | 0 |
| I_TBS / I_RU | 19 / 3 | 19 / 3 | 19 / 3 |
| Payload size (bits) | 1736 | 1736 | 1736 |
| Allocated resource units | 4 | 4 | 4 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Coding rate (target) | 3/4 | 3/4 | 3/4 |
| Coding Rate | 0.76 | 0.76 | 0.76 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks – C | 1 | 1 | 1 |
| Total symbols per resource unit | 144 | 144 | 144 |
| Total number of bits per resource unit | 576 | 576 | 576 |
| Tx time (ms) | 16 | 8 | 4 |
| Frequency offset | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 12 | 8 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |

Proposal 2: To define at least one of the new uplink FRCs for 16QAM as shown in table 2 above.

Preliminary link-level simulation has been performed in AWGN channel using the FRC parameters in table 2 above, and the simulation results for the SNIR required at 70% and 95% of maximum throughput are provided in Figure 1 below. It can be seen from the figure that the required SNIR are quite close for the different number of tones.

(a) 3 tones

(b) 6 tones

(c) 12 tones

Figure 1: SNIR required at 70% and 95% of maximum throughput for 16QAM in NB-IoT uplink

# 3.	Conclusion

This contribution has provided further proposals on BS RF requirements for the support of 16QAM in NB-IoT unicast in UL and DL according to the approved WF and the agreements in RAN1. They are summarized as follows.

Proposal 1: The same rated output power as declared for QPSK should be used for 16QAM, i.e., not to allow different rated output power for 16QAM.

Proposal 2: To define at least one of the new uplink FRCs for 16QAM as shown in table 2 above.

# References

[1]	RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon.

[2]	R4-2114943, “WF on BS and UE RF requirements in support of NB-IoT 16QAM”, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson.

[3]	RP-210534, “Status report for WI: Additional enhancements for NB-IoT and LTE-MTC”, Huawei.

[4]	3GPP TS 36.141: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[5]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 36.804: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.