3GPP TSG-RAN WG4 Meeting #101-e	R4-2117255

Electronic Meeting, 1st – 12th November 2021

Source:	AT&T

Title:	TP for TR 38.842 Addition of CA_n5-n12-n77

Agenda item:			7.31.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.842 to add CA_n5-n12-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n5-n12-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n12A-n77A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.X.2	Maximum output power

Table 5.X.2-1 UE Power Class 2 for One UL Carrier

| CA power class | Carrier n5 power class | Carrier n12 power class | Carrier n77 power class |
| --- | --- | --- | --- |
| 26dBm | 23dBm | 23dBm | 26dBm |

### 5.X.3	REFSENS requirements

MSD requirements for single uplink PC2 are captured in lower order combinations.

---Skipping to Dual Uplink Carrier---

## 6.X	CA_n5-n12-n77

### 6.X.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n12A-n77A | CA_n5A-n77A, CA_n12A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n5-n77 | CA power class | Carrier n5 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied.

Table 6.x.2-2 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n12-n77 | CA power class | Carrier n12 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n12A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also, when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 6.X.3	REFSENS requirements

According to the PC3 CA_n5A-n12A-n77A study captured in TR 38.717-03-02, the following co-existence issues due to dual uplink were identified:

-	IMD5 products are produced by Band n5 and n12 that might fall in Rx of band n77.

-	IMD5 products are produced by Band n5 and n77 that might fall in Rx of band n12.

-	IMD5 products are produced by Band n12 and n77 might fall in Rx of band n5.

For the IMD products produced by Band n5 and n12 UL, the MSD values of the corresponding PC3 CA case can be applied since this uplink configuration does not support PC2.

#### 6.X.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n5A-n12A-n77A are defined in Table 6.X.3.1-1.

Table 6.X.3.1-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n12A-n77A | n5 | 835 | 5 | 25 | 880 | 11.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 12.7 | FDD | IMD5zz |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

#### 6.X.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n5A-n12A-n77A are same as the Case A defined in Table 6.X.3.1-1.

---End of changes---

# References