3GPP TSG-RAN WG4 Meeting #101-e	R4-2117296

Electronic Meeting, Nov. 1 – Nov. 12, 2021

Agenda Item:	8.4.3.1

Source:	Apple

Title:	UL gaps for Tx power management

Document for:	Discussion

# 1	Introduction

In RAN#92e, revised WID on NR RF enhancements for FR2 is approved [1]. The purpose of this WI is to specify related FR2 UE features and associated requirements, including

- UL gaps for self-calibration and monitoring: [RAN4 RF/RRM, RAN2] Study and, if feasible, introduce UE specific and NW configured gap for general self-calibration and monitoring purposes including

    - UE Tx power management

    - Other self-calibration and monitoring are not precluded

    - Coherent uplink MIMO

  - Phase 1: Study and clearly identify the performance gain over the current baseline (Rel.16 requirements) Study of RF performance evaluation/testability related to UE self-calibration and monitoring. Study network impact of UE emissions during UL gap, if any.

  - Phase 2: Specify the UL gap configuration(s), related UE capability and interruptions, if needed, based on the identified performance gain in Phase 1 and UE fall back behavior i.e. if gaps are not available for UE requesting gaps.

Agreements in 100-e on RF aspect of the UL gap for Tx power management are captured in the way forward [2].

Agreement:

- Baseline is to verify that UE correctly behave without phantom and ensure the feasible requirement gain in Rel-17 with different test methods.

Agreement:

    - “P-MPR report+peak EIRP without phantom”, X dB EIRP gain and P-MPR requirement of Y when UL gap is activated should be achieved compared to the case where no gap is activated

    - Decide range for X value in this meeting for making decision in future meeting

      - Option 1: at least 6dB

      - Option 2: A value between 6dB and 3dB, which is typical in the field

    - Further discussion on the definition of Y in this meeting

      - Option 1: Y is absolute value

      - Option 2: Y is the relative value of gain

      - Option 3: no P-MPR requirement of Y

    - FFS on the implementation margin

In this paper, we further discuss RF aspects related to the UL gap configurations.

# 2	UL Tx power management

To meet MPE requirement, UE needs to perform Tx power back off if a target is detected within proximity of the antenna panel. With proximity sensor (PS) based human target position estimation, the corresponding P-MPR and/or operating duty cycle values can be determined and applied only when required, i.e. in situations where RF exposure caused to human targets can exceed the regulatory limits. One of the key objectives of Phase II work is to specify the related requirement and test cases, to ensure the performance gains are obtained from the introduction of UL gaps for proximity sensing.

2.1 Test metric

It has been agreed in RAN4#100e that both peak EIRP and P-MPR report without phantom will be used as test metric. The peak EIRP test metric is defined as the X dB EIRP gain that can be achieved when UL gap is activated against the case where no UL gap is activated. The P-MPR reporting metric is still under discussion. One option is to use the relative P-MPR difference between UL gap is activated versus the case where it is de-activated. Another option is to use the absolute P-MPR when UL gap is activated. Since the relative P-MPR difference is a duplication of relative peak EIRP with less accuracy, we do not see the value of using P-MPR difference in addition to the relative peak EIRP test metric. Therefore, the absolute P-MPR valuet when UL gap is activated is recommended as a relevant test metric.

Proposal 1: Absolute P-MPR report value when UL gap is activated can be used as a for requirement.

2.2 Test case

To simplify the discussion, minimum peak EIRP test cases defined in [4] is used as the starting point, where DFT-s-OFDM waveform with QPSK, inner full RB allocation is used, and there is no MPR. During the test, UE shall apply P-MPRf,c for carrier f of serving cell c, to meet the MPE requirement throughout the test. All UEs tested are expected to ensure MPE compliance as per FCC/ICNIRP requirements, even though the related MPE compliance test is out of 3GPP scope.

Test Procedure

The example test case using peak EIRP and P-MPR is shown in Fig. 1. Before T1 starts, the peak EIRP direction is found and locked, as per the minimum peak EIRP test case defined in [5]. During the test, average EIRP is separately calculated for T1 and T2 phases, and then compared. During T1 and T2 phases, a test function can be designed to allow non-zero P-MPR.

- During T1 phase, UL gap is configured and activated. Since UE can determine there is no object in proximity with UL gap, RF exposure (measured in terms of MPE) is not a concern and therefore P-MPR should be significantly reduced. Consequently, higher EIRP is expected.

- In the T2 phase, UL gap is deactivated. During T2, UE cannot determine the absence of an object in close proximity without UL gap. To ensure the potential RF exposure caused complies with regulatory limits, P-MPR is determined based on the conservative assumption that an object is always present. The Tx power is then backed off by an amount equal to P-MPR, thereby causing the measured EIRP to be less than the one in T1 phase.

- The difference of average EIRP during T1 and T2 can be test metric.

- The P-MPR computed in T1 phase can be used as a test metric.

Figure 1. Example test procedure with delta-EIRP as metric

Measurement Time

In peak EIRP test, the measurement duration is at least 1ms/1sf, using the reference measurement channels for TDD defined in the table in 38.521-2 Annex A.2.3. For the u=3 (SCS = 120 kHz) case the TDD slot pattern is 7D1S8U, the measurement is averaged over the 8 UL slots, which is exactly 1ms.

In UL gap test, the measurement time requires further discussion and should take into consideration different ULDL configurations and BPS detection false alarm rate. In the example shown in Fig. 2, the transmitted Tx power is plotted during the course of an UE transmission. As can be seen in the figure, when UL gap is activated, the proximity sensor senses nearby environment every 40ms. If no target is detected, the UE can transmit with high power, say X1dB. If the proximity sensor determines there is a target due to false alarm, then the UE will transmit at lower power, say X2dB. Therefore, when UL gap is activated measured EIRP should be averaged long enough to take into account the false alarm effect. We propose the EIRP is averaged over multiples of UL gap periodicity value (equal to 40 ms), such as 4s, to get a stable result.

Figure 2. Example Tx power output fluctuation due to false alarm

Proposal 2: When UL gap is activated, the peak EIRP measurement should be averaged across UL slots with PUSCH transmission over 4s.

2.3 Performance requirement

Requirement definition:

The EIRP gain obtained when UL gap is activated compared to the case when it is de-activated, depends on the UE max peak EIRP, UL duty cycle, UL waveform, constellation, and RB allocation etc. For identify the gain requirement, it is desirable to separate MPR by setting MPR = 0dB. This can be achieved by choosing DFT-s-OFDM waveform with inner full RB allocation and QPSK modulation. It was agreed not to use phantom for testing and therefore the gain requirement also shall be done Vaindependent of it.

The requirement of peak EIRP gain due to UL gap can be defined as

PgapOn - PgapOff > = (Ppeak_EIRP -21dBm-margin) + 10*log10(Z/20),                       (1)

where

      - PgapOn: measured peak EIRP when UL gap is ON over the measurement duration.

      - PgapOff: measured peak EIRP when UL gap is OFF.

      - Ppeak_EIRP is the measured peak EIRP when P-MPR = 0. This can be obtained during min peak EIRP test.

      - Value 21dBm corresponds to the Tx power value at which the FCC MPE limit of 0 dBm/cm2 can be met with P-MPR=0 and 20% duty cycle. This value was arrived at based on far field MPE calculation reported in R4-2014218.

      - Margin: implementation margin including, detection probability, false alarm and other RF impairments should be considered

      - Z is % UL duty cycle (in Eq. (1), we assumed 20% as the base line for UL duty cycle).

Equation (1) is proposed to accommodate different implementations, which have different Ppeak_EIRP and UL duty cycle values.

Here are some example requirements with different UE peak EIRP, duty cycle configuration:

      - Ppeak_EIRP = 26dBm, 20% UL duty cycle, margin = 2dB, PgapOn - PgapOff >= 3dB

      - Ppeak_EIRP = 28dBm, 20% UL duty cycle, margin = 2dB, PgapOn - PgapOff >= 5dB

      - Ppeak_EIRP = 26dBm, 40% UL duty cycle, margin = 2dB, PgapOn - PgapOff >= 6dB

Proposal 3: The minimum Delta EIRP gain is (Ppeak_EIRP-21dBm-margin) + 10*log10(Z/20) comparing UL gap is activated to when the UL gap is not activated.

For absolute P-MPR report, it is defined in 38.321, the P-MPR reporting is controlled by the “P” field and the “P-MPR” field.

The P-MPR report is 2 bits and defined in current 38.133, and the field mapping is defined as

Based on the current design, the absolute P-MPR when UL gap is configured should be in the range of 0 to 3dB. I.e, the P field in the PHR shall set to 0.

Proposal 4: The absolute P-MPR when UL gap is activated should be between 0 to 3dB.

Implementation margin with impact of miss detection and false alarm rate (FAR)

Implementation margins need to be taken into consideration such as RF impairments, Tx power variations due to temperature variation, false alarm and miss detection rate etc. Here, we provide numerical analysis based on the PS’s detection probability and false alarm probability values. Other factors need to be included in the margin value of Eq. (1).

To determine the Tx power gain with different detection and false alarm probability values, we perform an analysis to quantify the impact. As shown in Fig. 2, when BPS does not detect any target, higher Tx power X1 is transmitted. When BPS detects the target, lower Tx power X2 is used.

To analyze impact of miss detection, we assume target is in UE’s close proximity. As shown in Fig. 3, when there is a miss detection, higher Tx power will be used. The red reference line is the maximum Tx power level at which the FCC MPE limit can be ensured, with 100% detection. In practice since miss detections can happen, to always ensure FCC compliance, we need to back off the Tx power to a lower value, equal to X2 dBm in Fig. 3. If we denote detection probability as a, then the transmitted power over the measurement duration is calculated as

a*X2 + (1-a) * X1                       (2)

The total transmitted should be lower than FCC requirement.

Figure 3. Example Tx power output fluctuation due to false

False alarm can further reduce the measured Tx power when UL gap is ON, as observed in Fig. 3. Denote false alarm probability as b, then the impact of false alarm probability can be calculated as

X1*(1-b) + X2*b         (3)

Table I shows the Tx power gain = (X1-X2), with 1 dB and 1.5 dB back off values from the Tx power limit identified to ensure MPE compliance based on FCC requirements, computed based on equations (2) and (3). As shown in the table, the Tx power gain heavily depends on the detection probability and false alarm rate.

Table I: Summary of Tx power gain with different detection probability and false alarm rate

| Backoff value comparing to FCC requirement | Detection Prob | Tx power gain X1 – X2 (dB) | Achievable Tx power gain with BPS operation |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 10% FAR | 15% FAR | 20% FAR |
| 1dB | 75% | 3.0871 | 2.8604 | 2.7424 | 2.6211 |
| 1dB | 80% | 3.6071 | 3.3549 | 3.2231 | 3.0871 |
| 1dB | 85% | 4.3555 | 4.0714 | 3.9221 | 3.7675 |
| 1dB | 90% | 5.5500 | 5.2249 | 5.0527 | 4.8734 |
| 1.5dB | 75% | 4.2327 | 3.9535 | 3.8069 | 3.6551 |
| 1.5dB | 80% | 4.8610 | 4.5582 | 4.3985 | 4.2327 |
| 1.5dB | 85% | 5.7406 | 5.4098 | 5.2345 | 5.0518 |
| 1.5dB | 90% | 7.0973 | 6.7328 | 6.5385 | 6.3350 |

When there is no phantom in the test, margin in Eq. (1) can be calculated as the delta between UE’s peak EIRP with zero PMPR and the actual peak EIRP with UL gap activated, which represents the power gain reduction due to the aforementioned impairments. The margin is calculated using the X1 and X2 value in Table I. Table II shows the margin with peak EIRP of 25dBm, Table III shows the margin with peak EIRP of 26dBm. Table IV shows the margin with peak EIRP of 27dBm. Table V shows the margin with peak EIRP of 28dBm. The highlighted rows are considered as reasonable implementation choice of different combination.

Table II: Calculation of margin for peak EIRP of 25dBm

| Backoff value comparing to FCC requirement | Detection Prob | Margin |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10% FAR | 15% FAR | 20% FAR |
| 1dB | 75% | 2.1396 | 2.2576 | 2.3789 |
| 1dB | 80% | 1.6451 | 1.7769 | 1.9129 |
| 1dB | 85% | 0.9286 | 1.0779 | 1.2325 |
| 1dB | 90% | 0 | 0 | 0.1266 |
| 1.5dB | 75% | 1.5465 | 1.6931 | 1.8449 |
| 1.5dB | 80% | 0.9418 | 1.1015 | 1.2673 |
| 1.5dB | 85% | 0.0902 | 0.2655 | 0.4482 |
| 1.5dB | 90% | 0 | 0 | 0 |

Table III: Calculation of margin for peak EIRP of 26dBm

| Backoff value comparing to FCC requirement | Detection Prob | Margin |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10% FAR | 15% FAR | 20% FAR |
| 1dB | 75% | 3.1396 | 3.2576 | 3.3789 |
| 1dB | 80% | 2.6451 | 2.7769 | 2.9129 |
| 1dB | 85% | 1.9286 | 2.0779 | 2.2325 |
| 1dB | 90% | 0.7751 | 0.9473 | 1.1266 |
| 1.5dB | 75% | 2.5465 | 2.6931 | 2.8449 |
| 1.5dB | 80% | 1.9418 | 2.1015 | 2.2673 |
| 1.5dB | 85% | 1.0902 | 1.2655 | 1.4482 |
| 1.5dB | 90% | 0 | 0 | 0.1650 |

Table IV: Calculation of margin for peak EIRP of 27dBm

| Backoff value comparing to FCC requirement | Detection Prob | Margin |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10% FAR | 15% FAR | 20% FAR |
| 1dB | 75% | 4.1396 | 4.2576 | 4.3789 |
| 1dB | 80% | 3.6451 | 3.7769 | 3.9129 |
| 1dB | 85% | 2.9286 | 3.0779 | 3.2325 |
| 1dB | 90% | 1.7751 | 1.9473 | 2.1266 |
| 1.5dB | 75% | 3.5465 | 3.6931 | 3.8449 |
| 1.5dB | 80% | 2.9418 | 3.1015 | 3.2673 |
| 1.5dB | 85% | 2.0902 | 2.2655 | 2.4482 |
| 1.5dB | 90% | 0.7672 | 0.9615 | 1.1650 |

Table IV: Calculation of margin for peak EIRP of 28dBm

| Backoff value comparing to FCC requirement | Detection Prob | Margin |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10% FAR | 15% FAR | 20% FAR |
| 1dB | 75% | 5.1396 | 5.2576 | 5.3789 |
| 1dB | 80% | 4.6451 | 4.7769 | 4.9129 |
| 1dB | 85% | 3.9286 | 4.0779 | 4.2325 |
| 1dB | 90% | 2.7751 | 2.9473 | 3.1266 |
| 1.5dB | 75% | 4.5465 | 4.6931 | 4.8449 |
| 1.5dB | 80% | 3.9418 | 4.1015 | 4.2673 |
| 1.5dB | 85% | 3.0902 | 3.2655 | 3.4482 |
| 1.5dB | 90% | 1.7672 | 1.9615 | 2.1650 |

We observe that with reasonable detection error and false alarm rate assumed, the margin is distributed between 1.5 and 2.5dB. Also, false alarm in general has less impact compared to detection probability. Meanwhile, UE with higher peak EIRP requires more accurate detector in order to fully benefit from the higher transmit power. Even though the margin depends on many UE implementation choices including peak EIRP and target detection error/false alarm rates, it is still desirable to define a single set requirement to accommodate all different implementations. It is up to UE to carefully select the operation range to meet the requirement.

Note that in the above tables, the effects due to RF impairments, Tx power variations due to temperature variation, which can introduce additional degradation, are not considered yet. Overall, we propose and implementation margin of 2.5dB to account for RF impairments, Tx power variations due to temperature variation, false alarm and miss detection rate etc.

Observation: Higher peak EIRP requires better detection probability.

Proposal 5: Implementation margin of 2.5dB is allowed.

# 3	Summary

This contribution has provided our views on UL gap test methodology and metric. Our proposals are as follows:

Proposal 1: Absolute P-MPR report value when UL gap is activated can be used as the metric for requirement.

Proposal 2: When UL gap is activated, the peak EIRP measurement should be averaged across UL slots with PUSCH transmission over 4s.

Proposal 3: The minimum Delta EIRP gain is (Ppeak_EIRP-21dBm-margin) + 10*log10(Z/20) comparing UL gap is activated to when the UL gap is not activated.

Proposal 4: The absolute P-MPR when UL gap is activated should be between 0 to 3dB.

Proposal 5: Implementation margin of 2.5dB is allowed.

Observation: Higher peak EIRP requires better detection probability.

# References

- RP-211538, “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, Nokia Shanghai Bell, June 14-18, 2021

- R4-2114946, “WF on FR2 enhancement part 2: UL gap”, Apple, Aug 16-27, 2021

- 3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

- 3GPP TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”