3GPP TSG-RAN WG4 Meeting #101-e	R4-2117302

Electronic Meeting, Nov. 1 – Nov. 12, 2021

Agenda Item:	8.9.4.4

Source:	Apple

Title:	Discussion on the timing requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 100-e.

In this paper, we discuss the timing requirement for FR2 HST.

# 2	Discussion

Large propagation delay issue has been identified in the deployment scenario discussion, particularly for uni-directional deployment.

Uplink timing

- RAN4 will further study the below options to address uplink timing issues

  - Option 1: One shot UE autonomous large uplink timing adjustment

  - Option 2: Other implementation/deployment based solution

One-time large TA adjustment is an effective solution to handle the large propagation delay particularly for uni-directional network. When TCI state switching happens from one RRH to another RRH in uni-directional SFN deployment, large TA adjustment is expected. For example, in uni-directional deployment, with DPS transmission scenario, when UE is switching from TCI state associated with SSB index 5 to TCI state associated with SSB index 6, the propagation delay can be increase by more than 2us (more than 60Ts) with Ds=700m. In this case one time large TA adjustment is needed based on DL SSB received timing change. However, when UE moved from TCI state associated with SSB 4 to SSB5, no one time large TA is needed. To assist UE performs one time TA adjustment, network can signal the SSB index per RRH and uni-directional deployment to UE.

Fig.1 One-time large TA when UE switch between RRH

Proposal 1: One-time large TA adjustment can be enabled when switching between RRH for uni-directional deployment.

Proposal 2: Network signaling of the SSB index and order per RRH and whether this is uni-directional or bi-directional deployment, to assist UE one time TA adjustment.

For L1-RSRP measurement, the large propagation delay can be greater than 2us, which is roughly 3.5CP length in 120KHz numerology, the OFDM symbol are not aligned between different TCI state from different RRH. In this case, in order to allow the UE to perform L1-RSRP of the next TCI state measurement while receiving/transmitting on current TCI state, a scheduling restriction needs to be defined as one OFDM symbol before and after the SSB/CSI-RS symbol to be measured.

Proposal 3: Define scheduling restriction for SSB/CSI-RS based L1-RSRP measurement. UE is not expected to transmit or receive one symbol before and one symbol after each consecutive SSB symbols to be measured.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1: One-time large TA adjustment can be enabled when switching between RRH for uni-directional deployment.

Proposal 2: Network signaling of the SSB index per RRH and whether this is uni-directional or bi-directional deployment, to assist UE one time TA adjustment.

Proposal 3: Define scheduling restriction for SSB/CSI-RS based L1-RSRP measurement. UE is not expected to transmit or receive one symbol before and one symbol after each consecutive SSB symbols to be measured.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2115334, Way forward for FR2 HST RRM (part 1), Nokia, Aug 2021

- R4-2115335, Way forward for FR2 HST RRM part 2, Samsung, Aug 2021