3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2117308

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Qualcomm

Title:	Request adding support NR band n77/n78 with UL-MIMO for PC1.5 UPUE

Agenda item:		7.37.1

Document for:		Approval

# Introduction

This contribution provides our request to the RAN4 Rel-17 basket work item [1] to support NR band n77/n78 UL MIMO for PC1.5 HPUE.

Reference

[1] RP-211760: WID revision: Additional NR bands for UL-MIMO in Rel-17

# Request overview

For adding support for NR band n77/n78 with UL-MIMO for PC1.5 UE in Rel-17, a draft text proposal is included in below for updating the section 6.2D of 38.101-1 for UE power class PC1.5 for UL MIMO.

<Start of text proposal>

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 23 | +2/-3 |  |  |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  | 23 | +2/-3 |  |  |
| n77 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n80 |  |  |  |  | 23 | +2/-31 |  |  |
| n84 |  |  |  |  | 23 | +2/-3 |  |  |
| n95 |  |  |  |  | 23 | +2/-3 |  |  |
| n97 |  |  |  |  | 23 | +2/-3 |  |  |
| n98 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

<End of text proposal>

In addition, below is an update of tracking information for NR band n77 power class 1.5 band combination and for inputting to the Rel-17 basket work item [1].

| EN-DC configuration | Uplink EN-DC configuration | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | CA_n2A-n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com |  | Completed for PC3, Completed for PC2 |
| CA_n5A-n77A | CA_n5A-n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com |  | Completed for PC3, Completed for PC2 |
| CA_n13A-n77A | CA_n13A-n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com |  | Completed for PC3, Completed for PC2 |
| CA_n48A-n77A | CA_n48A-n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com |  | Completed for PC3, Completed for PC2 |
| CA_n66A-n77A | CA_n66A-n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com |  | Completed for PC3, Completed for PC2 |