3GPP TSG- RAN WG4 Meeting #101-e	R4-2117326

# Electronic Meeting, Nov. 1-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for SRS antenna port switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements for SRS antenna port switching should be introduced in RRM specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the requirements for SRS antenna port switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for SRS antenna port switching will be not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, new 8.2.1.2.18, 8.2.2.1, new 8.2.2.2.16, 8.2.3.1, new 8.2.3.2.16, 8.2.4.1, new 8.2.4.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

8.2.1	EN-DC Interruption

8.2.1.1	Introduction

This clause contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell(s) in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

UL/DL BWP is switched on PSCell or SCell in SCG, or

UE-specific CBW is changed on PSCell or SCell in SCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

E-UTRA SRS carrier based switching, or

UE dynamic Tx switches between two uplink carriers, or

NR SRS transmission with antenna port switching.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This clause contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

========================= Second Change Request ===========================

8.2.1.2.18	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a carrier. When a UE needs to transmit SRS with antenna port switching on a carrier of a serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [20].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

When the signal of cell UE transmt SRS with antenna port switching on is synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X1 slot as specified in Table 8.2.1.2.18-1.

When the signal of cell UE transmt SRS with antenna port switching on is not synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X2 slot as specified in Table 8.2.1.2.18-2.

Table 8.2.1.2.18-1: Interruption length X1 (slot) for synchronized signal of cells

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15 kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 1 | 1 |
| 60 | 2 | 1 |
| 120 | 3 | 1 |

Table 8.2.1.2.18-2: Interruption length X2 (slot) for asynchronized network

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 2 | 2 |
| 60 | 3 | 2 |
| 120 | 4 | 2 |

========================= Third Change Request ===========================

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

8.2.2.1	Introduction

This clause contains the requirements related to the interruptions on PCell and activated SCell if configured, when

up to 7 SCells are configured, de-configured, activated or deactivated, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell or SCell, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

UE-specific CBW is changed on PCell or SCell, or

NR SRS carrier based switching, or

UE dynamic Tx switches between two uplink carriers, or

NR SRS transmission with antenna port switching.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure according to TS38.331 [2] for SCell addition/release or MAC control signalling according to TS37.340 [17] for SCell activation/deactivation command.

This clause additionally contains requirements related to interruptions at inter-frequency SFTD between PCell in FR1 and neighbour cell in FR2.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

In addition to standalone NR carrier aggregation when no CCA is configured, the requirements in clause 8.2.2. and all subclauses of 8.2.2 apply when the UE is configured with

-A PCell not using CCA in downlink and one or more SCells using CCA in downlink or

-A PCell and one or more SCells using CCA in downlink

========================= Fourth Change Request ===========================

8.2.2.2.16	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a carrier. When a UE needs to transmit SRS with antenna port switching on a carrier of a serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [20].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

When the signal of cell UE transmt SRS with antenna port switching on is synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X1 slot as specified in Table 8.2.2.2.16-1.

When the signal of cell UE transmt SRS with antenna port switching on is not synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X2 slot as specified in Table 8.2.2.2.16-2.

Table 8.2.2.2.16-1: Interruption length X1 (slot) for synchronized signal of cells

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15 kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 1 | 1 |
| 60 | 2 | 1 |
| 120 | 3 | 1 |

Table 8.2.2.2.16-2: Interruption length X2 (slot) for asynchronized network

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 2 | 2 |
| 60 | 3 | 2 |
| 120 | 4 | 2 |

========================= Fifth Change Request ===========================

8.2.3	NE-DC Interruptions

8.2.3.1	Introduction

This clause contains the requirements related to the interruptions on PCell and SCell, when

E-UTRA PSCell transitions between active and non-active during DRX, or

E-UTRA PSCell transitions from non-DRX to DRX, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is added or released, or

E-UTRA PSCell/SCell in SCG or SCell(s) in MCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA SCG or NR MCG or

PUSCH/PUCCH carrier configuration and deconfiguration in NR MCG, or

UL/DL BWP is switched on PCell or SCell in MCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

E-UTRA SRS carrier based switching, or

NR SRS transmission with antenna port switching.

The requirements shall apply for NE-DC with an NR PCell.

This clause contains interruptions where victim cell is PCell or SCell belonging to MCG. Requirements for interruptions requirements when the victim cell is E-UTRA PSCell or E-UTRA SCell belonging to SCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gap, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gap, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

========================= Sixth Change Request ===========================

8.2.3.2.16	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a carrier. When a UE needs to transmit SRS with antenna port switching on a carrier of a serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [20].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

When the signal of cell UE transmt SRS with antenna port switching on is synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X1 slot as specified in Table 8.2.3.2.16-1.

When the signal of cell UE transmt SRS with antenna port switching on is not synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X2 slot as specified in Table 8.2.3.2.16-2.

Table 8.2.3.2.16-1: Interruption length X1 (slot) for synchronized signal of cells

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15 kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 1 | 1 |
| 60 | 2 | 1 |
| 120 | 3 | 1 |

Table 8.2.3.2.16-2: Interruption length X2 (slot) for asynchronized network

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 2 | 2 |
| 60 | 3 | 2 |
| 120 | 4 | 2 |

========================= Seventh Change Request ===========================

8.2.4	NR-DC: Interruptions

8.2.4.1	Introduction

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell.

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

NR SRS transmission with antenna port switching.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

========================= Eighth Change Request ===========================

8.2.4.2.14	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a carrier. When a UE needs to transmit SRS with antenna port switching on a carrier of a serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [20].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

When the signal of cell UE transmt SRS with antenna port switching on is synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X1 slot as specified in Table 8.2.4.2.14-1.

When the signal of cell UE transmt SRS with antenna port switching on is not synchronized with the signal of active serving cell interrupted, the interruption is allowed with up to X2 slot as specified in Table 8.2.4.2.14-2.

Table 8.2.4.2.14-1: Interruption length X1 (slot) for synchronized signal of cells

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15 kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 1 | 1 |
| 60 | 2 | 1 |
| 120 | 3 | 1 |

Table 8.2.4.2.14-2: Interruption length X2 (slot) for asynchronized network

| SCS of victim cell (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 2 | 2 |
| 60 | 3 | 2 |
| 120 | 4 | 2 |