3GPP TSG-RAN4 Meeting #101-e	                                                         R4-2117346

Electronic meeting, Nov. 1 – 12, 2021

Title:	Discussion on UE positioning accuracy requirements

Source:	CATT

Agenda Item:	5.1.4.2.1.2

Document for:	Discussion

1 Introduction

In RAN4#100-e meeting, the performance part of NR positioning was discussed in maintenance. Most issues have been agreed and the accuracy requirements have been included in specification TS 38.133. But there is still one open issue to be discussed for PRS RSRP measurement. The conclusions and remaining issues are captured in the WF [1].

In this paper, we have further discussion on the remaining issue and give our proposals.

2 Discussion

2.1 PRS-RSRP measurement accuracy

Based on the discussion in RAN4#100-e and previous meeting, the accuracy requirements of PRS RSRP are updated and captured in the specification. The accuracy requirements in normal condition have been decided, but still not conclude in the extreme condition.

- The accuracy requirements in extreme condition

For power based measurement such as SS-RSRP and CSI-RSRP, the requirements in extreme condition are defined and generally 1~3dB larger than the requirements in normal condition. For PRS RSRP measurement, we don’t see the difference with other power based measurement, so we suggest using the similar approach to define the requirements in extreme condition. So we give our proposals on the margin for the requirements in extreme condition.

Proposal 1: The PRS RSRP accuracy requirements in extreme condition are X dB larger than that in normal condition, and X is:

- 3dB for absolute accuracy for FR1.

- 3dB for absolute accuracy for FR2.

- 1dB for relative accuracy for FR1.

- 3dB for relative accuracy for FR2.

And for the accuracy requirements in extreme condition, we think it can be more related to SINR condition rather than PRS configuration, so we suggest to define a single requirement for each SINR side condition.

Proposal 2: For the PRS RSRP accuracy requirements in extreme condition, define a single value for each SINR side condition.

Based on the discussions above, a draft CR for UE positioning accuracy requirements is provided in another paper [2].

3 Summary

In this paper, we further discuss the accuracy requirements of  PRS RSRP measurement and the following proposals are given：

Proposal 1: The PRS RSRP accuracy requirements in extreme condition are X dB larger than that in normal condition, and X is:

- 3dB for absolute accuracy for FR1.

- 3dB for absolute accuracy for FR2.

- 1dB for relative accuracy for FR1.

- 3dB for relative accuracy for FR2.

Proposal 2: For the PRS RSRP accuracy requirements in extreme condition, define a single value for each SINR side condition.

- References

- R4-2115307, WF on NR Positioning Performance Requirements, Intel

- R4-2117347, DraftCR on UE positioning accuracy requirements, CATT