3GPP TSG-RAN WG4 Meeting # 101-e		R4-2117361

Electronic Meeting, November 01-12, 2021

Agenda item:	8.8.2.3

Source: 	CATT

Title: 	Discussion on L1-SINR measurements for FR1 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR1 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR1 HST was approved [1] in RAN4#100-e meeting. But L1-SINR issues are still open.

In this contribution, we provide our consideration of L1-SINR measurements for FR1 HST and give our proposals.

Discussion

In way forward [1], it is listed as below:

Issue 3-1: whether L1-SINR measurement is applicable to HST - Option 1 (CMCC, vivo): Yes. The configuration of CMR/IMR is up to network configuration, there is no restriction on L1-SINR for HST scenario - Option 2 (MTK, OPPO, Ericsson, HW, Apple, QC, Nokia, Intel): RAN4 needs to confirm whether L1-SINR measurement is applicable for HST or not Issue 3-2: If L1-SINR measurement is applicable to HST, whether it is necessary to specify upper bound of side condition for L1-SINR measurement - Companies are encouraged to evaluate whether an applicable upper bound for L1-SINR is needed and what is the concrete value of the upper bound, if needed. - Following is the simulation assumption for information Simulation parameters; Comments/values. Carrier frequency for Cell 1; 3.6 GHz. channel; AWGN. BS transmit antennas for CSI-RS; 1 tx. UE receive antennas; 2 rx. Data channel subcarrier spacing; The same as CSI-RS subcarrier spacing. Measurement period (in number of measurement samples); 1 sample. Subcarrier spacing; 15 kHz; 30 kHz. CSI-RS periodicity; 5 ms. Carrier frequency offset; Option 1: 972Hz for 15kHz; 1667Hz for 30kHz; Option 2: 1944Hz for 15kHz; 3334Hz for 30kHz. Propagation condition; High speed train condition specified in Table G.3 in TS 38.104. Number of PRBs; 48. Density; 3. Side condition (SNR) on CMR; -3 dB. Frequency tracking; TRS.

L1-SINR is introduced in Rel-16 enhancement for MIMO. In Rel-15, L1-RSRP reporting is specified to report the measured energy level. In multi-beam scenario, L1-SINR can reflect to actual beam quality with interference in multi-beam such as MU-MIMO case or multi-TRP. It also can be applied in HST scenarios. Therefore, we think L1-SINR measurement is applicable for HST.

Proposal 1: L1-SINR measurement is applicable for HST.

L1-SINR requirement is defined in clause 9.8 in TS38.133. If looking into the measurement period requirement for CSI-RS based CMR and on dedicated IMR configured/SSB based CMR and dedicated IMR configured/CSI-RS based CMR and dedicated IMR configured, the measurement time works for HST scenario. For the upper bound of SINR, the SINR is measured for multi-beams in the cell which should be same as current link level simulation. We think current L1-SINR measurement requirement can be reused in HST.

Proposal 2: Current L1-SINR measurement requirement can be reused in HST.

Conclusion

In this contribution, we provide our consideration of L1-SINR measurements for FR1 HST and present our proposals as below:

Proposal 1: L1-SINR measurement is applicable for HST.

Proposal 2: Current L1-SINR measurement requirement can be reused in HST.

Reference

[1] R4-2115439, WF on RRM for FR1 HST, CMCC