3GPP TSG-RAN WG4 Meeting # 101-e		R4-2117373

Electronic Meeting, November 1-12, 2021

Agenda item:	11.2.8

Source: 	CATT

Title: 	Discussion on the inconsistency issue of power saving for higher priority frequency layer

Document for:	Discussion

Introduction

RAN2#115-e approved “Reply LS on RRM relaxation in power saving” (R2-2108877) [1]. In this Reply LS, RAN2 asked a question whether there is inconsistency issue of the case when lowMobilityEvaluation and cellEdgeEvaluation criteria are fulfilled.

In this contribution, we provide our consideration and provide the proposal. We also provide a draft reply LS to RAN2 in [2].

Discussion

For inter-frequency measurements/inter-RAT measurements, UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority.

For the further discussion about the condition “Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ”, the figure as below is used for simply explanation.

In 38.133 [3], it is mentioned for UE fulfilling only low mobility criterion:

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax then the UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search where, - Thigher_priority_search = (60 * Nlayers) in clause 4.2.2.7 of 38.133, - Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information, - K2 = 60. - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is not configured with highPriorityMeasRelax then the UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search

The condition “Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ” means that UE is in the yellow zone.

For power saving, three scenarios are defined as:

Scenario #1: (lowMobilityEvaluation configured and UE has fulfilled) or (both lowMobilityEvaluation and cellEdgeEvaluation are configured but combineRelaxedMeasCondition is not configured and UE has fulfilled only lowMobilityEvaluation criterion)

Scenario #2: (cellEdgeEvaluation configured and UE has fulfilled) or (both lowMobilityEvaluation and cellEdgeEvaluation are configured but combineRelaxedMeasCondition is not configured and UE has fulfilled only cellEdgeEvaluation criterion) .For Scenario #2, means that UE is in the yellow+blue zone because SSearchThresholdP is lower than SnonIntraSearchP.

Scenario #3: both lowMobilityEvaluation and cellEdgeEvaluation are configured and UE has fulfilled both criteria

When the condition “Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ “is met, let’s look into the yellow zone. The different relaxation levels are summarized in table as below:

| With Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Scenario #1 | Scenario #2 | Scenario #3 |
| --- | --- | --- | --- |
| highPriorityMeasRelax is configured | 60 * (60 * Nlayers) second | (60 * Nlayers) second | 1 hour |
| highPriorityMeasRelax is not configured | (60 * Nlayers) second | (60 * Nlayers) second | 1 hour |

If the UE is in yellow zone and of scenario #1, and is highPriorityMeasRelax configured, if NW configured cellEdgeEvaluation in addition, it leads to a less relaxed level of 1 hour. It is not very logical.

Therefore we propose to fix it as:

| With Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Scenario #1 | Scenario #2 | Scenario #3 |
| --- | --- | --- | --- |
| highPriorityMeasRelax is configured | 60 * (60 * Nlayers) second | (60 * Nlayers) second | 60 * (60 * Nlayers) second |
| highPriorityMeasRelax is not configured | (60 * Nlayers) second | (60 * Nlayers) second | 1 hour |

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ of Senario #3, which means UE is out of yellow zone but inside blue zone with low speed, keep 1 hour as the previous agreement.

Proposal 1: For inter-frequency measurements/inter-RAT measurements for both lowMobilityEvaluation and cellEdgeEvaluation are configured and UE has fulfilled both criteria, UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority at least every 60 * (60 * Nlayers) seconds when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and highPriorityMeasRelax is configured.

Conclusion

In this contribution, we discussed the issue raised by RAN2 and provide proposal. We also provide a draft reply LS in [2].

Proposal 1: For inter-frequency measurements/inter-RAT measurements for both lowMobilityEvaluation and cellEdgeEvaluation are configured and UE has fulfilled both criteria, UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority at least every 60 * (60 * Nlayers) seconds when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and highPriorityMeasRelax is configured.

Reference

[1] R2-2108877, Reply LS on RRM relaxation in power saving

[2] R4-2117376

[3] TS38.133 v16.9.0