3GPP TSG-RAN WG4 Meeting #101-e	R4-2117377

Electronic Meeting, November 01-12, 2021

Title: 	On open issue for NTN System parameters

Source: 	CATT

Agenda item:	8.13.1.1

Document for:	Discussion

- Introduction

In RAN4#100-e meeting, a way forward for NTN general part was approved [1] with some open issues for further study. This document will discuss the remaining open issues for NTN system parameters and present our proposals.

- Discussion

2.1 TN satellite band numbering

It was decided to introduce NTN band numbering respecting existing band coding and signalling design. The numbering will be started from the largest number in FR1 with decreasing order on first come first serve basis, e.g. S band will be numbered as 256.

The only open issue is whether to use prefix “s” or prefix “n” for NTN band. The prefix is just for purpose of easy reading and some distinction of bands for different purpose. It does not matter RAN2 signalling aspect. We think either “s” or “n” is ok given that the above agreed scheme already distinguished TN bands and NTN bands from RAN4 perspective. We are ok to use prefix “s” if there is a preference to further distinguish NTN bands more clearly.

Observation: Either “s” or “n” is ok to be used as prefix for NTN bands. Prefix “s” can be used to further distinguish NTN bands.

Proposal 1: It is proposed to approve S band as s256 in Table 2-1.

Table 2-1 S band for satellite operation

| Satellite band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| s256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| Note: The NTN bands are numbered in decreasing order on first come first serve basis. |  |  |  |

2.2 NTN channel bandwidth

Proposal 2: It is proposed to approve the following channel bandwidth for S band in Table 2-2.

Table 2-2 channel bandwidth for S band

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n256 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| …… |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2.3 NTN Channel Spacing

It was decided to use 100 kHz for both MSS S-Band and L-Band channel raster in [2]. Then the following channel spacing definition is applicable for NTN.

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NTN carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

Proposal 3: It is proposed approve nominal channel spacing for NTN as,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

2.4 Channel raster

In TN system, the NR-ARFCN is defined for frequency range of 0Hz ~ 100GHz. The channel raster of NTN system is agreed as 100 kHz. The NR-ARFCN should be reused for NTN system. The Channel raster to resource element mapping should be applied also for NTN system.

Proposal 4: It is proposed to approve the NR ARFCN range for NTN as in Table 2-4

Table 2-4: Applicable NR-ARFCN per operating band in FR1

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| … | … | … | … |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| …… |  |  |  |

2.5 Synchronization raster

An important principle for the sync raster in 38.104 is to reduce the UE cell search time in NR. So the TN sync raster number has been designed as few as possible. For NTN, the time for a NTN UE staying within a NTN cell may be very short and the cell search should be as sooner as possible. It is FFS whether the synchronization raster entries for NTN operating band should be optimized. The detail synchronization raster entries need further study based on operator input.

Proposal 5: It is proposed to approve the NR ARFCN range for NTN as in Table 2-5.

Table 2-5: Applicable SS raster entries per operating band (FR1)

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| … | … | ... | … |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |

- Conclusion

This document discussed the open issues for NTN system parameters and concluded the following proposals:

Proposal 1: It is proposed to approve S band as s256 in Table 2-1.

Proposal 2: It is proposed to approve the following channel bandwidth for S band in Table 2-2.

Proposal 3: It is proposed approve nominal channel spacing for NTN as,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

Proposal 4: It is proposed to approve the NR ARFCN range for NTN as in Table 2-4

Proposal 5: It is proposed to approve the NR ARFCN range for NTN as in Table 2-5.

- Reference

[1]	R4-2115460, Way Forward on NTN_solutions_Part1, Thales

[2]	R4-2108099