3GPP TSG-RAN WG4 Meeting # 101-e 		                       R4-2117382

Electronic Meeting, November 01-12, 2021

Title: 	Further discussion on Rx requirements for NTN BS

Source: 	CATT

Agenda item:	8.13.3.2

Document for:	Discussion

- Introduction

A WF [1] on NTN BS RF requirements (BS type 1-H) is agreed in RAN4#100e meeting. This contribution will further discuss the following open issue for transmitter side.

- FRC

- NF and REFSENS

- Dynamic range

- In-band selectivity and blocking

- Out-of-band blocking

- Receiver intermodulation

- In-channel selectivity

- Discussion

  - FRC

Since RAN1 does not introduce any changes on channel coding and design. The current FCS can be reused.

  - NF and REFSENS

The noise figure should be an overall value composed by that of NTN-payload and NTN-Gateway. NTN manufacture’s input is needed on this value. A requirement structure is as shown in Table 2-1. SNR operating point, NF and IM can be reused for S band. FFS on noise figure.

Table 2-1 REFSENS for NTN BS class A/B/C

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5/10/15/20 | 15/30/60 | TBD | TBD |

Proposal 1: it is proposed to introduce REFENSE requirement for NTN BS as shown in Table 2.1-1.

- SNR operating point, NF and IM can be reused for S band. FFS on noise figure.

  - Dynamic range

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C or TAB connector for BS type 1-H inside the received BS channel bandwidth. The interfering signal level can be determined by system level simulation under NTN operating scenario.

  - In-band selectivity and blocking

This requirement depends on inter-operator co-existence scenario. Usually it depends on intra-RAT inter-system co-existence study. But not RAN4 put NTN-NTN scenario in Phase 2. RAN4 needs to decide which co-existence scenario this requirement should be based on.

Proposal 2: it is proposed that RAN4 decide which this requirement should be based on evaluations on NTN-NTN co-existence.

- If yes, NTN-NTN scenario needs to move to Phase 1 and simulation should be done ASAP.

  - Out-of-band blocking

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

There is no co-existence system and interference signal over the air for satellite from scenario point of view.

This requirement is not needed.

Proposal 3: out-of-band blocking requirement is not needed for satellite BS.

  - Receiver intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

We think this requirement is meaningless for NTN BS considering its operating scenario.

Proposal 4: receiver intermodulation requirement is not needed for satellite BS.

  - In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an interfering signal received at a larger power spectral density.

In the requirement for TN BS, the interference signal level and the wanted signal level differs about 20dB (as cited in Table 2.7-1 for some example CBW).

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

For NTN, all the UEs’ pathloss to satellite will not differ very much as shown in figure 2.7-1. In this case it seems the ICS requirement is not as important for NTN as for TN.

Figure 2.7-1 CDF of path loss for NTN

Proposal 5: RAN4 discuss whether to keep ICS requirement for NTN BS.

- If yes, the interference level should be changed for NTN BS.

- Conclusion

This paper gave some initial consideration on NTN BS Rx RF requirements. The following proposals are concluded.

Proposal 1: it is proposed to introduce REFENSE requirement for NTN BS as shown in Table 2.1-1.

- SNR operating point, NF and IM can be reused for S band. FFS on noise figure.

Proposal 2: it is proposed that RAN4 decide which this requirement should be based on evaluations on NTN-NTN co-existence.

- If yes, NTN-NTN scenario needs to move to Phase 1 and simulation should be done ASAP.

Proposal 3: out-of-band blocking requirement is not needed for satellite BS.

Proposal 4: receiver intermodulation requirement is not needed for satellite BS.

Proposal 5: RAN4 discuss whether to keep ICS requirement for NTN BS.

- If yes, the interference level should be changed for NTN BS.

- Reference

[1]	R4-2115641, WF on NTN BS RF requirements (BS type 1-H), CATT

[2]	R4-2117379, further discussion on NTN specification, CATT

[3]	R4-2117378, Further discussion on NTN BS class, CATT

[4]	R4-2117384, Co-existence simulation results for NTN <--> TN scenarios, CATT