3GPP TSG-RAN WG4 Meeting #101-e                                                                                                       R4-2117390

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	Discussion on the BS RX RF requirements for 52.6-71GHz

Agenda item:	8.16.4.2

Document for:	Approval

- Introduction

In RAN4#99-e, a WF [1] on BS RF RX requirements for 52.6 – 71 GHz was approved. The FRC, interferer signal levels, and receiver spurious emissions need to be further discussed.

In this contribution, we provide our views on the remaining issues in the WF [1].

- Discussion

FRC

As per the WF [1], the remaining issues concerning FRC are shown as below:

Further discuss definition of new FRC required for 480 kHz and 960 kHz SCS for reference sensitivity and ICS requirement.

Because the FRC is only related to SCS and bandwidth instead of frequency, so the existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6GHz.

Proposal 1：Existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6-71GHz.

There is need to define G-FR2-A1-6 and G-FR2-A1-7 for sensitivity for 400MHz/480kHz and 400MHz/960kHz respectively. And there is also need to define G-FR2-A1-8 and G-FR2-A1-9 for ICS for 400MHz/480kHz and 400MHz/960kHz respectively.  The TBS (bits) for new FRCs with 95% SU are shown in the following Table 2-1 with assumed RB number. The TBS (bits) can be further discussed when the NRB for 400MHz/480kHz and 400MHz/960kHz is finally decided.

Table 2-1: TBS for new FRCs for 52.6-71GHz

| FRC | CBW(MHz) | SCS(kHz) | TBS(bits) |
| --- | --- | --- | --- |
| G-FR2-A1-6 | 400([66PRBs]) | 480 | [5632] |
| G-FR2-A1-7 | 400([32PRBs]) | 960 | [2792] |
| G-FR2-A1-8 | [33PRBs] | 480 | [2856] |
| G-FR1-A1-9 | [16PRBs] | 960 | [1416] |

The SNR value for each new FRC shall be specified by link level simulation, the Simulation assumptions in R4-1801031[2] for FR2 FRC simulation can be as starting point for 52.6-71GHz. The MCS index 4 and target coding rate = 308/1024, MMSE receiver, DMRS pattern: 1+1 (2, 11), no data in DMRS symbol, PTRS configuration: KPTRS=2, LPTRS=1, No phase noise simulation, no data in DMRS REs can be used for 52.6-71GHz.

In-band blocking

As per the WF [1], the remaining issues concerning in-band blocking are shown as below:

Further discuss interferer signal levels for in-band blocking.

Existing FR2-1 OTA in-band blocking requirement shall apply from FUL_low - ΔfOOB to FUL_high + ΔfOOB. The ΔfOOB for FUL_high – FUL_low ≤ 4000 MHz is defined in table 10.5.2.3-0.

Table 10.5.2.3-0: ΔfOOB offset for NR operating bands in FR2 BS type; Operating band characteristics; ΔfOOB (MHz). BS type 2-O; FUL_high – FUL_low ≤ 4000 MHz; 1500.

Existing ΔfOOB for FUL_high – FUL_low ≤ 4000 MHz is aligned with ΔfOBUE for FUL_high – FUL_low ≤ 4000 MHz.  So when FUL_high – FUL_low > 4000 MHz, ΔfOOB can be aligned with ΔfOBUE. The ΔfOBUE for FUL_high – FUL_low ≤ 4000 MHz is discussed in [] and  is proposed to specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6GHz. So we propose to specifyΔfOOB = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6GHz.

Proposal 2: To specifyΔfOOB = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

Receiver spurious emission

As per the WF [1], the remaining issues concerning receiver spurious emission are shown as below:

For receiver spurious emission further consider following options: Option 1: Use FR2 approach with necessary adaptations on step size for spurious emissions. Option 2: Use ETSI EN 303 722 and/or 303 753 for spurious emissions.

For option1, the existing FR2-1 receiver spurious emission requirements (category B) including frequency range, limit, and measurement bandwidth are aligned with FR2-1 transmitter spurious emission requirement (category B). This is reasonable because that spurious emission of transmitter and receiver can’t be differentiated by a victim for TDD system. The OTA spurious emissions limits are specified as TRP.

For option 2, the transmitter spurious emissions and receiver spurious emissions in ETSI EN 303 72[2] are copied and shown in the following table, from table, the emission limit for 1 GHz to 142 GHz for transmitter (EIRP) is -30dBm/1MHz, meanwhile,  the emission limit for 1 GHz to 142 GHz for receiver (EIRP) is -47dBm/1MHz. So it can be observed that the frequency range, and emission limit for transmitter spurious emissions and receiver spurious emissions are different, so transmitter spurious emissions and receiver spurious emission is not aligned. The OTA spurious emissions limits are specified as EIRP, the -47dBm/ 1MHz EIRP in ETSI EN 303 72 is more stringent than -30dBm/1MHz TRP of FR2-1 approach in TS 38.104,  so the spurious emission in ETSI EN 303 72 is more stringent than FR2-1 approach.  Considering large path loss for 52.6GHz, the spurious emission limit doesn’t need to be tightened. Therefore, we propose option1, the step frequencies (Fstep,X,x=1-6) for NR operating bands in 52.6-71GHz can be aligned with  our proposed step frequencies for transmitter spurious emissions[3] shown in table 2-3.

Proposal 3: Use FR2 approach with necessary adaptations on step size for spurious emissions (option1).

Table 4: Transmitter unwanted emissions in the spurious domain Frequency range; Emission Limit ERP (≤ 1 GHz) EIRP (> 1 GHz); Measurement Bandwidth. 30 MHz to 47 MHz; -36 dBm; 100 kHz. 47 MHz to 74 MHz; -54 dBm; 100 kHz. 74 MHz to 87,5 MHz; -36 dBm; 100 kHz. 87,5 MHz to 118 MHz; -54 dBm; 100 kHz. 118 MHz to 174 MHz; -36 dBm; 100 kHz. 174 MHz to 230 MHz; -54 dBm; 100 kHz. 230 MHz to 470 MHz; -36 dBm; 100 kHz. 470 MHz to 694 MHz; -54 dBm; 100 kHz. 694 MHz to 1 GHz; -36 dBm; 100 kHz. 1 GHz to FL GHz; -30 dBm; 1 MHz. FH GHz to 142 GHz; -30 dBm; 1 MHz. Note: See ERC REC 74-01 [i.7]. Table 5: Receiver spurious emissions Frequency band; Emission Limit ERP (≤ 1 GHz) EIRP (> 1 GHz); Measurement Bandwidth. 30 MHz to 1 GHz; -57 dBm; 100 kHz. 1 GHz to 142 GHz; -47 dBm; 1 MHz. Note: Note: See ERC REC 74-01 [i.7].

Table 2-3: Step frequencies for 66-71GHz, and 57-71GHz with proposedΔfOBUE = 3500 MHz

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | FDL,low | FDL,high | fOBUE | WB | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n257 | 18 | 23.5 | 25 | 26.5 | 29.5 | 1.5 | 3 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 24.25 | 27.5 | 1.5 | 3.25 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 39.5 | 43.5 | 1.5 | 4 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 37 | 40 | 1.5 | 3 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26 | 27.5 | 28.35 | 1.5 | 0.85 | 29.85 | 30.35 | 38.35 |
| nxxx (lic) | 46 | 61 | 62.5 | 66 | 71 | 3.5 | 5 | 74.5 | 76 | 91 |
| nyyy (unlic) | 18 | 43 | 53.5 | 57 | 71 | 3.5 | 14 | 74.5 | 85 | 127 |

Receiver intermodulation

As per the WF [1], the agreements and remaining issues concerning receiver intermodulation are shown as below:

- The interferer levels for general receiver intermodulation for NR operation in 52.6 – 71 GHz range can be derived by applying an offset below the in-band blocking levels, unless technical concerns are identified with this approach by the next RAN4 meeting. Further discuss interferer signal levels for receiver intermodulation.

The analyses of TR 38.817-2[4] show 8 dB offset below in-band blocking levels principles for FR2 are same with the relation between general blocking level and receiver intermodulation level in UTRA, E-UTRA, MSR(37.104), and NR FR1. So, the 8 dB offset below OTA in-band blocking levels principles for receiver intermodulation interferer level is a general principle, so it can be reused for 52.6-71GHz.

Proposal 4: Re-use the 8 dB offset below OTA in-band blocking levels for receiver intermodulation interferer level for 52.6-71GHz.

- Conclusion

This contribution provides analysis on BS RX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1：Existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6-71GHz.

Proposal 2: To specifyΔfOOB = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

Proposal 3: Use FR2 approach with necessary adaptations on step size for spurious emissions (option1).

Proposal 4: Re-use the 8 dB offset below OTA in-band blocking levels for receiver intermodulation interferer level for 52.6-71GHz.

- References

- R4-2115644，WF on BS RX RF requirements，Ericsson, Nokia, RAN4#100-e

- Draft ETSI EN 303 722 V0.0.11 (2021-06),Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 GHz to 71 GHz band; Harmonised Standard for access to radio spectrum, ETSI

- R4-2117389, Discussion on the BS TX RF requirements for 52.6-71GHz,CATT,RAN4#101-e

- TR 38.817-2.