3GPP TSG-RAN WG4 Meeting #101-e                                                             R4-2117393

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	BS TX RF requirements for 1024QAM for NR FR1

Agenda item:	8.6.3

Document for:	Approval

- Introduction

In RAN4 #100-e, a WF on 1024QAM BS RF requirements was approved [1]. The open issues are shown as follows:

RAN4 will make decision on Nov 2021 RAN4 meeting with following options: - Option 1: 2.5% for all classes - Option 2: 2.5% for LA and MR classes; 2.8% for WA class - Option 3: 2.8% for all BS classes

In this contribution, we provide our views on these issues for 1024QAM for NR FR1 based on simulation results.

- Discussion

To compare the performance gain of 2.5% TX EVM compared to 2.8% TX EVM, the level simulation are done with the simulation assumptions in WF [2] are listed in Table A.1-1 in Annex.

The throughput comparison of 2.8% and 2.5% TX EVM with 3% Rx EVM for Rank 1 in TDL-A is shown in Figure 2-1.

Figure 2-1 Throughput comparison of 2.8% and 2.5% TX EVM for rank1 in TDL-A

We summarize the crossover SNR between 256QAM and 1024QAM for rank 1 as shown in Table 2.1-1. From the Table 2.1-1, it is observed that

  - The crossover SNR with 2.8% TX EVM in TDL-A is ~29.89dB.

  - The crossover SNR with 2.5% TX EVM in TDL-A is ~28.54dB.

Observation 1: The crossover SNR for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 29.89dB and 28.54dB respectively.

Table 2.1-1 Summary of crossover SNR between 256QAM and 1024QAM in TDL-A, Rank 1

| Channel | txEVM rxEVM | 3% |
| --- | --- | --- |
| TDL-A, 1T4R | 2.5% | 28.54 |
| TDL-A, 1T4R | 2.8% | 29.89 |

The throughput gain of 1024QAM compared to 256QAM for rank 1 is shown in Table 2.1-2. From table 2.1-2, the following is observed,

  - The throughput gain of 1024QAM compared to 256QAM with 2.8% TX EVM in TDL-A is increased by ~20.2%.

  - The throughput gain of 1024QAM compared to 256QAM with 2.5% TX EVM in TDL-A is increased by ~22.6%.

Observation 2: The throughput gain of 1024QAM compared to 256QAM for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 20.2% and 22.6% respectively.

Table 2.1-2 Summary of throughput gain of 1024QAM compared to 256QAM (percentage@40dB)

in TDL-A , Rank1

| Channel | txEVM rxEVM | 3% |
| --- | --- | --- |
| TDL-A, 1T4R | 2.5% | 22.6% |
| TDL-A, 1T4R | 2.8% | 20.2% |

From simulation results analysis, the 2.5% TX EVM compared to 2.8% TX EVM can only achieve 1.35dB SNR gain and 2.4% throughput gain. The gain of 2.5% TX EVM compared to 2.8% TX EVM is marginal. Referring to LTE [3], the required EVM for 1024QAM is 2.5%. For NR, the even larger bandwidth will add difficulties to CFR (Crest Factor Reduction), TX linearity (particular PA non-linearity) and I/Q compression, which may require larger EVM budget compared to LTE. To reduce difficulty of product implementation, we think 2.8% EVM is a good compromise from simulation results. Therefore, we propose to adopt option 3 for BS EVM requirement for 1024QAM.

Observation 3: The 2.8% TX EVM compared to 2.5% TX EVM only losses 1.35dB SNR gain and 2.4% throughput gain, which is not obvious, but it is benefit for product implementation for NR.

Proposal 1: To adopt Option 3: 2.8% for all BS classes.

- Conclusions

This contribution discusses the EVM requirement for 1024QAM based on link level simulation results. The following observations and proposals are concluded.

Observation 1: The crossover SNR for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 29.89dB and 28.54dB respectively.

Observation 2: The throughput gain of 1024QAM compared to 256QAM for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 20.2% and 22.6% respectively.

Observation 3: The 2.8% TX EVM compared to 2.5% TX EVM only losses 1.35dB SNR gain and 2.4% throughput gain, which is not obvious, but it is benefit for product implementation for NR.

Proposal 1: To adopt Option 3: 2.8% for all BS classes.

- References

- R4-2115638，WF on 1024QAM requirements，Ericsson, RAN4#100-e

- R4-2108077，WF on Link level simulation assumptions，CATT, RAN4#99-e

- 3GPP TS 36.104 V17.0.0 (2020-12)

- Annex

Table A.1-1 Summary of link simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 40MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz (used as Baseline) |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table according to the agreement in RAN1 #104, and other MCSs are not precluded |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| Number of HARQ transmission | 8 HARQ processes, maximum 4 transmissions |
| RV sequence | {0,2,3,1} |
| RANK | One and Two |
| BS antenna configuration | Take 1 and 2 Tx antenna as baseline. Companies also welcome to bring 8TX results if they demonstrate differing trends |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE-IRC |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | DMRS type : Type 1, Single symbol, additional DM-RS (dmrsAdditionalPosition): pos1 |
| TX EVM | 2.5%, 2.8% |
| RX EVM | 3%. |