3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117432

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.1.1

Source:	Apple Inc.

Title:	Discussion on PDSCH requirements in intercell interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e PDSCH demodulation requirements with MMSE-IRC for inter-cell interference was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues, simulation assumptions for requirements definition and simulation results.

# 2. Discussion

## Common Test Parameters

In RAN4#100-e the agreements for common parameters were:

- Channel BW - RAN4 introduce test case under 10MHz for FDD 15kHz and 40MHz for TDD 30kHz only with assumption that interference-plus-noise covariance estimation granularity does not exceed the PRB bundling size regardless the channel bandwidth from RAN4 requirements introduction perspective. - Test setup methodology for signal powers - Use SNR and INR methodology for signal powers setup

Open issues on common test parameters are:

- Network Type

- SSB Configuration

- Propagation Condition

The network type is still open with the following options:

- Synchronized for FDD and TDD

- FFS asynchronized for FDD

  - Interested companies are encouraged to provide view on test setup for asynchronized FDD scenarios (especially, time and frequency offsets)

For ICI scenarios the interference and target cells might not be synchronized but considering synchronized network would have the worse impact on UE performance. Moreover, we don’t see any impact to MMSE-IRC receiver processing for asynchronized network. We don’t see the purpose of considering asynchronous network for ICI scenarios.

Proposal #1: Only consider synchronized network for ICI requirements.

The SSB configuration for interference cell was discussed with the following options in [1]:

- Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

- Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

We already agreed that TRS from serving and interfering cells are overlapping. We prefer to keep SSB non overlapping as the impact to UE demodulation is unknown. The tracking loops might be affected in the actual test with overlapping SSBs and depending on the operating SINR. Give than SSB configuration is very flexible and multiple SSB configurations are possible between cells, we don’t see the necessity to have overlapping SSB for target and interfering cells.

Proposal #2: Configure non-overlapping SSBs on target and interfering cells.

## Target PDSCH Parameters

In RAN4#100-e the MCS for requirements definition for ICI cases was discussed.

  - Down selection between MCS 4 and MCS 13 based on results for agreed INR values based on the following criteria

    - Option 1: Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC

    - Option 2: SINR is not lower than -6 dB

    - Option 3: Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB)

    - Option 4: SNR > INR

    - Other options are not precluded

    - Using of multiple options is not precluded

To guarantee performance of UE processing, the operating SINR should be reasonable, hence we propose to ensure that the SINR is not lower than -6dB for the selected simulation assumptions. There was discussion on difference between SNR and INR to avoid possible handover. The threshold for handover is based on network configuration. There wouldn’t be a handover if either the threshold is set very high, or if the interfering cell is not configured in the Measurement object. If the interference cell (s) is not configured in the MO, the UE wouldn’t measure and report it and there would be no chance of handover. But if possible, selecting SNR > INR would be prefered.

Proposal #3: Down select the MCS based on operating SINR > -6dB and SNR > INR.

## Interference Model

The interference model was discussed in RAN4#100-e, with the following open issues in [1]:

- Deployment

- INR values

- Number of explicitly modeled interferers

Deployment

Homogeneous network assumption is agreed as the baseline.

- Consider Homogeneous deployment assumptions

- FFS whether for consider HetNet deployment assumptions

We think homogeneous network assumption is more suitable to NR and we should use that to define requirements for PDSCH demodulation in ICI.

Proposal #4: Only consider homogeneous network deployment for requirements with inter-cell interference.

INR values for Homogeneous network

For homogeneous network the following values of INR were discussed for synchronous network:

- Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

- Option 2: INRs 7.77 and 2.29 dB in case of 2 interference cells and FFS in case of 1 interference cell

- Option 3: INRs 13.91 and 3.34 dB in case of 2 interference cells and FFS in case of 1 interference cell

- Other options are not precluded

We evaluate the performance for different options for INR levels.

Option 1 with 1 interference cell.

Table 1: Performance in ICI with 1 interference cell

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |
| 2x2 | TDLA10-10 | 4 | 2.7 | 1.6 | -3.2 |
| 2x2 | TDLC300-100 | 4 | 3.0 | 2.4 | -2.5 |
| 2x2 | TDLA10-10 | 13 | 9.8 | 9.1 | 4.3 |
| 2x2 | TDLC300-100 | 13 | 10.7 | 10.3 | 5.5 |
| 2x4 | TDLA10-10 | 4 | -0.5 | -2.7 | -7.5 |
| 2x4 | TDLC300-100 | 4 | 0.2 | -1.3 | -6.1 |
| 2x4 | TDLA10-10 | 13 | 6.1 | 4.5 | -0.3 |
| 2x4 | TDLC300-100 | 13 | 7.2 | 6.1 | 1.3 |

Option 1 with 2 interference cells.

Table 2: Performance in ICI with 2 interference cells – Opt 1

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |
| 2x2 | TDLA10-10 | 4 | 5.1 | 3.9 | -3.3 |
| 2x2 | TDLC300-100 | 4 | 5.3 | 4.7 | -2.5 |
| 2x2 | TDLA10-10 | 13 | 12.1 | 11.4 | 4.2 |
| 2x2 | TDLC300-100 | 13 | 13.1 | 12.7 | 5.5 |
| 2x4 | TDLA10-10 | 4 | 2.1 | -0.6 | -7.8 |
| 2x4 | TDLC300-100 | 4 | 2.5 | 0.8 | -6.4 |
| 2x4 | TDLA10-10 | 13 | 8.5 | 6.4 | -0.7 |
| 2x4 | TDLC300-100 | 13 | 9.4 | 8.4 | 1.2 |

The performance in terms of SINR at 70% max TP is very comparable between 1 cell and 2 cell interference with MMSE-IRC.

Observation #1: Performance with 1 cell and 2 cell interference with MMSE-IRC is very comparable.

With option 3 the INR levels are very high. Such high interference levels might not be practical in homogeneous deployment in our understanding and we don’t see the motivation to consider these for requirements definition.

Proposal #5: Do not consider INR values of 13.91 and 3.34 dB for requirements definition with ICI.

Number of explicitly modeled interference cells

Options discussed in RAN4#100-e:

  - Companies are encouraged to check performance with 1 and 2 interference cells for initial simulations

  - Further discuss the assumptions for requirements definition

In terms of UE processing there is no difference whether interference is from 1 cell or 2 cell for MMSE-IRC receiver. With 2 interfering cells the INR levels are higher than 1 cell to achieve similar performance. We don’t see a strong necessity to introduce requirements to verify MMSE-IRC receiver performance with 2 interference cells.

Proposal #6: Define requirements with 1 interference cell.

Proposal #7: Define requirements with INR 3.1 dB in case of 1 interference cell.

## Simulation Results

For the simulation parameters agreed in [1], we provide simulation results with MMSE-IRC and baseline MMSE receiver.

INR Option 1

1 Cell: 3.1 dB

2 Cells: 5.43, -1.5 dB

Table 5: Performance in ICI with 1 interference cell

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC | Gain with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | TDLA10-10 | 4 | 2.7 | 1.6 | -3.2 | 1.0 |
| 2x2 | TDLC300-100 | 4 | 3.0 | 2.4 | -2.5 | 0.6 |
| 2x2 | TDLA10-10 | 13 | 9.8 | 9.1 | 4.3 | 0.7 |
| 2x2 | TDLC300-100 | 13 | 10.7 | 10.3 | 5.5 | 0.3 |
| 2x4 | TDLA10-10 | 4 | -0.5 | -2.7 | -7.5 | 2.2 |
| 2x4 | TDLC300-100 | 4 | 0.2 | -1.3 | -6.1 | 1.5 |
| 2x4 | TDLA10-10 | 13 | 6.1 | 4.5 | -0.3 | 1.6 |
| 2x4 | TDLC300-100 | 13 | 7.2 | 6.1 | 1.3 | 1.1 |

Table 5: Performance in ICI with 2 interference cell – Opt 1

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC | Gain with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | TDLA10-10 | 4 | 5.1 | 3.9 | -3.3 | 1.2 |
| 2x2 | TDLC300-100 | 4 | 5.3 | 4.7 | -2.5 | 0.6 |
| 2x2 | TDLA10-10 | 13 | 12.1 | 11.4 | 4.2 | 0.8 |
| 2x2 | TDLC300-100 | 13 | 13.1 | 12.7 | 5.5 | 0.5 |
| 2x4 | TDLA10-10 | 4 | 2.1 | -0.6 | -7.8 | 2.7 |
| 2x4 | TDLC300-100 | 4 | 2.5 | 0.8 | -6.4 | 1.7 |
| 2x4 | TDLA10-10 | 13 | 8.5 | 6.4 | -0.7 | 2.0 |
| 2x4 | TDLC300-100 | 13 | 9.4 | 8.4 | 1.2 | 1.1 |

For MCS4 with 4RX the SINR is < -6dB. We also observe that the performance gain with MMSE-IRC is slightly more with TDLA channel.

Observation #2: With 1 cell and 2 cells interference with option 1 INR levels, SINR at 70% max TP with 4RX and MCS 4 is < -6dB.

Observation #3: Performance gain with MMSE-IRC if slightly better with TDLA channel compared to TDLC.

INR Option 2

2 Cells: 7.77, 2.29 dB

Table 5: Performance in ICI with 2 interference cell – Opt 2

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC | Gain with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | TDLA10-10 | 4 | 7.4 | 5.9 | -3.5 | 1.5 |
| 2x2 | TDLC300-100 | 4 | 7.5 | 6.7 | -2.7 | 0.8 |
| 2x2 | TDLA10-10 | 13 | 14.3 | 13.3 | 3.9 | 1.0 |
| 2x2 | TDLC300-100 | 13 | 15.1 | 14.6 | 5.2 | 0.6 |
| 2x4 | TDLA10-10 | 4 | 4.2 | 1.0 | -8.4 | 3.3 |
| 2x4 | TDLC300-100 | 4 | 4.7 | 2.7 | -6.7 | 2.0 |
| 2x4 | TDLA10-10 | 13 | 10.5 | 7.8 | -1.6 | 2.7 |
| 2x4 | TDLC300-100 | 13 | 11.6 | 9.8 | 0.4 | 1.7 |

INR Option 3

2 Cells: 13.91, 3.34 dB

Table 5: Performance in ICI with 2 interference cell – Opt 3

| Ant. Config | Channel Model | MCS | SNR |  | SINR for MMSE-IRC | Gain with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | TDLA10-10 | 4 | 12.1 | 9.6 | -4.9 | 2.5 |
| 2x2 | TDLC300-100 | 4 | 12.2 | 10.5 | -3.9 | 1.7 |
| 2x2 | TDLA10-10 | 13 | 18.8 | 16.8 | 2.3 | 2.1 |
| 2x2 | TDLC300-100 | 13 | 19.8 | 18.4 | 4.0 | 1.4 |
| 2x4 | TDLA10-10 | 4 | 9.1 | 3.1 | -11.4 | 6.0 |
| 2x4 | TDLC300-100 | 4 | 9.3 | 5.5 | -9.0 | 3.9 |
| 2x4 | TDLA10-10 | 13 | 15.0 | 9.4 | -5.0 | 5.6 |
| 2x4 | TDLC300-100 | 13 | 16.1 | 12.3 | -2.2 | 3.9 |

With higher INR levels there is more gain with MMSE-IRC over baseline MMSE since there is more interference rejection.

Based on the results MCS13 might be suitable for requirements definition and also TDLA channel model.

Proposal #8: Define PDSCH demod requirements for ICI with MCS 13 and TDLA channel model.

# 3. Conclusion

In this paper, we present our views on the open issues, simulation assumptions for requirements definition and simulation results for PDSCH demodulation requirements in ICI. Our observations and proposals are captured below:

Proposal #1: Only consider synchronized network for ICI requirements.

Proposal #2: Configure non-overlapping SSBs on target and interfering cells.

Proposal #3: Down select the MCS based on operating SINR > -6dB and SNR > INR.

Proposal #4: Only consider homogeneous network deployment for requirements with inter-cell interference.

Observation #1: Performance with 1 cell and 2 cells interference with MMSE-IRC is very comparable.

Proposal #5: Do not consider INR values of 13.91 and 3.34 dB for requirements definition with ICI.

Proposal #6: Define requirements with 1 interference cell.

Proposal #7: Define requirements with INR 3.1 dB in case of 1 interference cell.

Observation #2: With 1 cell and 2 cells interference with option 1 INR levels, SINR at 70% max TP with 4RX and MCS 4 is < -6dB.

Observation #3: Performance gain with MMSE-IRC if slightly better with TDLA channel compared to TDLC.

Proposal #8: Define PDSCH demod requirements for ICI with MCS 13 and TDLA channel model.

# Reference

- R4-2115730, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel.