3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117434

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.2

Source:	Apple Inc.

Title:	Discussion on PDSCH requirements in MU-MIMO scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e PDSCH demodulation requirements with MMSE-IRC for intra-cell inter-user interference was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues, simulation assumptions for requirements definition for requirements in MU-MIMO and provide simulation results for phase 1 evaluation.

# 2. Discussion

## Inter-user Interference modeling

The agreements in [1] related to inter-user interference modeling:

Number of paired UEs 1 target UE + 1 interference UE Rank for target and interference PDSCH - For rank of target UE is 1 - Use rank 1(Target UE) + 1(Co-scheduled UE) Antenna configuration - 2T2R,2T4R for rank 1+1 - 4T4R for rank 2+1(if introduced) and rank 2+2(if introduced) Codebook Type - Type I SP codebook PRB bundling size - 2 PRBs for 2 and 4Tx cases Precoding granularity - 2 PRBs for 2 and 4Tx cases PMI matrix selection for target UE - Random for 2 and 4Tx cases

Open Issues for inter-user interference modeling are:

- Rank for interference PDSCH when target UE rank is 2

- PMI matrix selection for Co-scheduled UE for 2TX and 4TX

- DMRS ports for case with rank 1+1

- Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE

Rank for interference PDSCH

With target UE rank 2, the following options were discussed in RAN4#100-e for interference UE rank:

- For rank of target UE is 2

  - Option 1: Rank 2(Target UE)+ Rank 1(Co-schedule UE)

  - Option 2: Rank 2(Target UE)+ Rank 2(Co-schedule UE)

  - Option 3: Rank 2(Target UE)+Variable rank(Co-scheduled UE)

With rank 1 or rank 2 on interference UE, other than different interference levels there is no impact to UE processing with baseline MMSE-IRC receiver. With option 3 the test configuration would get very complicated and we wouldn’t be achieving any additional coverage. We also would need input from TE vendors on feasibility of supporting it. Hence, we don’t support option 3.

Observation 1: With rank 2 on target UE and variable rank on interference UE the test set-up would be very complicated.

With rank 1 or rank 2 on co-scheduled UE, depending on the option for signal power, the interference level would be different.

Signal power assumptions

- Option 1: Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1

- Option 2: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

With option 1 for signal power assumption, the SIR would always be 0 dB irrespective of the rank on target and interference UE. Whereas, with option 2 above, the SIR would be 3dB with rank combination 2+1 and 0dB with rank combination 2+2.

Observation #2: Depending on signal power assumption the SIR and rank combination the SIR level would be either 0dB or 3dB.

Since the rank of interfering UE would not affect UE processing for MMSE-IRC receiver, either rank combination 2+1 or 2+2 is acceptable.

Proposal #1: Define requirements with either rank combination 2+1 or 2+2 for 4x4 case.

PMI matrix selection for Co-scheduled UE

The options for PMI selection discussed in RAN4#100-e are:

- Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

- Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

  - Option 2A : Use following method to randomly select PMI matrix for interference UE that is not identical to that of Target UE for rank 2+1

1) Randomly select the PMI matrix in codebook with rank2 and rank1 respectively.

2) Normalize the PMI matrix for each layer for both target UE and co-scheduled UE to make the norm of each PMI matrix of each layer equal to 1/3.

3) If the PMI matrix of rank 1 equals to the PMI matrix of any one layer of rank 2, reselect PMI matrix for Rank 1 with PMI index plus 1 and go back to step 2)

- Option 3: Cover both Option 1 and Option 2 for phase 1 evaluation.

We provide simulation results with orthogonal and random precoders for 2TX and 4TX with different rank combinations below. For 2TX the target and co-scheduled UEs DMRS ports are in the same CDM group. With 4TX, target UE rank 2, the target and co-scheduled UEs DMRS ports are in different CDM groups.

2TX with co-scheduled UEs in same CDM group

Figure 1: Performance with orthogonal and random precoders for 2TX

For rank combination 1+1 with 2 TX, we observe 0.6~0.8 dB degradation in MCS4 and 1.5~2dB degradation with MCS13 with random precoder compared to orthogonal precoder for co-scheduled UEs.

Observation #3: With random precoder we observe 0.6~0.8 dB degradation in MCS4 and 1.5~2dB degradation with MCS13 with random precoder compared to orthogonal precoder for co-scheduled UEs.

4TX with co-scheduled UEs in different CDM group

Figure 2: Performance with orthogonal and random precoders for 2TX

With 4TX and target UE with rank 2, we observe severe degradation with random precoder with 2 layers per co-scheduled UE. In some cases with 2 layers per co-scheduled UE and random precoder max throughput is not achieved at reasonable SNR.

Observation #4: There is severe performance degradation with random precoder and 2 layers per co-scheduled UE, especially with TDLC channel.

It would be a reasonable assumption that in MU-MIMO scenarios the network would employ some algorithm to ensure that the interference among co-scheduled UEs is reduced for better network performance. Also, given the observations above with random precoder and the degradation compared to orthogonal precoder, we recommend to use orthogonal precoders for co-scheduled UEs.

Proposal #2: Define requirements with orthogonal precoder for co-scheduled UEs.

DMRS ports for 2TX

For DMRS ports for co-scheduled UEs, the following options were discussed:

DMRS ports for case with rank 1+1

- Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

- Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

- Option 3: Variable DMRS port mapping during the test.

  - FFS the percent of each mapping and other details

Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE

- Option 1: 1 for target UE and co-scheduled UE.

- Option 2: 2 for target and co-scheduled UE

  - Note: It depends on issue with DMRS ports mapping.

We evaluate performance for the following with 2Tx and rank combination 1+1:

- Diff CDM grp: Co-scheduled UEs on different CDM groups

- Same CDM Grp-Opt1: Co-scheduled UEs on same CDM group with num CDM grps without data=2

- Same CDM Grp-Opt2: Co-scheduled UEs on same CDM group with num CDM grps without data=1

Figure 3: Performance with different DMRS port configs

Based on the results we see that there is no significant performance delta between the 3 options for DMRS port configs.

Observation #5: For 2TX with 1 layer per co-scheduled UE there is no significant performance delta between various DMRS port mapping between co-scheduled UEs.

Option 3 above with variable DMRS port mapping during the test would not add any significant benefit while make the test setup very complex. Hence, we propose not to configure variable DMRS port mapping for 2TX cases with 1+1 rank combination.

For 4TX it was agreed to have co-scheduled UEs on different CDM groups, hence we could configure same DMRS ports for 2TX cases to verify UE performance in both cases. Also, if UEs are scheduled on same CDM group, there would be no need to configure CDM groups without data as 2. Hence to reduce the overhead and have coverage for co-scheduled UEs in same CDM group, we propose to configure co-scheduled UEs in same CDM group with num CDM groups as 1 for both target and interfering UE.

Proposal #3: For 2TX configure co-scheduled UEs on same CDM group with number of CDM groups without data as 1.

## PDSCH Parameters

The open issues related to PDSCH parameters are:

- MIMO correlation

- Propagation conditions

- MCS

- Signal power and SNR assumptions

MIMO Correlation

For MIMO antenna correlation the following options were down selected in [1]:

- Option 1: Use ULA Low for 2TX and XPL Low for 4TX

  - FFS the correlation matrix for XPL Low

- Option 2: ULA Low

We don’t have a definition for XPL Low in 38.101-4. If XPL-Low is defined based on existing XPL configurations with a1=0, a2=0, b=0, g=0, then there would be no difference between ULA low and XPL low correlation model. Hence, we prefer to use ULA Low correlation model for all antenna configurations.

Observation # 6: With XPL-Low defined with a1=0, a2=0, b=0, g=0, there would be no difference between ULA low and   XPL-Low correlation models.

Proposal #4: Define requirements with ULA Low MIMO correlation model for all antenna configurations.

Signal power and SNR assumptions

For signal power and SNR definition the following options were captured in [1]:

Signal power assumptions

- Option 1: Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1

- Option 2: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

SNR assumptions

- Option 1: SNR = STargetUE/N

- Option 2: SNR = (STargetUE+ SInterfUE)/N

For the 2 options for signal power and SNR assumptions above, with option 1 the signal power for MU case would be the same as SU case. Whereas, with option 2, the power would be scaled based on the number of layers per UE. In our opinion, the total power per RE should be the same irrespective of whether the layers are for a single UE or different UEs.

Also, in LTE we have requirements for simultaneous transmission for 2 UEs. In Annex B.4.1 of TS 36.101 the beamforming model includes normalizing of the transmit powers which is aligned to option 2 above.

Observation #7: In LTE for simultaneous transmission, the transmit powers are normalized by the total number of layers.

Hence, for MU-MIMO we should use option 2 for signal power and SNR assumption.

Proposal #5: Use the following assumptions for MU-MIMO requirements: 			     	      Signal power assumption - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal. 			       		       SNR definition - SNR = (STargetUE+ SInterfUE)/N

The MCS and channel model were agreed to be decided based on the simulation results. We present the simulation results for 2Tx and 4TX with the assumption of orthogonal precoders. We also present results with MMSE receiver for comparison against MMSE-IRC.

For 2TX we provide results with different DMRS port mapping for 1=1 rank combination.

- Diff CDM grp: Co-scheduled UEs on different CDM groups

- Same CDM Grp-Opt1: Co-scheduled UEs on same CDM group with num CDM grps without data=2

- Same CDM Grp-Opt2: Co-scheduled UEs on same CDM group with num CDM grps without data=1

Figure 4: Performance of MMSE-IRC vs MMSE for 2x2 case

For 2x2, we observe that with MCS 4 there is no significant delta between MMSE and MMSE-IRC performance. With MCS13 there is improvement in performance with MMSE-IRC compared to MMSE receiver.

Observation #8: For 2x2 there is no significant performance delta between MMSE-IRC and MMSE for MCS4 and there is performance improvement with MMSE-IRC over MMSE for MCS13.

Also, there is no significant performance delta with TDLA and TDLC channel. Given that MU-MIMO is more suitable to low mobility scenarios where PMI feedback from co-scheduled UEs is used to determine suitable precoder for co-scheduled UEs, TDLA would be a more suitable channel model for MU-MIMO requirements.

Proposal #6: Define requirements for MU-MIMO with 2Tx with MCS13 and TDLA channel.

For 4TX we provide results with orthogonal precoders for 2+1 and 2+2 rank combinations.

Figure 5: Performance of MMSE-IRC vs MMSE for 4x4 case

For 4x4 with MCS 13 and 2 layers for UE, the performance with TDLC channel is worse than TDLA by more than 2dB. Also, for 64QAM with 2+1 rank combination with TDLC there is more than 2 dB performance degrade compared to TDLA channel. And, with 2+2 rank combination with TDLC channel, with MMSE-IRC receiver, performance is not good.

Observation #9: Performance with TDLC channel is degraded by over 2 dB compared to TDLA channel for MCS13 with 2 layers per UE and MCS 19 with  2+1 rank combination.

Observation #10: With TDLC and 2 layers per UE with MCS-19, MMSE-IRC cannot achieve good performance.

Based on the observations we propose to only consider TDLA channel for MU-MIMO requirements. Also, we recommend selecting between 2+1 and 2+2 rank combination for requirements definition.

Proposal #7: For 4x4 define requirements with either 2+1 or 2+2 rank combination with TDLA channel.

## Simulation Results

For Phase 1 evaluation, with the agreed simulation assumptions captured in the Appendix, we present results with random and orthogonal precoder for 2TX and 4TX cases for MMSE and MMSE-IRC receiver. The SNR at 70% Max TP is presented in the tables below.

Simulation Assumptions:

Channel BW and SCS: 10MHz, 15KHz

MIMO Correlation: ULA Low

SNR Assumption: SNR = (STargetUE+ SInterfUE)/N

For 2Tx, we present results with 1 layer per co-scheduled UE with different DMRS port mapping.

Table 1: FDD 2TX Simulation Results

| DMRS port mapping | Ant. Config | Prop. Channel | MCS | Orthogonal Precoder |  | Random Precoder |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | MMSE | MMSE-IRC |
| Diff CDM Grp | 2x2 | TDLA30 | 4 | 2.6 | 2.5 | 3.2 | 3.1 |
|  |  |  | 13 | Inf | 12.2 | Inf | 13.7 |
|  |  | TDLC300 | 4 | 2.9 | 2.8 | 3.6 | 3.3 |
|  |  |  | 13 | Inf | 13.5 | Inf | 15.5 |
|  | 2x4 | TDLA30 | 4 | -1.3 | -1.1 | -0.6 | -0.5 |
|  |  |  | 13 | 9.9 | 7.1 | 15.3 | 8.3 |
|  |  | TDLC300 | 4 | -0.8 | -0.7 | -0.2 | -0.2 |
|  |  |  | 13 | 11.1 | 7.7 | 19.6 | 9.3 |
| Same CDM Grp-Opt 1 | 2x2 | TDLA30 | 4 | 2.5 | 2.2 | 3.1 | 2.8 |
|  |  |  | 13 | Inf | 12.4 | Inf | 14.0 |
|  |  | TDLC300 | 4 | 2.8 | 2.5 | 3.5 | 3.1 |
|  |  |  | 13 | Inf | 13.3 | Inf | 15.4 |
|  | 2x4 | TDLA30 | 4 | -1.5 | -1.6 | -0.8 | -0.9 |
|  |  |  | 13 | 9.9 | 6.7 | 15.8 | 8.1 |
|  |  | TDLC300 | 4 | -0.9 | -1.2 | -0.3 | -0.5 |
|  |  |  | 13 | 11.0 | 7.4 | 18.7 | 9.1 |
| Same CDM Grp - Opt 2 | 2x2 | TDLA30 | 4 | 3.0 | 2.8 | 3.6 | 3.4 |
|  |  |  | 13 | Inf | 12.7 | Inf | 14.3 |
|  |  | TDLC300 | 4 | 3.3 | 3.0 | 4.1 | 3.7 |
|  |  |  | 13 | Inf | 13.7 | Inf | 15.7 |
|  | 2x4 | TDLA30 | 4 | -0.8 | -0.8 | -0.1 | -0.1 |
|  |  |  | 13 | 10.1 | 7.1 | 15.7 | 8.4 |
|  |  | TDLC300 | 4 | -0.3 | -0.4 | 0.2 | 0.1 |
|  |  |  | 13 | 11.3 | 7.7 | 18.5 | 9.3 |

For 4Tx, we present results with co-scheduled UE DMRS on different CDM grp with different rank combinations.

Table 2:FDD 4TX Simulation Results

| Ant. Config | Rank Combo. | Prop. Channel | MCS | Orthogonal Precoder |  | Random Precoder |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | MMSE | MMSE-IRC |
| 4x4 | 2+1 | TDLA30 | 13 | 18.2 | 10.8 | 30.3 | 12.2 |
|  |  |  | 19 | Inf | 16.1 | Inf | 17.9 |
|  |  | TDLC300 | 13 | 22.5 | 11.4 | Inf | 12.9 |
|  |  |  | 19 | Inf | 18.6 | Inf | 21.2 |
|  | 2+2 | TDLA30 | 13 | Inf | 14.5 | Inf | 18.3 |
|  |  |  | 19 | Inf | 20.7 | Inf | 25.5 |
|  |  | TDLC300 | 13 | Inf | 16.6 | Inf | 25.9 |
|  |  |  | 19 | Inf | Inf | Inf | Inf |

# 3. Conclusion

In this paper, we present our views on the open issues, simulation assumptions for requirements definition for requirements in MU-MIMO and provide simulation results for phase 1 evaluation. Our observations and proposals are captured below:

Inter-user interference modeling

Observation 1: With rank 2 on target UE and variable rank on interference UE the test set-up would be very complicated.

Observation #2: Depending on signal power assumption the SIR and rank combination the SIR level would be either 0dB or 3dB.

Proposal #1: Define requirements with either rank combination 2+1 or 2+2 for 4x4 case.

Observation #3: With random precoder we observe 0.6~0.8 dB degradation in MCS4 and 1.5~2dB degradation with MCS13 with random precoder compared to orthogonal precoder for co-scheduled UEs.

Observation #4: There is severe performance degradation with random precoder and 2 layers per co-scheduled UE, especially with TDLC channel.

Proposal #2: Define requirements with orthogonal precoder for co-scheduled UEs.

Observation #5: For 2TX with 1 layer per co-scheduled UE there is no significant performance delta between various DMRS port mapping between co-scheduled UEs.

Proposal #3: For 2TX configure co-scheduled UEs on same CDM group with number of CDM groups without data as 1.

PDSCH Parameters

Observation # 6: With XPL-Low defined with a1=0, a2=0, b=0, g=0, there would be no difference between ULA low and   XPL-Low correlation models.

Proposal #4: Define requirements with ULA Low MIMO correlation model for all antenna configurations.

Observation #7: In LTE for simultaneous transmission, the transmit powers are normalized by the total number of layers.

Proposal #5: Use the following assumptions for MU-MIMO requirements: 			     	      Signal power assumption - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal. 			       		       SNR definition - SNR = (STargetUE+ SInterfUE)/N

Observation #8: For 2x2 there is no significant performance delta between MMSE-IRC and MMSE for MCS4 and there is performance improvement with MMSE-IRC over MMSE for MCS13.

Proposal #6: Define requirements for MU-MIMO with 2Tx with MCS13 and TDLA channel.

Observation #9: Performance with TDLC channel is degraded by over 2 dB compared to TDLA channel for MCS13 with 2 layers per UE and MCS 19 with  2+1 rank combination.

Observation #10: With TDLC and 2 layers per UE with MCS-19, MMSE-IRC cannot achieve good performance.

Proposal #7: For 4x4 define requirements with either 2+1 or 2+2 rank combination with TDLA channel.

# Reference

- R4-2115733, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon.

# Appendix

Table A-1: Simulation parameters for MU-MIMO evaluation

| Parameter | Target UE | Co-Scheduled UE |
| --- | --- | --- |
| Duplex mode | FDD |  |
| Active DL BWP index | 1 | 1 |
| CBW/SCS | 10 MHz/ 15 KHz |  |
| RB allocation | Full bandwidth |  |
| MIMO layer | Rank 1,2 | Rank 1,2 |
| Antenna Config and Correlation | 2x2, ULA Low 2x4, ULA Low 4x2, ULA Low 4x4 ULA Low |  |
| Channel | TDLA-30-10 TDLC-300-100 |  |
| PDSCH configuration |  |  |
| Mapping type | Type A | Type A |
| Starting symbol (S) | 2 | 2 |
| Length (L) | 12 | 12 |
| PDSCH DMRS configuration |  |  |
| DMRS Type | Type 1 | Type 1 |
| Number of additional DMRS | 1 | 1 |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 | 1 |
| DMRS Ports | 1 Layer:1000 2 Layers 1000,1001 | 1 Layer:1001/1002 2 Layers 1002,1003 |
| Precoding Model |  |  |
| Granularity | 2 PRB |  |
| Type | Randomly generated per slot from SP Type I codebook Option 1: With orthogonalization Option 2: Without orthogonalization |  |
| HARQ Parameters |  |  |
| Number of HARQ Processes | 4 | N/A |
| Maximum HARQ transmission | 4 | N/A |