3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117435

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.2

Source:	Apple Inc.

Title:	TP to TR 38.833: Interference Modeling for intra-cell inter-user interference

Release:	Rel-17

Document for:	Approval

# 1. Introduction

In NR demodulation performance enhancements in R17 RAN4 is evaluating the performance of inter-cell inter-user interference with MMSE-IRC. In this contribution we provide the text proposal to TR 38.833 on Interference Modeling for inter-cell inter-user interference.

# Reference

- R4-2115733, “WF on MMSE-IRC receiver for intra-cell inter-user interference”, Huawei, HiSilicon.

# Text Proposal

Start of Change

## Interference Model

The PDSCH and DMRS of the paired UEs are precoded prior to transmission. The precoder for each user is denoted by W1' and W2' respectively.

Wi is the precoder matrix of ith UE, of size NTX x NLi. Where, NTX is the number of TX antenna, NLi is the number of layers from ith UE. The combined precoder W = [W1 W2], of size NTX x NL.

The precoders Wi are randomly selected from the codebook with a constraint that the combined precoder W has orthogonal columns, i.e., the off-diagonal entries of WHW are zero.

To maintain the average per UE signal power as NLi /NL, where NL is the total number layers across all users, an additional scaling is applied to the combined precoder as:

W' = 1NLc   *W

W' = [W1' W2']

End of Change