3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117437

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.3.2

Source:	Apple Inc.

Title:	Discussion on Network Assistance for CRS-IM in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e the initial phase for evaluating techniques to cope with CRS interference in NR was concluded and way forward [1] was agreed. In RAN#93-e, it was agreed to introduce requirements for NR PDSCH demodulation for neighbor cell LTE CRS-IM and the revised WID was agreed [2].

- CRS interference handling in scenarios with overlapping spectrum for LTE and NR Phase I: Evaluate techniques to cope with CRS interference in scenarios with overlapping spectrum for LTE and NR - Candidate reference receiver to enable neighboring cell CRS-IM - The performance benefit of neighboring cell LTE CRS-IM over the existing rate matching solutions specified in Rel-15 and Rel-16 shall be evaluated. - Feasibility of the considered solution regarding NR PDSCH processing timeline need to be checked. - Priority will be given to solutions not having RAN1 specification impact. - Synchronous network scenario is prioritized. As second priority, RAN4 could evaluate the feasibility and usefulness of the asynchronous network scenario and specify if feasible and useful. - 15 kHz SCS for NR is prioritized. RAN4 should evaluate the feasibility and usefulness of 30 kHz SCS for scenarios with LTE and NR deployed in neighboring BSs/areas and specify if feasible and useful. Note: The work can be started from May 2021 meeting. Phase II: Define NR PDSCH demodulation requirements for neighbouring cell LTE CRS-IM in scenarios with overlapping spectrum for LTE and NR - Use LLR weighting as baseline reference receiver. - Focus on synchronous network scenario. - 15 kHz SCS for NR is prioritized. - Other aspects will be further discussed in RAN4 and RAN #94e. Note: The 30 kHz SCS scenario will be discussed after RAN #94e meeting.

In this contribution we present our views on network assistance for CRS-IM.

# 2. Discussion

In RAN4#100-e and RAN#93-e network assistance for CRS-IM was discussed but there was no consensus. The evaluation results for Phase 1 captures results with and without network assistance based on inputs from companies, but there was no agreement on how such results are derived. In our view the impact of no network assistance information cannot be quantified by numerical results as the results don’t capture the impact to UE processing, additional complexity or power consumption.

Observation #1: Impact of network assistance cannot be quantified by numerical results as they don’t capture impact to UE complexity, processing and power consumption.

For CRS-IM we have 2 receiver assumptions – LLR weighting and CRS-IC. In RAN#93-e there was a discussion on what information is needed at the receiver side for CRS-IM with these receiver assumptions. Although the processing for LLR weighting and CRS-IC is different at the UE side, the related LTE CRS configuration information needed for both are the same.

From the discussion in RAN#93-e, we categorize the LTE information as follows:

  - Presence information - includes the presence of LTE cell, MBSFN configuration, CRS muting information

  - CRS location information - includes LTE carrier frequency, bandwidth, v-shift, number of CRS ports

  - CRS sequence information - includes physical cell id, slot number and symbol number

For both CRS-IC and LLR weighting we need the LTE/ CRS presence, location and sequence information.

Observation #2: For both LLR weighting and CRS-IC we need LTE/CRS presence, location and sequence information.

The information about LTE/CRS from the interfering cell(s) could be either provided via network assistance information to the UE or the UE would need to blindly detect it.

For the UE to detect the information of LTE interference, it would need to do LTE cell identification and PBCH decoding. It cannot obtain information on MBSFN configuration and CRS muting from cell identification or PBCH decoding alone.

Observation #3: UE needs to detect the LTE interfering cell and perform PBCH decoding to obtain CRS location and sequence information if this information is not provided via network assistance.

Observation #4: UE cannot obtain LTE MBSFN configuration and CRS muting information from cell identification and PBCH decoding alone.

At a minimum, the network needs to indicate the LTE presence information to the UE, where the presence information includes presence of LTE cell, MBSFN configuration and CRS muting information.

Observation #5: At a minimum the network needs to indicate LTE presence information to the UE that includes presence of LTE cell, MBSFN configuration and CRS muting information.

In RAN4#101-e and RAN#93-e companies suggested that the UE can get all the information for CRS-IM by inter-RAT measurements. To perform inter-RAT measurements, the UE must be configured with LTE MO in the first place.

Observation #6: For the UE to blindly detect the LTE interfering cell(s) LTE-MO for inter-RAT measurements for the UE.

The purpose of LTE MO and inter-RAT measurements in our understanding is for the network to be able to handover the UE if necessary to an LTE cell. Also, the inter-RAT measurements would be made during the measurement gaps configured to the UE. Configuring measurement gaps would decrease the overlap NR throughput.

Observation #7: UE performs inter-RAT LTE measurements when LTE MO is configured and during measurement gaps.

Observation #8: Measurement gaps decrease the overall NR throughput.

In good network conditions for NR, we don’t see the necessity of LTE MO or inter-RAT measurements being configured to the UE which would just be an overhead to the UE and further impact the achievable throughout. The network might not configure the UE for inter-RAT measurements, which might make it even more challenging for the UE to detect the LTE interference cells.

Observation #9: In good network conditions there is no necessity for LTE MO and inter-RAT measurements and might not be configured to the UE.

Observation #10: Without LTE MO configured it would be very complex for the UE to detect the LTE interfering cells.

Based on the observations above, we don’t think it is always feasible for the UE to obtain the information on interference LTE location and sequence from inter-RAT measurements. We recommend that RAN4 agrees on introducing network assistance and further discusses what assistance information should be included.

Observation #11: It is not always feasible for the UE to obtain the information on interference LTE location and sequence from inter-RAT measurements.

Proposal #1: RAN4 agrees to introduce network assistance for CRS-IM.

On the information to be included as network assistance, the presence, location and sequence information should be made available to the UE via RRC configuration for CRS-IM.

Proposal #2: UE is provided with LTE presence, location and sequence as assistance information for CRS-IM.

CRS-IM is an advanced UE feature and needs additional UE capability signalling. Since the UE needs to support this feature only in bands overlapping with LTE or LTE re-farmed bands, we should introduce this as a per band capability.

Proposal #3: Introduce UE capability signalling for CRS-IM as a per band capability.

# 3. Conclusion

In this paper, we present our views on network assistance for CRS-IM. Our observations and proposals are captured below:

Observation #1: Impact of network assistance cannot be quantified by numerical results as they don’t capture impact to UE complexity, processing and power consumption.

Observation #2: For both LLR weighting and CRS-IC we need LTE/CRS presence, location and sequence information.

Observation #3: UE needs to detect the LTE interfering cell and perform PBCH decoding to obtain CRS location and sequence information if this information is not provided via network assistance.

Observation #4: UE cannot obtain LTE MBSFN configuration and CRS muting information from cell identification and PBCH decoding alone.

Observation #5: At a minimum the network needs to indicate LTE presence information to the UE that includes presence of LTE cell, MBSFN configuration and CRS muting information.

Observation #6: For the UE to blindly detect the LTE interfering cell(s) LTE-MO for inter-RAT measurements for the UE.

Observation #7: UE performs inter-RAT LTE measurements when LTE MO is configured and during measurement gaps.

Observation #8: Measurement gaps decrease the overall NR throughput.

Observation #9: In good network conditions there is no necessity for LTE MO and inter-RAT measurements and might not be configured to the UE.

Observation #10: Without LTE MO configured it would be very complex for the UE to detect the LTE interfering cells.

Observation #11: It is not always feasible for the UE to obtain the information on interference LTE location and sequence from inter-RAT measurements.

Proposal #1: RAN4 agrees to introduce network assistance for CRS-IM.

Proposal #2: UE is provided with LTE presence, location and sequence as assistance information for CRS-IM.

Proposal #3: Introduce UE capability signalling for CRS-IM as a per band capability.

# Reference

- R4-2115740, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom.

- RP-212636, “Revised WID: Further enhancement on NR demodulation performance”, China Telecom.

- TR 38.833, “Further enhancement on NR demodulation performance”.