3GPP TSG-RAN4 Meeting #101-e	R4-2117446

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.10.2.1

Source:	Apple

Title:	Discussion on SRS antenna port switching

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement for SRS antenna port switching has been discussed and the conclusions were captured in the WF[1]. However, there are couple of open issues from last meeting, and in this contribution, we continue discussing the RRM requirement for SRS antenna port switching.

# Scope of SRS antenna switching requirement

In last meeting, some agreements on scope of SRS antenna switching requirement were (open issues are highlighted in yellow),

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching - Agreements: - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 - Performance degradation on these symbols can be expected - FFS whether and how to capture this in TS 38.133 Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration - FFS - Option 1 (CATT): - Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission. - Option 2 (Nokia, Ericsson): - RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted NOT in the same slot, or - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. - Option 3 (Apple, Xiaomi, Huawei, QC, vivo, CATT): - RAN4 to define a generic requirement regardless of SRS resource configuration. - Option 3a: define interruption requirements for a limited set of SRS configurations.

In TS38.101-1, it was defined that the transient period of 15 us applies before and after SRS transmission to all the transmit CCs in CA with the CC sounding SRS, and UE RF requirements do not apply during this transient period. In the last meeting, it was agreed that “do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 and performance degradation on these symbols can be expected”. Since scheduling restriction itself could be represented by RF clarification, we don’t see strong rationale to clarify such performance degradation in RRM spec either.

Proposal 1: No need to specify performance degradation on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in TS38.133.

Since in last meeting we also proposed to use the whole 6 symbols for the minimum requirement design of SRS antenna port switching, a generic requirement could be applied regardless of consecutive or non-consecutive SRS transmission in slot.

Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration.

# Impact of SRS antenna port switching to other requirements

In last meeting, some agreements on impact of SRS antenna port switching to other requirements were (open issues are highlighted in yellow),

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA - Agreement in 2nd round: - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol. - FFS when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC - Agreements in 2nd round - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC - Agreements in 2nd round - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements - Agreements in 2nd round - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. - FFS whether spec clarification/note is needed to reflect the above agreements in the corresponding requirement section. Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS - FFS - Option 1 (CATT): If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification. - Option 2 (Apple, QC, OPPO, vivo): Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed. - Option 3 (vivo): Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation. - Option 3a (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. - Option 4 (Huawei, Ericsson, Nokia, CATT): Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

In last meeting, there was no conclusion on prioritization when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. In table 5.2.1.4-1 in TS38.214, RAN1 specified the relation between CSI reporting and DL CSI-RS as following table,

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

And in TS38.214 section 6.2.1.3, the prioritization for CSI reporting and SRS carrier switching is summarized as below,

| PUCCH/PUSCH for CSI (L1-RSRP/L1-SINR) reporting | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI | AP SRS | AP SRS |
| AP CSI | AP SRS | AP SRS |
| AP CSI | P/SP SRS | AP CSI |
| P/SP CSI | P/SP SRS | P/SP SRS |

Since this prioritization is defined between two colliding UE activities, we don’t need to consider CSI reporting type for the scenario when CSI measurement is colliding with SRS antenna port switching. Regarding the CSI reporting vs. SRS case, RAN1 had clear definition in TS38.214 and there is no need to revisit those conclusions.

We think the CSI reporting priority could be reused for CSI measurement priority, and SRS carrier switching prioritization rule could also be reused for SRS antenna port switching prioritization. Thus, UE is not required to perform SRS antenna port switching when P/SP SRS resource and the AP L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, SRS antenna port switching shall be prioritized, as summarized in the following table.

Table 1. Prioritization between L1-RSRP/L1-SINR measurement and SRS antenna port switching

| CSI-RS and SSB for L1-RSRP/L1-SINR | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI-RS and SSB | AP SRS | AP SRS |
| AP CSI-RS | AP SRS | AP SRS |
| AP CSI-RS | P/SP SRS | AP CSI |
| P/SP CSI-RS and SSB | P/SP SRS | P/SP SRS |

Proposal 3:

- For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

- For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized.

Regarding the impact of SRS antenna port switching to other RRM requirement (e.g., handover, BWP switching, SCell activation/deactivation), we think the same methodology in LTE spec could be re-used. In LTE TS36.133, RAN4 has added some clarification in the impacted RRM requirements, e.g.,

So, we could clarify the relaxation/applicability in those RRM requirements whose procedure could be impacted by SRS antenna port switching, e.g., the delay requirement could be extended, or the requirement only applies when SRS antenna port switching is not colliding with the reference signal. How to coordinate between SRS antenna port switching and those UE activities could be left to UE implementation, and therefore RAN4 only needs to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those activities. To make the requirement applicability clear, we prefer to capture such applicability clarification into the corresponding requirement section in TS38.133.

Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

Regarding impact of SRS antenna port switching to CSF and other RS, we think network shall control to avoid such collision for serving cell, but if such collision happens as error case, no UE requirement shall be imposed. However, it’s not necessary to define such error cases in the RAN4 spec since it shall be covered in RAN1.

Proposal 5: RAN4 to not define any solution and requirement for SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report.

# Interruption requirement applicability

In last meeting, some agreements on interruption requirement applicability were (open issues are highlighted in yellow),

Issue 1-3-1: Interruption requirement applicability Discussion in GTW (Aug 17): - Chair: Common understanding that txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. No common understanding if interruptions should be limited to one direction (DL or UL) or both DL and UL. Recommend to continue discussion. - Agreements in 2nd round - According to RAN2 capability definition, txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. If any issue is identified, this conclusion could be revisited. Issue 1-3-2: Interruption requirement for UE with or without per-FR MG capability - Agreements in 2nd round - No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. - But the requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”. Issue 1-3-3: Would the interruption requirement differentiate between sync and async cases? - Agreements in 2nd round - RAN4 to specify interruption requirement design for async case first - FFS on whether and how to specify interruption requirement design for sync case Issue 1-3-4: txSwitchImpactToRx for intra-band case - FFS: - Option 1: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. - Option 2: Clarify that For intra-band case, the interruption always applied.

Regarding issue 1-3-3, we need to consider the TA between DL and UL as well as the time difference between CCs in CA(MTTD/MRTD). Regarding the different CA scenarios, in R16 carrier based SRS switching, we have conclusion in R4-2012155 that,

- Interruption requirements for CA case 1, case 2 and case 3

  - The same interruption requirements as for async case in the spec shall apply

Where,

  - Case 1: CA is co-location deployed

  - Case 2: Single TAG CA, or carriers in the same TAG for multiple TAG CA

  - Case 3: uplink time difference does not exceed a threshold X

    - X = [5] us

So here, we think the same principle shall be applied since sync and async may have the same interruption.

Proposal 6: No need to further discuss the minimum interruption requirements for sync cases.

Regarding issue 1-3-4, we did not see the rationale to have such clarification in RAN4 since the definition of txSwitchImpactToRx in TS38.306 doesn’t differentiate intra-band contiguous or non-contiguous CA case.

Proposal 7: No need to differentiate txSwitchImpactToRx applicability for intra-band contiguous CA and intra-band non-contiguous CA case.

# Interruption requirement design for SRS antenna port switching

In last meeting, some agreements on interruption requirement design were (open issues are highlighted in yellow),

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level - FFS - Option 1 (CATT, Apple, vivo, QC, Xiaomi, MTK, Intel, HW, OPPO): based on slot level - Option 1a (CATT): The interruption requirements will be defined based on slot level, i.e. no requirement for the case of interruption on flexible slot. It is not necessary to further discuss on components of making interruption if the interruption requirements are defined based on slot level. - Option 1b (vivo): The interruption requirement is preferred to be defined based on slot level. The mis-alignment in the UL frame boundary between anchor and victim cells needs to be further discussed. - Option 1c(LG): Slot based interruption requirement could be used in synchronous and asynchronous case, and only UL symbols based interruption should be defined when SRS antenna port switching is configured in the flexible slot in synchronous case. - Option 2 (Ericsson, Nokia): based on symbol level - Option 2a (Nokia): The transient period before and after the SRS resource will cause UL interruption of 1 OFDM symbol at 60kHz. At 15kHz or 30kHz SCS, RAN4 should discuss if and how to capture the transient periods from RF session in the RRM specifications. Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1 - FFS - Option 1 (CATT, Apple, QC, Xiaomi, CMCC, OPPO, HW, MTK, Intel): The composites of interruption requirement for SRS antenna port switching in FR1 include: - SRS Transmission time (use 6 symbols as minimum requirement), and - Antenna switching time before and after SRS transmission occasion (2*15us). - Option 2 (vivo): - For interruption requirements, the interruption time is preferred to include power adjustment period, antenna switching time and SRS transmission time. - RAN4 can consider to define interruption requirements for a limited set of SRS configuration in R17. - Option 3 (LG): The composites of interruption requirement for SRS antenna port switching in FR1 include: - Antenna port switching time (15usec) before the first SRS transmission - Time from the first SRS transmission to the last SRS transmission - Antenna port switching back time (15usec) after the last SRS transmission - It should be excluded when the SRS resource is configured in the last symbol of a slot and the next slot is downlink - Option 4 (Nokia, Ericsson): - When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission period. - When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission periods and the guard period in-between. - The position of the transient period needs to be clarified before defining the interruption and/or the scheduling restriction in RRM spec. - Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured. - Option 5 (Ericsson): The components within the interruption requirement shall include SRS transmission time for the configured number of symbols, as well as SRS antenna switching time before and after the SRS transmission. Issue 1-4-3: Interruption requirement proposals - FFS on the options in summary R4-2115395. Up to conclusions from other issues.

The guard period in RAN1 TS38.214 is defined as below,

In our understanding, RAN1 has chosen to use the most conservative way to define the guard period and no signals would be expected to transmit in guard period. We propose to use the RAN1 guard period as baseline to determine the interruption requirement. However, similar as interruption requirements for SRS carrier-based switching, even though the GP is at symbol level, but the interruption requirement shall still be defined in unit of slot for NR and in unit of subframe for LTE. In last meeting, some companies thought the interrupting requirement may be defined in unit of symbols since transient time for SRS antenna port switching is 15us from RF spec, but we have different understanding: with the MTTD/MRTD and TA, we cannot precisely determine which DL symbols or UL symbols on other CCs are interrupted by SRS antenna port switching, and therefore using slot level interruption requirement would be a reasonable solution.

Proposal 8: Interruption requirement of SRS antenna port switching shall be defined based on slot level.

Regarding the components within interruption time of SRS antenna port switching in FR1, the following figure illustrates the SRS antenna port switching in a UL slot. We think the interruption requirement shall apply for all possible switching patterns, and therefore the minimum requirement in the worst case shall consider following components in the SRS antenna port switching:

- Antenna port switching time between SRS symbols in slot (transient time)

- SRS transmission time

- Transient time before and after SRS transmission occasion (15us as shown in figure 1)

Figure 1. Illustration of SRS antenna port switching in a uplink slot

In order to converge the options, we think 6 symbol can be used as the longest time duration containing SRS symbols and guard symbols to define an unified minimum requirements for different SRS antenna switch patterns. Even though some SRS resource sets for antenna switching may take two slots (one slot can only be used for 3 SRS symbols if those 3 SRS symbols are with different antenna port), RAN4 does not need to specialize this case and the interruption could be checked slot by slot. For instance, in T1R4 case, UE may use 2 slots for 4 times of SRS antenna port switching, but the interruption requirement in each slot could be same, and we only need to determine the interruption length in each slot. Thus, we propose,

Proposal 9: The composites of interruption requirement for SRS antenna port switching in FR1 include:

- SRS Transmission time (use 6 symbols as minimum requirement), and

- Antenna switching time before and after SRS transmission occasion (2*15us).

Based on all of the above analysis, the interruption requirement could be defined as below,

Proposal 10: the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

# Miscellaneous issues

In last meeting, some agreements on other miscellaneous issues were (open issues are highlighted in yellow),

Issue 1-5-1: Network to obtain the interrupted symbol info - Agreements in 2nd round - No need to discuss this issue in R17 FeRRM WI. Issue 1-5-2: Interruption requirement related with prioritization rule in RAN1 - FFS - Option 1 (MTK, Apple, OPPO, vivo): - For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”. - Option 2 (Nokia, Ericsson) : - The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. - Option 3 (Huawei, Ericsson, CATT): - Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Issue 1-5-3: Two SRS colliding on same symbol - FFS - For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: - Option1 (Apple): Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected. - Option2: Up to UE implementation. - Option3 (Nokia): This is up to RAN1 discussion.

Regarding issue 1-5-2, we think RAN4 only need to define the interruption requirement based on the RAN4 RRM scenarios and don’t need to revisit any scenarios which have already been specified in RAN1 spec. In our understanding, option 1 and option 3 is quite similar from RRM perspective. We propose to only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1.

Proposal 11: RAN4 only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1.

Regarding issue 1-5-3, we agree with other companies’ view in last meeting that this is an issue covered by RAN1.

Proposal 12:

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- This is up to RAN1 discussion, and no need to discuss this case in RAN4.

# Conclusion

In this contribution we discuss the RRM requirements for SRS antenna port switching.

Proposal 1: No need to specify performance degradation on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in TS38.133.

Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration.

Proposal 3:

- For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

- For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized.

Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

Proposal 5: RAN4 to not define any solution and requirement for SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report.

Proposal 6: No need to further discuss the minimum interruption requirements for sync cases.

Proposal 7: No need to differentiate txSwitchImpactToRx applicability for intra-band contiguous CA and intra-band non-contiguous CA case.

Proposal 8: Interruption requirement of SRS antenna port switching shall be defined based on slot level.

Proposal 9: The composites of interruption requirement for SRS antenna port switching in FR1 include:

- SRS Transmission time (use 6 symbols as minimum requirement), and

- Antenna switching time before and after SRS transmission occasion (2*15us).

Proposal 10: the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

- Unit of interruption requirement is slot for NR and subframe for LTE.

Proposal 11: RAN4 only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1.

Proposal 12:

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- This is up to RAN1 discussion, and no need to discuss this case in RAN4.

# References

[1] R4-2115336, WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple, RAN4 #100-e meeting