3GPP TSG-RAN4 Meeting #101-e	R4-21xxxxx

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.20.3.2

Source:	Apple

Title:	Discussion on RRM requirement with eDRX for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement with eDRX for RedCap has been discussed and the conclusions were captured in the WF[1].

RAN4 also received an incoming LS [2] from RAN2:

Agreements: - eDRX feature is optional for any UE (including RedCap and non-RedCap UEs). - RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec. - RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec. - The maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s. - The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s. - When IDLE eDRX cycle is longer than 10.24s, PH calculation formula defined in LTE is re-used, i.e. PH_CN: H-SFN mod TeDRX,_CN,H= (UE_ID_H mod TeDRX_CN,H) - where TeDRX_CN,H is equal to IDLE eDRX cycle. - When IDLE eDRX cycle is longer than 10.24s, CN PTW_end calculation formula defined in LTE is re-used, i.e. PTW_end is radio frame satisfying SFN = (PTW_start + L*100 - 1) mod 1024, - where L is PTW length configured by upper layers. Working assumption: - When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 1024/N* ieDRX, where - ieDRX = floor(UE_ID_H /TeDRX,H) mod N - FFS N = 4 or 8, FFS if N can take other values.

And additional agreement in RAN2 meeting is:

- For RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle. - For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle. - For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle. - Introduce an additional new IE for INACTIVE eDRX to contain all values of INACTIVE eDRX cycles (also include values >10.24, if agreed in future). - For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle. - For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle. - eDRX feature is optional for any UE (including RedCap and non-RedCap UEs). Agreements via email - from offline 105 third round - eDRX is optional for any gNB (either supporting RedCap or not), which means it is up to gNB implementation whether to support eDRX Agreements online: - For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring: Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle. Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. - For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring: Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle. Option 2: T is determined by RAN paging cycle.

In SA2 LS S2-2106978, it was also clarified that,

SA2 would like to thank RAN2 for their LS on introducing extended DRX for RedCap UEs (S2-2105241/R2-2104374). SA2 would like to provide the following feedback to RAN2: - 5G System currently supports extended DRX in RRC/CM idle mode in the 5GC/NAS over E-UTRA/NB-IoT (since Rel-16) for eDRX cycles > 10.24s. SA2 has aligned with RAN2 request to support extended DRX in RRC/CM idle mode also for NR for eDRX cycles up to 10485.76 seconds. - For RRC Inactive, SA2 agreed to support eDRX of up to 10.24s. - Regarding eDRX extension beyond 10.24s for RRC Inactive, SA2 did not agree to support it in Rel-17 and instead will study potential solutions in Rel-18.

Since now we have concrete conclusions from RAN2 and SA2, we discuss the RRM requirement with eDRX for RedCap in this contribution.

# General aspects on eDRX enhancments

The remaining issues from last meeting are as below,

Issue 1-1-1: Scopes where RRM requirement with eDRX in IDLE/INACTIVE mode are defined - Agreement: - Only consider defining the RRM requirement with eDRX in IDLE/INACTIVE mode only. Issue 1-1-2: Whether prioritizing the eDRX requirements for FR1 and de-prioritizing the eDRX requirements for FR2. Agreement: - Agreements: - Define eDRX requirements for FR1 - FFS whether to define eDRX requirements for FR2 Issue 1-1-3: Whether have the eDRX requirements for FR1 and FR2 on separate tables. Agreement: No agreement. Open issues: Option 1: Yes (MTK Ericsson vivo) Option 2: No Option 3: FFS (Apple ZTE OPPO ,Huawei, Xiaomi vivo)

Regarding issue 1-1-2, we think both FR1 and FR2 are in the RAN2 discussion scope, and therefore we could define the eDRX requirement for both FR1 and FR2.

Proposal 1: RAN4 to define RedCap RRM requirement with eDRX for both FR1 and FR2.

Regarding the issue 1-1-3, we prefer to discuss the technical part first to understand how much the discrepancy is for FR1 and FR2; and then we could decide whether or not to separate FR1 and FR2 requirements into two tables.

Proposal 2: RAN4 to determine whether have the eDRX requirements for FR1 and FR2 on separate tables after reaching the technical conclusions on FR1 and FR2 eDRX requirement.

# Different methodologies based on eDRX length when defining performance requirements

The remaining issues from last meeting are as below,

Issue 1-2-1: Whether classify eDRX into different groups based on eDRX length when defining requirements - Agreement: - option 1 (classify eDRX into different groups based on eDRX length when defining requirements) Issue 1-2-2: If the answer of issue 1-2-1 is Yes, then the concrete groups Agreement: - No. Open issues: - Option 1: Three groups - 1: eDRX value 2.56s/5.12s/10.24s; 2: 10.24s<eDRX_cycle_length≤2621.44s; 3: 2621.44s<eDRX_cycle_length≤10485.76s (xiaomi ) - Option 2: Two groups - 1: eDRX cycle lengths up-to 10.24s 2: eDRX > 10.24s (QC ZTE OPPO MTK Huawei) - Option 3: FFS (Eric, Apple, vivo) Issue 1-2-3: If the answer of issue 1-2-1 is Yes, which category considers PTW and which category does not PTW Agreement: - No. Open issues: - Option 1: 1: Without PTW (eDRX length is up-to 10.24s) 2: With PTW (eDRX > 10.24s); (QC, xiaomi, OPPO, MTK, Huawei) - Option 2: 1: Without PTW (eDRX length is up-to 10.24s); 2: With PTW (10.24s<eDRX_cycle_length≤T1); 3: FFS T1<eDRX_cycle_length≤T2;Values of T1 and T2 are FFS (Ericsson, Apple, vivo)

Regarding issue 1-2-2, based on conclusion from RAN2 and SA2,

- Lower bound for eDRX configuration in RRC_IDLE and RRC_INACTIVE is 2.56 seconds.

- eDRX cycle in IDLE mode could be up to 10485.76 seconds

- eDRX cycle in Inactive mode could be up to 10.24 seconds

So IDLE mode UE could have eDRX range from 2.56s to 10485.76s while in Inactive mode UE could have eDRX range from 2.56s to 10.24s. Thus, for IDLE mode, classify eDRX into 2 groups based on eDRX length when defining requirements:

- Group 1: eDRX cycle lengths up-to 10.24s;  Group 2: 10485.76s≥eDRX > 10.24s.

For Inactive mode, only one group is needed when defining requirements:

- Group 1: eDRX cycle lengths up-to 10.24s.

Regarding issue 1-2-3, In WID [RP-211574] it states that “eDRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g., common set of eDRX values) between RRC Inactive and Idle”. RAN2 agreed that “Lower bound for eDRX configuration in RRC_IDLE and RRC_INACTIVE is 2.56 seconds”, that means when eDRX cycle= 2.56s/5.12s/10.24s, we don’t need to consider PTW and PH, and therefore the paging monitoring mechanism could be as same as legacy DRX case.

Proposal 3:

For IDLE mode, classify eDRX into 2 groups based on eDRX length when defining requirements:

- Group 1: eDRX cycle lengths up-to 10.24s;  Group 2: 10.24s<eDRX cycle≤10485.76s.

For Inactive mode, only one group is needed when defining requirements: eDRX cycle lengths up-to 10.24s.

Proposal 4:

If eDRX cycle length is up-to 10.24s, no need to consider PTW in RRM requirement.

If eDRX > 10.24s, RRM requirement needs to consider PTW.

# eDRX requirements for idle state

The remaining issues from last meeting are as below,

Issue 1-3-1: Design principles of LTE eDRX requirements Agreement: - No. Open issues: - Option 1: RAN4 to use LTE eDRX requirements approach as baseline when developing NR eDRX requirements. (Eric QC xiaomi OPPO MTK) - Option 2: For 10.24s<eDRX_cycle_length≤2621.44s, RAN4 to use LTE RRM requirement of UE categories other than Cat-M/Cat-NB in IDLE mode as baseline to define the RRM requirement for RedCap UE in IDLE and INACTIVE mode. For 2621.44s<eDRX_cycle_length≤10485.76s, FFS on RRM requirement for RedCap UE in IDLE and INACTIVE mode (Apple vivo) - Option 3: discuss eDRX enhancements in RAN4 after the final conclusions on eDRX configurations are made in RAN2 () - Option 4: FFS () - Option 5(Huawei): No need to discuss using which requirements as a baseline as redcap UE is a mixed configuration of LTE edrx, eMTC and NB () Issue 1-3-2: How to determine PTW length Agreement: - No. Open issues: - Option 1: Use the same PTW lengths as in LTE for eDRX as baseline for eDRX cycle lengths greater than 10.24s. (Eric MTK) - Option 1a: Propose to use scaling factors. - Option 1b: No need to use scaling factors. - Option 2: FFS (Apple QC xiaomi OPPO vivo, Huawei) Issue 1-3-3: Whether to define for transition between different states Agreement: - No. Open issues: - Option 1: Define the requirements for transition between different states (states need to be identified) (Apple OPPO vivo) - Option 2: No - Option 3: FFS (Eric QC xiaomi MTK) Issue 1-3-4: Assumptions on measurements on eDRX with PTW Agreement: - No. Open issues: - Option 1: When eDRX is configured for Redcap, the assumption that all measurements are performed within PTW of an eDRX cycle in LTE, can still be reused for NR (Eric vivo) - Option 1a: When eDRX is used, the number of samples needed for Nserv in 5G NR (measured in DRX cycles) must be achieved in a single PTW.(MTK) - Option 2: FFS (Apple QC xiaomi OPPO Huawei) Issue 1-3-5: Nserv value Agreement: - No. Open issues: - Option 1: Concrete requirements from R4-2113956 (MTK) - Option 3: The legacy DRX requirements could be reused for the new 2.56s eDRX cycle. - Option 4: FFS (Eric Apple QC xiaomi OPPO vivo Huawei) Issue 1-3-6: Cell reselection requirements for RedCap UE with eDRX cycle (intra frequency) Agreement: - No. Open issues: - Option 1:FFS Issue 1-3-7: Cell reselection requirements for RedCap UE with eDRX cycle (inter frequency) Agreement: - No. Open issues: - Option 1: FFS

Regarding issue 1-3-1, since the RedCap eDRX requirement includes both FR1 and FR2 and different eDRX range in legacy LTE, MTC and NB-LTE has different requirements, it’shard to say which one could be used as baseline for RedCap. We agree to directly discuss the RedCap requirement without this issue unless the principle is straightforward to all companies.

Proposal 5: No need to discuss the design principles of LTE eDRX requirements.

Regarding issue 1-3-2, since RAN2 agreed that “The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s.” So the 1.28s shall be used as granularity to define the PTW length in RRM requirement. LTE method could be reused in this case, which means PTW lengths is specified in RRM requirement with unit of 1.28s.

Proposal 6: PTW lengths are specified in RRM requirement with unit of 1.28s.

Regarding issue 1-3-3, in current LTE spec, the transition requirement was defined as below,

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

So similar requirement could also be defined for RedCap UE with eDRX.

Proposal 7: Define the requirements for transition between different states:

When the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

Regarding issue 1-3-4, in LTE IDLE mode, the serving cell measurement requirement with eDRX for legacy UE and NB-IoT UE was specified as,

Both requirements assumed the serving cell measurement period (samples for PHY averaging could) shall be contained in a single PTW length. We think it also makes sense to use this methodology for RedCap requirement for serving cell measurement in IDLE. For FR2, the beam sweeping also needs to be contained in the same PTW for deriving the proper measurement result to deliver to high layer.

Proposal 8: For both FR1 and FR2, when eDRX>10.24s is used at NR RedCap UE in IDLE mode, the number of samples needed for Nserv of serving cell measurement (measured in DRX cycles) must be contained in a single PTW length.

Proposal 9: For both FR1 and FR2, the lower bound of PTW length in serving cell measurement of IDLE mode is Nserv*DRX_cycle1.28*1.28s.

Regarding the issue 1-3-5, when DRX≤10.24 is used, the requirement could be specified like legacy DRX case with Nserv=2.

For FR2, the beam sweeping factor for 2.56s≤eDRX≤10.24s is assumed to be 3 for balance between mobility performance and power consumption. However, when eDRX>10.24s and PTW is used, we may need to still assume the beam sweeping factor based on the DRX cycle used in PTW window, otherwise, the PTW window cannot contain sufficient samples for UE to complete the beam sweeping and PHY averaging.

For instance, when DRX is 0.32s in PTW, if N1=3 is used as beam sweeping factor and sample number per beam is 2; so the Nserv*DRX_cycle=3*2*0.32=1.92s, and the PTW length could be 2*1.28s=2.56s to cover the whole Nserv*DRX_cycle. However, if SSB periodicity is 160ms in this case and UE actually has 8 local Rx beams for L3 measurement, within 2.56s, UE only have 16 SSB occasions for serving cell measurement. Even though UE may wake up every 160ms for measurement with the best effort, that means all the samples could only be used for serving cell measurement (2*8) but there is no way to monitor paging if it’s FDM with SSB. Same problem applies for neighbor cell measurement.

Thus, we propose for FR2 serving and neighboir cell measurement that,

Proposal 10: for FR2 IDLE mode,

When 2.56s≤eDRX≤10.24s without PTW, the beam sweeping factor N1 is assumed as 3 for serving cell and intra-freq/inter-freq neighbor cell measurement.

When 10.24<eDRX with PTW, the beam sweeping factor N1 is assumed as same as the ones used for normal DRX cycles (e.g., N1=8 for DRX_cycle=0.32s, and N1=5 for DRX_cycle=0.64s) for serving cell and intra-freq/inter-freq neighbor cell measurement.

Proposal 11: the serving cell measurement requirement with eDRX for RedCap UE in IDLE mode is:

| eDRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of eDRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 2.56 | 1 | 3 | N1*2 |
| 5.12 |  |  |  |
| 10.24 |  |  |  |

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥Nserv*DRX_cycle1.28*1.28 ( Nserv*DRX_cycle1.28) | 1 | 8 | N1*2 |
|  | 0.64 |  |  | 5 | N1*2 |
|  | 1.28 |  |  | 4 | N1*2 |
|  | 2.56 |  |  | 3 | N1*2 |

Regarding issue 1-3-6 and issue 1-3-7, the similar methodology could be used for intra-freq and inter-freq measurements. Like in LTE requirement for intra-freq and inter-freq measurement with eDRX, the measurement and evaluation period shall be contained in one single PTW but cell detection period could be contained in different PTWs.

Proposal 12: For both FR1 and FR2, when eDRX>10.24s is used at NR RedCap UE in IDLE mode,

- the number of samples needed for Tmeasure,NR /Tevaluate,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) must be contained in a single PTW length,

- but the number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) could be splitted into different PTWs.

Since measurement is somehow contained in evaluation period, that means normally the number of samples needed for evaluation is greater than the number of samples needed for measurement. So, following is proposed.

Proposal 13: For both FR1 and FR2,

- the lower bound of PTW length in intra-frequency cell measurement of IDLE mode is Tevaluate, NR_Intra1.28*1.28s.

- the lower bound of PTW length in inter-frequency cell measurement of IDLE mode is Tevaluate, NR_Inter1.28*1.28s.

Similar as serving cell measurement, when DRX≤10.24 is used, the samples of intra-frequency and inter-frequency measurement requirement could be specified like legacy 2.56s DRX case. We also think it makes sense to mostly reuse the requirement from LTE NB eDRX, e.g., sample number for cell measurement, evaluation and detection. But the time period shall not be shorter than legacy DRX case, e.g., in eDRX=2.56, we still use 23 as the sample number for detection. Based on proposal 10 for FR2 case, the requirement for intra-frequency and inter-frequency measurement with eDRX is defined as below.

Proposal 14: the intra-frequency cell measurement requirement with eDRX for RedCap UE in IDLE mode is:

| eDRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 2.56 | 1 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 |  |  | 102.4 x N1 (20 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  |  | 102.4 x N1 (10 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |  |  |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 |  | 1 | 8 | eDRX_cycle_length* 23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥Tevaluate, NR_Intra1.28*1.28s (Tevaluate, NR_Intra1.28) |  | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 |  |  | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 |  |  | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 15: the inter-frequency cell measurement requirement with eDRX for RedCap UE in IDLE mode is same as intra-frequency case in proposal 14.

# eDRX requirements for inactive state

The remaining issues from last meeting are as below,

Issue 1-4-1: whether inactive eDRX requirements can base on the corresponding idle state eDRX requirements Agreement: - No. Open issues: - Option 1: RAN4 to take the eDRX requirements for idle mode as baseline for inactive mode.(Apple, Xiaomi, MTK) - Option 2: For the inactive performance requirements, whether reuse idle state requirements or not should be determined. - Option 3: FFS (Eric, QC OPPO vivo MTK) - Option 4(Huawei) : RedCap UE in inactive mode shall monitor both CN paging and RAN Paging. - If the extended DRX for RRC_INACTIVE is up to 10.24s, then no PTW for RRC_INACTIVE, the requirements can be defined only based on inactive state eDRX. - If the extended DRX for RRC_INACTIVE can be larger than 10.24s, two PTW (one for RRC-IDLE and the other for RRC_INACTIVE) can be present at the same time. This scenario is complex and how to define the measurement needs further discussion. Issue 1-4-2: Behavior when Redcap UE has both RRC_IDLE eDRX and RRC_INACTIVE eDRX configuration Agreement: - No. Open issues: - Option 1: If the extended DRX for RRC-Inactive is up to 10.24s which depends on CT1/SA2 conclusion, the cell reselection requirements can be only based on monitoring of RAN initiated POs; If the extended DRX for RRC_INACTIVE is larger than 10.24s, two PTW can be present at the same time. How to define the measurement needs further discussion. (Huawei) - Option 2: FFS (Eric MTK) - Option 3: more input from RAN2 (Apple QC Xiaomi OPPO vivo MTK)

Based on conclusion in RAN2 and SA2, in Inactive mode UE can only use eDRX up to 10.24s and no PTW shall be considered in this case. RAN2 had following agreement in last meeting:

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle.

Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle.

Option 2: T is determined by RAN paging cycle.

Table 1. Summary of RAN2 agreement for paging monitoring cycle T

| IDLE eDRX[s] | Inactive eDRX[s] | Outside CN PTW or during CN PTW | T |
| --- | --- | --- | --- |
| >10.24 | Not configured | During CN PTW | Shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle |
| >10.24 | Not configured | Outside CN PTW | FFS in RAN2 |
| >10.24 | ≤10.24 | During CN PTW | Shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle. |
| >10.24 | ≤10.24 | Outside CN PTW | INACTIVE eDRX cycle |
| ≤10.24 | Not configured | NA | FFS in RAN2 |
| ≤10.24 | ≤10.24 | NA | Shortest of IDLE eDRX cycle and INACTIVE eDRX cycle; i.e., INACTIVE eDRX cycle |

Regarding issue 1-4-1, we think the sample number used in IDLE mode requirement with eDRX<=10.24 could be reused for inactive mode requirement.

Proposal 16: RAN4 to take the sample number of eDRX requirements for idle mode as baseline for inactive mode.

However, the measurement cycle for inactive mode requirement with eDRX shall be specified based on the RAN2 agreements summarized in table 1.

Proposal 17: the measurement cycle for inactive mode requirement with eDRX shall be specified based on the paging monitoring cycle of T from RAN2 agreements summarized in table 1.

# Conclusion

We discuss the RRM requirement with eDRX for RedCap in this contribution.

Proposal 1: RAN4 to define RedCap RRM requirement with eDRX for both FR1 and FR2.

Proposal 2: RAN4 to determine whether have the eDRX requirements for FR1 and FR2 on separate tables after reaching the technical conclusions on FR1 and FR2 eDRX requirement.

Proposal 3:

For IDLE mode, classify eDRX into 2 groups based on eDRX length when defining requirements:

- Group 1: eDRX cycle lengths up-to 10.24s;  Group 2: 10.24s<eDRX cycle≤10485.76s.

For Inactive mode, only one group is needed when defining requirements: eDRX cycle lengths up-to 10.24s.

Proposal 4:

If eDRX cycle length is up-to 10.24s, no need to consider PTW in RRM requirement.

If eDRX > 10.24s, RRM requirement needs to consider PTW.

Proposal 5: No need to discuss the design principles of LTE eDRX requirements.

Proposal 6: PTW lengths are specified in RRM requirement with unit of 1.28s.

Proposal 7: Define the requirements for transition between different states:

When the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

Proposal 8: For both FR1 and FR2, when eDRX>10.24s is used at NR RedCap UE in IDLE mode, the number of samples needed for Nserv of serving cell measurement (measured in DRX cycles) must be contained in a single PTW length.

Proposal 9: For both FR1 and FR2, the lower bound of PTW length in serving cell measurement of IDLE mode is Nserv*DRX_cycle1.28*1.28s.

Proposal 10: for FR2 IDLE mode,

When 2.56s≤eDRX≤10.24s without PTW, the beam sweeping factor N1 is assumed as 3 for serving cell and intra-freq/inter-freq neighbor cell measurement.

When 10.24<eDRX with PTW, the beam sweeping factor N1 is assumed as same as the ones used for normal DRX cycles (e.g., N1=8 for DRX_cycle=0.32s, and N1=5 for DRX_cycle=0.64s) for serving cell and intra-freq/inter-freq neighbor cell measurement.

Proposal 11: the serving cell measurement requirement with eDRX for RedCap UE in IDLE mode is:

| eDRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of eDRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 2.56 | 1 | 3 | N1*2 |
| 5.12 |  |  |  |
| 10.24 |  |  |  |

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥Nserv*DRX_cycle1.28*1.28 ( Nserv*DRX_cycle1.28) | 1 | 8 | N1*2 |
|  | 0.64 |  |  | 5 | N1*2 |
|  | 1.28 |  |  | 4 | N1*2 |
|  | 2.56 |  |  | 3 | N1*2 |

Proposal 12: For both FR1 and FR2, when eDRX>10.24s is used at NR RedCap UE in IDLE mode,

- the number of samples needed for Tmeasure,NR /Tevaluate,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) must be contained in a single PTW length,

- but the number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) could be splitted into different PTWs.

Proposal 13: For both FR1 and FR2,

- the lower bound of PTW length in intra-frequency cell measurement of IDLE mode is Tevaluate, NR_Intra1.28*1.28s.

- the lower bound of PTW length in inter-frequency cell measurement of IDLE mode is Tevaluate, NR_Inter1.28*1.28s.

Proposal 14: the intra-frequency cell measurement requirement with eDRX for RedCap UE in IDLE mode is:

| eDRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 2.56 | 1 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 |  |  | 102.4 x N1 (20 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  |  | 102.4 x N1 (10 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |  |  |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 |  | 1 | 8 | eDRX_cycle_length* 23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥Tevaluate, NR_Intra1.28*1.28s (Tevaluate, NR_Intra1.28) |  | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 |  |  | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 |  |  | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 15: the inter-frequency cell measurement requirement with eDRX for RedCap UE in IDLE mode is same as intra-frequency case in proposal 14.

Proposal 16: RAN4 to take the sample number of eDRX requirements for idle mode as baseline for inactive mode.

Proposal 17: the measurement cycle for inactive mode requirement with eDRX shall be specified based on the paging monitoring cycle of T from RAN2 agreements summarized in table 1.

| IDLE eDRX[s] | Inactive eDRX[s] | Outside CN PTW or during CN PTW | T |
| --- | --- | --- | --- |
| >10.24 | Not configured | During CN PTW | Shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle |
| >10.24 | Not configured | Outside CN PTW | FFS in RAN2 |
| >10.24 | ≤10.24 | During CN PTW | Shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle. |
| >10.24 | ≤10.24 | Outside CN PTW | INACTIVE eDRX cycle |
| ≤10.24 | Not configured | NA | FFS in RAN2 |
| ≤10.24 | ≤10.24 | NA | Shortest of IDLE eDRX cycle and INACTIVE eDRX cycle; i.e., INACTIVE eDRX cycle |

# References

[1] R4-2115364, WF on further RRM enhancement for NR and MR-DC, vivo, RAN4 #100e

[2] R2-2109196, LS to RAN4 on eDRX, vivo, RAN2 #115e