3GPP TSG-RAN4 Meeting #101-e	R4-2117453

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.20.3.3

Source:	Apple

Title:	Discussion on RRM relaxation requirement for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM relaxation for RedCap has been discussed and the conclusions were captured in the WF[1].

Issue 2-1-1: whether relaxation methods for IDLE/INACTIVE states should be postponed until more progress in reached in RAN2 or not - Agreement: - Do not postpone. Issue 2-1-2: priority issue - Agreement: - Regarding RAN2’s progress, RAN4 starts discussion from stationary and not-cell-edge relaxation criterion for RRC_IDLE/INACTIVE as high priority Issue 2-1-3: Mechanism for R17 RedCap UE in IDLE/Inactive mode - Agreements: - Reuse the same mechanism as Rel-16 RRM measurement relaxation in IDLE/Inactive mode - Method 1: Using scaling factors (value for FFS) - Method 2: Using a period of time (value for FFS) - Criteria for using Method 1 and/or 2 are FFS - Note: EMR is not supported by RedCap UEs and the relaxation mechanisms related to EMR carriers do not apply to RedCap

RAN2 also sent one LS[2] to RAN4 for the agreements of RRM relaxation in last meeting, duplicated as below.

Agreements in RAN2#113bis-e: - At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. - Network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. FFS whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed. Agreements in RAN2#114-e: - An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive). - Subscription based relaxation criteria will not be considered in Rel-17 RRM relaxation. - Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2). - Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17: When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs: - Option 1) UE performs Rel-17 RRM relaxation method - Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation - When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration. Agreements in RAN2#115-e: - Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs. - Do not introduce beam change based criterion in Rel-17. - The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED. - Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one.

In this contribution, we further discuss the RRM relaxation requirement for RedCap.

# RRM measurement relaxation for Redcap

Based on the following agreements from RAN2,

- When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration.

- Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one.

It means: if RedCap UE is configured with RRM relaxation criteria in R17, only two scenarios shall be considered:

(1)R17 stationary criteria only, or

(2)R17 stationary criteria + R17 not-at-cell-edge criteria

Proposal 1: RAN4 only needs to consider two scenarios for R17 RedCap RRM relaxation in IDLE/inactive mode:

(1)configured R17 stationary criteria only, or

(2)configured both R17 stationary criteria and R17 not-at-cell-edge criteria

Regarding IDLE/Inactive mode RRM relaxation, RAN2 agreed that network can configure R17 stationarity criterion/criteria together with a R17 not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. We think the RRM requirement delay requirement relaxation in RAN4 could be similar as R16 power saving WI.

In R16 power saving WI, when one of the criterion is fulfilled, UE would have a scaling factor K1/K2 to relax the RRM requirement, and if both of the criterion is fulfilled, RAN4 has a relative long fix value for RRM requirement interval, duplicated as below.

Proposal 2: except EMR, RAN4 to reuse the same mechanism of R16 RRM relaxation in power saving WI for R17 RedCap UE in IDLE/Inactive mode, i.e.,:

- if single criteria (stationarity or not-at-cell-edge) is fulfilled, use K1_RedCap /K2_RedCap to relax the RRM requirement. K1_RedCap /K2_RedCap has the similar applicability condition as K1/K2 in R16 power saving WI, and FFS on the values for K1_RedCap /K2_RedCap, e.g., K1_RedCap =3 and K2_RedCap =60.

- if both criterion (stationarity and not-at-cell-edge) are configured and fulfilled, use a fixed long measurement period for requirement relaxation, e.g., like 1 hour in R16 power saving WI.

Regarding RRC connected mode RRM relaxation, RAN2 still has many open issues (reporting configuration, or relaxation mechanism based on network reconfiguration or not) to discuss and RAN4 shall wait more conclusions from RAN2 before going to the requirement design. In the RAN2 LS it also mentioned that “For RRC_CONNECTED, RAN2 will continue discussing the details”.

However, besides the conclusion from last RAN2 meeting, we think in connected mode, when the criterion is met, this is reported to the network; and when the criterion turns to be “not met”, this is also reported to the network since network needs to know when UE shall quit the RRM relaxation mode.

Proposal 3: For RedCap RRM relaxation in RRC connected mode, RAN4 to defer requirement design until sufficient conclusions are made in RAN2.

Proposal 4: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network.

# Other aspects regarding RRM measurement relaxation for Redcap

The remaining issue from last meeting is:

Issue 2-3-2: Requirements when UE moves between different states Agreement: - No. Open issues: - Option 1: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network. () - Option 2: RAN4 to consider minimum requirements at transitions for different criterions switching (Apple OPPO); - Option 3: RAN4 to discuss assumptions how to evaluate RRM measurement relaxation. A distinction into RRC states and neighbour cell types may serve well in this context. () - Option 4: FFS (QC MTK Huawei) - Option 5: Option 1 is RAN2 issue () - Option 6: RAN4 to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes based on same transition principle in release 16 relaxation requirements in IDLE/INACTIE states. Whether to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes in CONNECTED state is FFS (Eric Apple QC vivo Huawei)

We think the transition requirement between states shall be introduced like in R16. In R16 power saving, we have following transition requirement:

When switching from low mobility scenario or not-at-cell-edge scenario to low mobility and not-at-cell-edge scenario during cell-reselection period, the UE shall fulfill ……… When switching from low mobility and not-at-cell-edge scenario to low mobility scenario or not-at-cell-edge scenario during cell-reselection period, the UE shall fulfill ……… When switching from normal mode to low mobility scenario or not-at-cell-edge scenario or low mobility and not-at- cell-edge scenario during cell-reselection period, the UE shall fulfill …… When switching from low mobility scenario or not-at-cell-edge scenario or low mobility and not-at-cell-edge scenario to normal mode during cell-reselection period, the UE shall fulfill …… No requirement is defined for multiple transitions of scenarios within one measurement period.

And also based on proposal 1,

Proposal 5: RAN4 to introduce minimum requirements at transition for the following cases:

- When switching from R17 stationary scenario to R17 stationary and R17 not-at-cell-edge scenario

- When switching from R17 stationary and R17 not-at-cell-edge scenario to R17 stationary scenario

- When switching from normal mode to R17 stationary scenario or R17 stationary and R17 not-at-cell-edge scenario

- When switching from R17 stationary scenario or R17 stationary and R17 not-at-cell-edge scenario to normal mode

# Conclusion

In this contribution, we further discuss the RRM relaxation requirement for RedCap.

Proposal 1: RAN4 only needs to consider two scenarios for R17 RedCap RRM relaxation in IDLE/inactive mode:

- configured R17 stationary criteria only, or

- configured both R17 stationary criteria and R17 not-at-cell-edge criteria

Proposal 2: except EMR, RAN4 to reuse the same mechanism of R16 RRM relaxation in power saving WI for R17 RedCap UE in IDLE/Inactive mode, i.e.,

- if single criteria (stationarity or not-at-cell-edge) is fulfilled, use K1_RedCap /K2_RedCap to relax the RRM requirement. K1_RedCap /K2_RedCap has the similar applicability condition as K1/K2 in R16 power saving WI, and FFS on the values for K1_RedCap /K2_RedCap, e.g., K1_RedCap =3 and K2_RedCap =60.

- if both criterion (stationarity and not-at-cell-edge) are configured and fulfilled, use a fixed long measurement period for requirement relaxation, e.g., like 1 hour in R16 power saving WI.

Proposal 3: For RedCap RRM relaxation in RRC connected mode, RAN4 to defer requirement design until sufficient conclusions are made in RAN2.

Proposal 4: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network.

Proposal 5: RAN4 to introduce minimum requirements at transition for the following cases:

- When switching from R17 stationary scenario to R17 stationary and R17 not-at-cell-edge scenario

- When switching from R17 stationary and R17 not-at-cell-edge scenario to R17 stationary scenario

- When switching from normal mode to R17 stationary scenario or R17 stationary and R17 not-at-cell-edge scenario

- When switching from R17 stationary scenario or R17 stationary and R17 not-at-cell-edge scenario to normal mode

# References

[1] R4-2115364, WF on R17 Redcap eDRX enhancements and RRM measurement relaxations, vivo, RAN4 #100e

[2] R2-2109197, LS on RRM Relaxations for RedCap, Huawei, RAN2#115e