3GPP TSG-RAN4 Meeting #101-e	R4-2117463

# Online, 1 – 12 November, 2021

Agenda item:	8.8.2.4

Source:	Apple

Title:	On remaining issues for R17 FR1 HST

Document for:	Discussion

# Introduction

In RAN4#100e FR1 HST enhancement was widely discussed, with agreement captured in [1]. There are still some open issues left. In this contribution, we will discuss the remaining issues under topic 3 captured in [1], with focus on CSSF and signalling design.

# Discussion

We will discuss the open issues following the order captured in [1]

- CSSF

- Agreements

  - For CSSF, it depends on network. There is no need to have the limitation on the number of Scell (s) in the spec

    - FFS: Performance degradation may occur when total evaluation period is longer than 5 secs in HST

The FFS part was raised in the last RAN4 meeting. Proponent of this FFS claimed that the UE won’t stay in the same cell for more than 5 seconds assuming 500kHz and typical network deployment. We can understand that if large number of SCells are configured then the measurement period on each SCC would be quite long. If it is longer than 5 second, then the measurement on SCC may become useless in some extent since UE is supposed to handover to the next cell. Note that CSSF on PCC remains the same regardless the number of configured SCells, so we assume UE can still trigger handover on PCC properly.

However, we don’t consider this as performance degradation, since UE is not required to report measurement result faster than the measurement requirements and network shall know that since network knows the CA configurations. In other word, if measurement period on SCC is longer than 5s, then NW shall not expect any measurement report on SCC within 5s.

On the other hand, 500km/h is the maximum supported velocity in this work item. In practise, the train may not move that fast. For instance, 350km/h is the maximum velocity in commercial HST in China. Therefore, UE can stay longer than 5s in the same serving cell.

Proposal 1: more justification is needed to allow performance degradation when total evaluation period is longer than 5 secs.

- for HST CA, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced CA requirements

Background for information: According to current RAN2 spec TS38.331, highSpeedMeasFlag-r16 is configured in IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB. And the IE ServingCellConfigCommon is used to configure cell specific parameters when configuring a UE with a SCells.

- Option 1: Yes

- Option 2: No

According to existing HST signalling design in LTE, a specific flag is used to indicate the application of enhanced CA requirements:

highSpeedEnhancedMeasFlag If the field is present, the UE shall apply the high speed (350 km/h) measurement enhancements as specified in TS 36.133 [16]. If highSpeedEnhMeasFlag2 is present, the UE indicating measurementEnhancements2 shall ignore this field.

highSpeedEnhancedDemodulationFlag If the field is present, the UE shall apply the advanced receiver in SFN scenario (350 km/h) as specified in TS 36.101 [42]. If this field is included in HighSpeedConfig and highSpeedEnhDemodFlag2 is present, the UE indicating demodulationEnhancements2 shall ignore this field in HighSpeedConfig.

highSpeedEnhDemodFlag2 If the field is present, the UE shall apply the further enhanced receiver in HST-SFN scenario (500 km/h) as specified in TS 36.101 [42].

highSpeedEnhMeasFlag2 If the field is present, the UE shall apply the high speed (500 km/h) measurement enhancements as specified in TS 36.133 [16].

highSpeedEnhMeasFlagSCell If configured with value TRUE, the UE shall apply the high speed (350 km/h) SCell measurement enhancements as specified in TS 36.133 [16].

highSpeedInterRAT-NR If the field is present, the UE shall apply the enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16].

highSpeedMeasGapCE-ModeA If the field is present, the UE in CE mode A shall apply the measurement gap sharing table associated with high-velocity scenario for measurements, as specified in TS 36.133 [16].

We prefer to reuse existing methodology unless critical issue is identified.

Proposal 2: following existing HST signalling design in LTE, a specific indication can be used to indicate the application of enhanced CA requirements.

- for inter-frequency measurement, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced requirements

- Option 1: Yes

- Option 2: No

We see no harm to use separate indicators for intra-frequency and inter-frequency measurement, respectively. In case of no HST inter-frequency layer deployed along the railway, UE doesn’t need to perform fast measurement.

Proposal 3: for flexibility, separate indicators can be used to indicate the application of enhanced measurement requirement for intra-frequency and inter-frequency layers.

- for idle state, whether NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply

- Option 1: Yes

- Option 2: No

According to existing R16 HST design, such differentiation is already allowed in E-UTRAN to NR inter-RAT measurements. Thus from NW deployment point of view, coexistence of HST and non-HST layers are possible. Therefore, we propose to have similar design in this WI.

Proposal 4: for idle state, NW is allowed to indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply.

- for connected state, whether NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply

- Option 1: Yes

- Option 2: No

If coexistence of HST and non-HST layers are possible, we think we can apply same methodology for both idle and connected mode.

Proposal 5: for connected state, NW is allowed to indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply.

# Conclusion

In this contribution, we further discuss the remaining issues under topic 3 in [1]. After discussion the following conclusions are provided:

Proposal 1: more justification is needed to allow performance degradation when total evaluation period is longer than 5 secs.

Proposal 2: following existing HST signalling design in LTE, a specific indication can be used to indicate the application of enhanced CA requirements.

Proposal 3: for flexibility, separate indicators can be used to indicate the application of enhanced measurement requirement for intra-frequency and inter-frequency layers.

Proposal 4: for idle state, NW is allowed to indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply.

Proposal 5: for connected state, NW is allowed to indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply.

# References

- R4-2115439, WF on RRM for FR1 HST, CMCC