3GPP TSG-RAN WG4 Meeting#101      	R4-2117471

E-meeting, 1st – 12th November, 2021

Title:	Receiver performance for pi/2 BPSK with spectral shaping

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.4.4

Document for:	Discussion

# 1	Introduction

This contribution relates to a study item agreed in RAN#91-e, namely “optimizations of pi/2 BPSK uplink power in NR” [1]. In this paper we study the BS receiver performance with different shaping filters. This work is related to the following study item objective:

- Identify shaping filter characteristics necessary to enable the new power capability while ensuring good and robust BS receiver performance.

  - Justify specification of a pulse shaping filter for this new identified UE power capability if it differs from filter impulse response specification in TS38.101-1 clause 6.4.2.4.1.E

# 2	Discussion

Frequency responses for the used filters are shown and compared against the current frequency flatness requirement in Figure 1.

The current specification assumes the spectral shaping to be transparent to gNB receiver. With such assumption the best performance is obtained by applying the same spectral shaping filter to both DMRS and data.

The [0.335, 1.0, 0.335] spectral shaping filter used in this paper is the most aggressive 3-tap filter that meets the current spectral flatness requirements as shown in our paper from RAN4#99e [2]. Less aggressive 3-tap filters are included too. 3-tap filter is implemented in frequency domain due to lower complexity.

Results include also 1+D filter scheme. In this scheme shaping is not used for transmitting DMRS, however the shaping is compensated in channel estimation of the receiver. This scheme requires specifying the used shaping filter so the receiver is aware of it. Also it has the drawback that shaping benefits are not obtained for DMRS symbols in transmitter, i.e. backoff can be higher for DMRS symbols than data symbols.

Observation 1: 1+D filter scheme used in simulations requires specifying the shaping filter function, all other schemes do not require specification changes.

Figure 1 Frequency responses of the filters compared to spectral flatness requirement

Simulation parameters are shown in Table 1 and the link simulation results for 2-64 PRB bandwidths in Figure 2. Simulation parameters are according to agreed way forward [3].

In these simulations we have used receiver that is aware of the used shaping filter also for the three tap filters. Receiver enhancement provides up to 0.3dB gain depending on the case. We have provided results without the enhancement in our previous RAN4 meeting contribution [4].

It can be observed that link loss due to more aggressive spectral shaping is higher with small bandwidths, hence it could be beneficial to use milder filtering when bandwidth is small.

Observation 2: Link loss due to more aggressive spectral shaping filtering is higher with small bandwidths.

Link loss of 1+D filter is low with 2-4 PRB but higher with 8-16 PRB bandwidth. This result requires that the receiver is aware of the filter used.

Observation 3: Link loss due to 1+D filter is low with 2-4 PRBs but higher with 8-16 PRBs.

Due to the receiver enhancement the link loss of three tap filters reduces with wider bandwidths. In small bandwidths the receiver enhancement has low or negligible gain with the simulation assumptions used. In 64PRB case link loss of three tap filters is already lower than link loss of 1+D in all simulated cases. Receiver can benefit from the knowledge of shaping also in transparent case where the same shaping is used for DMRS, however depending on the scenario, the gain may be relatively small.

Observation 4: Using shaping aware receiver the link loss of three tap filters is lower than link loss of 1+D with higher bandwidth.

When comparing results between three channel models, it can be noted that for given number of PRBs, the performance difference between the used filters is quite similar for all the channel models, however magnitude of differences varies slightly.

Observation 5: For given number of PRBs, the performance difference between the used filters is quite similar for all the channel models.

Table 1 Simulation assumptions

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 100MHz |
| SCS | 30kHz |
| Channel model | TDL-C 300ns, TDL-A 30ns, TDL-D 30ns |
| UE speed | 3km/h |
| Channel estimation | Frequency domain |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| DMRS config | Low PAPR sequence type 2, 2 symbols |
| Waveform | DFT-S-OFDM |
| HARQ config | No retransmissions |
| Num PRBs | 2,4,8,16,64 |
| MCS | 0 |
| Channel | PUSCH, 14 OFDM symbols |
| Frequency hopping | No |
| BLER | 10% |
| Spectral shaping filter | 3-tap, FD implementation 1+D (not used for DMRS symbols) |

Figure 2 Required SNR for 10% BLER for different channel profiles

# 3	Conclusion

In this contribution we have studied receiver performance of pi/2 BPSK with spectral shaping. Based on the results we have made following observations:

Observation 1: 1+D filter scheme used in simulations requires specifying the shaping filter function, all other schemes do not require specification changes.

Observation 2: Link loss due to more aggressive spectral shaping filtering is higher with small bandwidths.

Observation 3: Link loss due to 1+D filter is low with 2-4 PRBs but higher with 8-16 PRBs.

Observation 4: Using shaping aware receiver the link loss of three tap filters is lower than link loss of 1+D with higher bandwidth.

Observation 5: For given number of PRBs, the performance difference between the used filters is quite similar for all the channel models.

# 4 	References

- RP-210910, Optimizations of pi/2 BPSK uplink power in NR, Moderator (Huawei)

- R4-2109742, Achievable UE Tx power for pi/2 BPSK with different shaping filters, Nokia, Nokia Shanghai Bell

- R4-2108018, WF on Waveform configuration, parameters for link simulations and agreements, Qualcomm

- R4-2112805, Receiver performance for pi/2 BPSK with spectral shaping, Nokia, Nokia Shanghai Bell