3GPP TSG-RAN WG4 Meeting #101-e     	R4-2117476

e-Meeting, November 1 – November 12, 2021

Title:	Discussion on RF requirements for non-scheduled gap

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.18.2.1

Document for:	Approval

# Introduction

The following can be noted from the work item description (WID) for Rel-17 coverage enhancement [1]:

  - “Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary, by RAN4 [RAN1, RAN4]

      - Potential optimization of DM-RS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]”

The remaining open issues concerning the phase continuity and power consistency conditions for PUSCH and PUCCH repetitions to enable joint channel estimation as exchanged in RAN4 [2]-[5] and RAN1 LSs [6]-[9] will be discussed in this contribution.

# Discussion

The following can be noted from the WF made in RAN4#100-e [10]:

- On off power less than 1ms gap - Option 1: Define new transmit off power for gap symbols explicitly for Rel-17 coverage enhancement case - Alt.1: Define a relaxed transmit off power due to shorter measurement period - Alt.2: Carrier leakage requirement applies for the non-scheduled gap - FFS how to define carrier leakage requirement with no signal transmitted during the gap - Option 2: RAN4 do not introduce new transmit off power - i.e. no requirement applies during the gap - Option 3: The existing OFF power level of -50dBm apply for less than 1 ms - FFS whether to and how to introduce measurement uncertainty

It is worth noting that the discussion on whether to introduce a new transmit off power was triggered for solving the potential issue on the maximum unscheduled gap to be greater than what is currently specified in RAN4. However, with the conclusion made in RAN4#100-e meeting that the maximum unscheduled gap is 13 symbols, then the constraint on off power is not more stringent (if not more relaxed), since the current constraint is 1ms, and 13 symbols is always less than 1ms regardless of SCS. Therefore, the above Option 2 is preferred.

Proposal 1. RAN4 does not introduce new transmit off power constraint.

# 3	Conclusion

In this contribution, we proposed the following for phase continuity and power consistency for PUSCH and PUCCH repetitions:

Proposal 1. RAN4 does not introduce new transmit off power constraint.

# 4 	References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R4-2103393	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), February 2021.

- R4-2105417	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), April 2021.

- R4-2107880	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), May 2021.

- R4-2114991 Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), August 2021.

- R1-2009784	LS on PUCCH and PUSCH repetition, Qualcomm (moderator), November 2020.

- R1-2104119	Reply on LS on PUCCH and PUSCH repetition, Qualcomm (moderator), April 2021.

- R1-2106212	LS on joint channel estimation for PUSCH and PUCCH, China Telecom (moderator), May 2021

- R1-2108458	Reply on LS on PUCCH and PUSCH repetition, Qualcomm (moderator), August 2021.

- R4-2114992	WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, August, 2021.