3GPP TSG-RAN WG4 Meeting #101-e	R4-2117493

Online Meeting, Nov 1 – 12, 2021

Agenda item:	8.21.2.3

Source: 	Qualcomm Incorporated

Title: 	On NR positioning measurements in RRC_INACTIVE

Document for:	Discussion

# Introduction

RAN4 continued discussing support of NR positioning measurements in RRC_INACTIVE in RAN4#100-e. Agreements and open issues were captured in a WF [1]. We will discuss the following topics in this contribution.

- Applicability of requirements in RRC_INACTIVE

- Impact on RRC_INACTIVE state functions

- UE capability

- Measurement period

# Applicability of requirements in RRC_INACTIVE

Initially, RAN4 agreed to specify requirements for DL only positioning measurements based on agreements in other WGs at the time [1]. However, more recently RAN1 and RAN2 have reached agreements (below) to enable support of UL+DL positioning in inactive state [2, 3].

RAN4

Agreements:

At least UE RRM requirements for DL RSTD and DL PRS-RSRP measurements in RRC-INACTIVE state are specified.

RAN2

Agreement

- gNB can configure the UE with periodic SRS (assuming periodic SRS is supported in RRC_INACTIVE) by RRCRelease with suspendConfig at least when periodic event is configured for deferred MT-LR.  Other cases can be further discussed.

RAN1

Agreement:

From RAN1 perspective, it is feasible to support transmission of SRS for positioning by UEs in RRC _INACTIVE state for UL and DL+UL positioning under certain validation criteria

- FFS: Type(s) of SRS for positioning (i.e., periodic, semi-persistent, aperiodic)

- FFS: Details of validation criteria which may also be discussed in RAN2

- Send LS to RAN2 informing them of this agreement

Agreement:

- Send LS to RAN2 with the outcome of RAN1 discussion on types of SRS for positioning to be supported by UEs in RRC_INACTIVE state

- From RAN1 perspective, support of semi-persistent SRS for positioning by RRC_INACTIVE UEs is feasible

- It is up to RAN2 to confirm support of semi-persistent SRS for positioning by RRC_INACTIVE UEs and determine necessary signalling details

Agreement:

- For RRC_INACTIVE UEs, SRS for positioning bandwidth, SCS and CP type are configured by RRC and can be different from that of initial UL BWP configured by the system information

Agreement:

For OLPC of SRS for positioning transmission by RRC_INACTIVE UEs,

- Reuse validity criteria for pathloss measurement defined for RRC_CONNECTED UEs in Rel.16

  - FFS on UE fallback behavior (i.e. whether to reuse fallback to pathloss measurement by RRC_INACTIVE UE for the cell, from which the SS/PBCH is received to obtain MIB, is not accurate)

- For spatial relation of SRS for positioning transmission by RRC_INACTIVE UEs,

  - FFS whether to define validity criteria or reuse validity criteria for OLPC pathloss measurement to determine whether spatial relation with configured RS is valid

In view of the above RAN1 and RAN2 agreements, we understand that there are no impediments to supporting RTT positioning in inactive state and RAN4 should specify corresponding requirements.

Proposal 1: UE requirements for UE Rx-Tx time difference measurements in RRC-INACTIVE state should be specified by RAN4.

Additional applicability questions were raised in RAN4#100-e and captured in the FFS of the agreement below [1].

Agreements:

  - RAN4 shall define inactive state positioning measurements for FR1 and FR2.

FFS:

Further study the following applicability:

    - Option 1: (Ericsson)

      - RAN4 to define periodic inactive state positioning measurements and reporting of positioning measurement which involves state transition to connected state from inactive state.

    - Option 2: (Huawei, Intel)

      - Measurement requirements do not apply for a PRS resource if it has instances colliding with paging.

Regarding option1, in our view there is no need to specify requirements for state transition to RRC_CONNECTED for the purpose of reporting positioning measurements performed in RRC_INACTIVE. Per RAN2 agreement (below), LPP UL message segmentation is supported [2].

RAN2 agreement

Proposal 3: UL LPP message segmentation can also be used by the UE in RRC_INACTIVE state; i.e., a LPP message body can be sent in several shorter LPP messages instead of one long LPP message by using the SDT "Subsequent Data Transmission" phase.  FFS spec impact.

Proposal 2a: Support of SDT is a necessary condition for supporting positioning measurements in RRC_INACTIVE.

Proposal 2b: No need to specify requirements for state transition to RRC_CONNECTED for the purpose of reporting positioning measurements performed in RRC_INACTIVE.

Regarding option 2, we understand that collisions between PRS and paging occasions should be addressed in the measurement delay requirements and applicability in RRC_INACTIVE. In our view paging occasions should have higher priority. Note that RAN1 has also reached an agreement (see Section 3) that is consistent with our understanding. The specifics of how to handle the collisions in the measurement requirements need more discussion.

Proposal 3: Paging occasions should have higher priority than positioning measurements in RRC_INACTIVE. FFS how to reflect this in the positioning measurement requirements in RRC_INACTIVE.

# Impact on RRC_INACTIVE state functions

With respect to the question of the impact of positioning measurements performed in inactive state on other UE functions, RAN4 has agreed that further discussion is needed to determine the impact [1].

Agreements:

RAN4 to discuss impact of positioning measurements on RRC INACTIVE state functions.

We note here that RAN1 has reached a relevant agreement regarding this question and RAN4 should take this into account during the discussion of requirements for positioning in inactive state [3]. Based on the RAN1 agreement, the impact of PRS-based measurements on acquiring system information, cell (re-)selection and paging reception should be minimized. RAN4 should follow this principle when specifying requirements for positioning in RRC_INACTIVE.

RAN1

Agreement:

- From RAN1 perspective, in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT)

  - FFS how to determine conflicts in DL PRS and other DL signals/channels reception by UE

  - FFS how to handle retuning time for the case when DL PRS and other DL signals/channels are allocated in different BW and/or have the same or different SCS as initial DL BWP

- Send LS to RAN4 (cc RAN2) and ask if there is any feedback

Observation 1: Requirements for positioning measurements in RRC_INACTIVE should be specified in such a way that the impact of PRS-based measurements on acquisition of system information, cell (re-)selection and paging reception is minimized.

# UE capability

In RAN4#100-e there was limited discussion about the need for a new UE capability to indicate support of NR positioning measurements in RRC_INACTIVE. Most companies indicated preference to wait for more progress in RAN1 [1].

- Option 1: (Ericsson, Huawei, vivo, CATT, Nokia)

  - RAN4 to wait for RAN1 progress regarding UE measurement capability within DL RSTD and PRS-RSRP

- Option 2: (Qualcomm)

  - A new UE capability would be required to support the feature.

Our expectation is that requirements for positioning measurements in RRC_INACTIVE will be substantially different from the NR positioning requirements introduced in Rel-16. In the present discussion, RAN4 has already identified a number of factors that will affect the measurement requirements in RRC_INACTIVE (see discussion in Section 5). For example, we anticipate that power consumption considerations will shape the measurement requirements in inactive state, while this was not the case for the existing requirements in connected state. Therefore, it cannot be assumed that a UE that supports Rel-16 NR positioning also supports positioning in RRC_INACTIVE.

Proposal 4: A new UE capability is required to indicate support of NR positioning measurements in RRC_INACTIVE.

# Measurement period

In RAN44#100-e, RAN4 agreed to use the measurement period formulation in Rel-16 as the baseline (starting point) for requirements in inactive state [1]. At the same time, RAN4 also identified a number of factors that should be considered when specifying the new requirements for positioning in inactive state.

Agreements:

Use the framework or formula of Rel-16 RSTD measurement period as a baseline to derive the inactive state RSTD measurement period.

Agreements:

MG is not to be considered in the measurement period requirements in RRC_INACTIVE state.

FFS:

Further study the following factors and impacts for the measurement requirements in RRC_INACTIVE state:

- Whether to use the reduced number of samples if it is agreed in Rel-17.

- Whether to use the summation-based approach for total frequency layers.

- The impact of paging periods.

- Analysis on PRS resource configuration, positioning measurement period and DRX behaviors in the UE RRC_INACTIVE state.

- How to define the measurement interval Teffect.

- How to define the parameter Kcarrier.

- Others

- Option 1: (vivo)

  - UE RRM requirements for DL PRS-RSRP measurements and DL RSTD measurements in RRC-INACTIVE state are specified based on reduced number of samples if it is agreed in Rel-17.

- Option 2: (vivo, Huawei)

  - UE RRM requirements for DL PRS-RSRP measurements and DL RSTD measurements in RRC-INACTIVE state are specified with summation-based approach for total frequency layers.

- Option 3: (Nokia, Intel)

  - RAN4 starts with analysis on PRS resource configuration, positioning measurement period and DRX behaviors in the UE RRC_INACTIVE state. Consider following for minimum requirements.

    - A UE follows DRX cycle for paging to measure PRS. A UE completes PRS measurements during active DRX period for paging. A new measurement period requirement can be discussed.

    - Others procedure are not precluded for positioning measurements in inactive mode regarding power saving and measurement latency reduction.

- Option 4: (ZTE)

  - When defining core requirements for UE positioning under INACTIVE mode, the principle can be to replace the measurement gap related parameters with paging periods and re-use the R16 requirements for CONNECTED mode.

- Option 5: (Huawei, Intel)

  - The measurement interval Teffect should take DRX cycle but not MGRP into account.

  - The parameter Kcarrier should take one additional PFL into account.

When formulating positioning measurement requirements in inactive state, in addition to the principle of minimizing impact to other UE functions, we understand one important consideration is UE power consumption. To preserve power savings, the cadence of the measurements should be dependent on the DRX cycle length. This is reflected in proposals from several companies in RAN4#100-e. In addition, it would be beneficial to limit the PRS processing time (per measurement occasion) to a reasonable fraction of the DRX cycle so that the UE does not have to be awake most of the time. This last point motivates the need to introduce a new UE PRS processing capability for measurements performed/processed in inactive state.

Proposal 5: Preserve power savings in RRC_INACTIVE by specifying that the cadence of positioning measurements be dependent on the DRX cycle length.

Observation 2: To further preserve power savings in RRC_INACTIVE, the UE should aim to limit the processing time per measurement occasion. A new UE PRS processing capability for measurements performed/processed in inactive state would be useful to achieve this objective.

Even though we support the idea that the PRS measurement period requirement in inactive state should depend on the DRX cycle length, there are questions about how the feature would work at the system level since the LMF may not be aware of the RRC state of the UE nor the DRX cycle length. How would the LMF set an appropriate value for the response time in the location request sent to the UE without this information?

Observation 3: If the measurement period in RRC_INACTIVE depends on the DRX cycle length, which is not known by the LMF, then the LMF may not be able to select an appropriate value for the response time in the location request.

Proposal 6: If RAN4 agrees that the measurement period requirement in RRC_INACTIVE depends on the DRX cycle length, RAN4 should send an LS to RAN2/RAN3 saying that RAN4 assumes that the LMF would have knowledge of the DRX cycle and asking for new signaling to be introduced.

Proposal 7: FFS how to minimize the impact of positioning measurements in RRC_INACTIVE on cell (re-)selection.

# Conclusions

Proposal 1: UE requirements for UE Rx-Tx time difference measurements in RRC-INACTIVE state should be specified by RAN4.

Proposal 2a: Support of SDT is a necessary condition for supporting positioning measurements in RRC_INACTIVE.

Proposal 2b: No need to specify requirements for state transition to RRC_CONNECTED for the purpose of reporting positioning measurements performed in RRC_INACTIVE.

Proposal 3: Paging occasions should have higher priority than positioning measurements in RRC_INACTIVE. FFS how to reflect this in the positioning measurement requirements in RRC_INACTIVE.

Observation 1: Requirements for positioning measurements in RRC_INACTIVE should be specified in such a way that the impact of PRS-based measurements on acquisition of system information, cell (re-)selection and paging reception is minimized.

Proposal 4: A new UE capability is required to indicate support of NR positioning measurements in RRC_INACTIVE.

Proposal 5: Preserve power savings in RRC_INACTIVE by specifying that the cadence of positioning measurements be dependent on the DRX cycle length.

Observation 2: To further preserve power savings in RRC_INACTIVE, the UE should aim to limit the processing time per measurement occasion. A new UE PRS processing capability for measurements performed/processed in inactive state would be useful to achieve this objective.

Observation 3: If the measurement period in RRC_INACTIVE depends on the DRX cycle length, which is not known by the LMF, then the LMF may not be able to select an appropriate value for the response time in the location request.

Proposal 6: If RAN4 agrees that the measurement period requirement in RRC_INACTIVE depends on the DRX cycle length, RAN4 should send an LS to RAN2/RAN3 saying that RAN4 assumes that the LMF would have knowledge of the DRX cycle and asking for new signaling to be introduced.

Proposal 7: FFS how to minimize the impact of positioning measurements in RRC_INACTIVE on cell (re-)selection.

# References

[1] R4-2115367, WF on Rel-17 positioning enhancements RRM_2, RAN4#100-e

[2] RP-212365, Status report of NR positioning enhancements, RAN#93-e

[3] Chair’s Notes RAN1#106bis-e 8.5 v05, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_106b-e/Inbox/Havish_sessions/Chair's%20Notes%20RAN1%23106bis-e%208.5%20v05.doc