3GPP TSG-RAN WG4 Meeting #101-e 	R4-2117527

Electronic Meeting, 01st Nov 2021 - 12th Nov 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for PUSCH demodulation requirements for FR1 256QAM

Agenda item:	8.12.3.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the PUSCH demodulation requirements for FR1 256QAM topic are captured in the summary ‎[1] and WF ‎[2].

In this contribution we collect our simulation results.

Simulation setup

Table 1: Agreed test parameters from WF [2].

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

Simulation results

Following the updates of simulation scenarios.

1T2R

Table 2: Results SNR@70%TPUT, 1T2R. Rank 1

| PUSCH 1T2R |  |  |  |  |  | 70%TPUT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.04 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.64 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.47 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 17.01 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 18.29 |
|  | FR1 Mapping type B | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.03 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.62 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.47 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16.98 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 18.32 |

1T4R

Table 3: Results SNR@70%TPUT, 1T4R. Rank 1

| PUSCH 1T4R |  |  |  |  |  | 70%TPUT |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 12.75 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13.01 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 12.96 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13.35 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 14.22 |
|  | FR1 Mapping type B | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 12.76 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13.01 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 12.94 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13.35 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 14.21 |

1T8R

Table 4: Results SNR@70%TPUT, 1T8R. Rank 1

| PUSCH 1T8R |  |  |  |  |  | 70%TPUT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.62 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.83 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.8 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10.11 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10.62 |
|  | FR1 Mapping type B | 15kHz, 5MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.62 |
|  |  | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.83 |
|  |  | 30kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9.82 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10.13 |
|  |  | 30kHz, 100MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10.61 |

Conclusion

In this contribution we have provided our simulation results for PUSCH demodulation requirements for FR1 256QAM.
We have not made any observations or proposals.

- References

- R4-2115801	Email discussion summary for [100-e][329] NR_perf_enh2_Demod_Part3_NWM, Moderator (Huawei, HiSilicon), RAN4#100-e.

- R4-2115748	WF on FR1 PUSCH demodulation requirements for Fr1 256QAM, Huawei, HiSilicon, RAN4#100-e.