3GPP TSG-RAN WG4 Meeting#101      	R4-2117551

E-meeting, 1st – 12th Nov, 2021

Title:	TP to FWA TR General E-UTRA UE Tx requirements

Source:	Nokia

Agenda item:	7.28.3.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss common E-UTRA Tx UE RF requirement for FWA HPUE. Discussion is built on work done during SI and captured into TR [1]. TP is proposed.

# 2	Discussion

From the WID[2] we can see the necessary common objectives the WI needs to address.

Common Objectives:

- UE MOP, MPR, and ACLR, considering the interactions between them (e.g. MRP and ACLR) and other related parameters

The TP in this contribution addresses these issues for E-UTRA.

# 3 	References

[1] https://www.3gpp.org/ftp/Specs/archive/37_series/37.880/37880-h10.zip

[2] RP-212533, Revised WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands, Nokia, RAN#93e

******************************** Start of the TP ********************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

******************************** No change ********************************

# 6	Common requirements for a UE

Editor note: This section relates to the common objectives of the WI.

## 6.1	Maximum output power

### 6.1.1	E-UTRA

For E-UTRA defining PC1 operation for a band has been done already for bands 3, 14, 20, 28, 31 72, 87 and 88. Concerning the actual power class definition relevant bands need to be added into [2] Table 6.2.2-1: UE Power Class.

In addition to above also section 6.2.2A in [2] needs attention as there is following sentence

For inter-band carrier aggregation with one uplink component carrier assigned to one E-UTRA band in Band 3, 20, 28, or 31, the requirements for power class 1 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

Reason for this sentence is that if the UL band in CA configuration is PC1 then the already specified CA configuration may not work properly as CA work was done for PC3 UL. Therefore, the relevant bands need to be added into this list.

## 6.2	MPR

### 6.2.1	E-UTRA

E-UTRA MPR defined in [2] Table 6.2.3-1 is defined to be power class agnostic hence no changes are required.

Table 6.2.3-1: Maximum Power Reduction (MPR) for Power Class 1, 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |
| 64 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 2 |
| 64 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  |  |  | ≤ 5 |

## 6.3	ACLR

### 6.3.1	E-UTRA

E-UTRAACLR is already defined for PC1 in Table 6.6.2.3.1-2 hence no changes are required.

Table 6.6.2.3.1-2: Additional E-UTRAACLR requirements for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |
| NOTE 1: E-UTRAACLR1 shall be applicable for >23dBm |  |  |  |  |  |  |

In [2] clause 6.6.2.3.2 it is stated that UTRAACLR is not applicable to the power class 1 UE operating in Band 3, 20, 28, 31 or 72. This sentence needs case by case treatment when relevant E-UTRA PC1 band is introduced and if UTRA operation does not anymore exist in that band then that band is added to the list, otherwise UTRA ACLR applies.

******************************** End of the TP ********************************