3GPP TSG-RAN WG4 Meeting #101-e	R4-2117648

1 –12 November 2021

Source:	Qualcomm Incorporated

Title:	RF switching time for V2X intra-band con-current operation with different carriers in TDD bands.

Agenda item:	8.15.4.2

Document for:	Approval

# 1.	Introduction

In RAN4#100-e there was a discussion on the RF switching time length for NR-to-NR sidelink switching for V2X intra-band operation with different carriers in TDD bands.  In this paper we would like to further discuss issues related to this topic.

# 2. 	Discussion

Currently, [1] defines the UL retuning timeline for the operating scenarios of SRS carrier switching and uplink TX switching. For SRS carrier switching it is mentioned in section 6.2.1.3 of [1] that

In the above text the N OFDM symbols represent the PUSCH preparation time that can be configured according to the tables in section 6.4 of [1] based on UE timing capability.  It can also be seen that the total time from the DCI grant to UL transmission is the sum of the PUSCH preparation time and the RF retune time.

For uplink switching section 6.1.6 of [1] gives the following:

For uplink switching the total time from DCI grant to transmission on the retuned frequency is the sum of the UE processing time (Toffset) and the switching gap NTx1-Tx2 period which begins at T0.

We think that Uu to SL and SL to Uu switching should follow a similar timeline as mentioned above for SRS carrier switching and uplink switching. For mode 1 operation where intra-band Uu to SL switching is initiated by a PDCCH DCI grant we propose that the total switching time from SL grant to SL transmission should include both the PSSCH preparation time given in section 8.6 of [1], for which the PSSCH preparation time N2 is given below, and the Uu to SL RF switching time.

Observation: For SL to Uu switching the PSSCH preparation time N2 is given by the table below

|  | PSSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 1- PSSCH preparation time for SL to Uu switching (table 8.6-1 [1])

Proposal 1: In mode 1 for intra-band Uu to SL switching the total switching time from SL grant to SL transmission shall be the sum of the PSSCH preparation time, and the Uu to SL RF switching time.

If proposal 1 is acceptable then we are agreeable to a Uu to SL RF switching time of 140us.

Similarly for mode 1 intra-band switching from SL to Uu we propose that the total switching time from Uu grant to Uu transmission should be the sum of the PUSCH preparation time given in section 6.4 of [1] and the SL to Uu RF switching time.

Proposal 2: In mode 1 operation for intra-band SL to Uu switching the total switching time from Uu grant to Uu transmission shall be the sum of the PUSCH preparation time, and the SL to Uu RF switching time.

If proposal 2 is acceptable then we are agreeable to an intra-band SL to Uu RF switching time of 140us.

SL to Uu and Uu to SL switching involves two modems and has greater signalling complexity and longer preparation time associated with it compared switching involving a single modem. Therefore, for SL to Uu switching we propose using the PUSCH preparation time based on the number of N2 symbols given in the table below.

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 2- PUSCH preparation time for SL to Uu switching (table 6.4-1 [1])

Proposal 3: For SL to Uu switching use PUSCH preparation time where N2 is given by table 2.

# Conclusion

In this paper the total switching time from SL grant to SL transmission and the total time from Uu grant to Uu transmission was discussed and the following proposals are made:

Proposal 1: In mode 1 for intra-band Uu to SL switching the total switching time from SL grant to SL transmission shall be the sum of the PSSCH preparation time and the Uu to SL RF switching time.

Proposal 2: In mode 1 operation for intra-band SL to Uu switching the total switching time from Uu grant to Uu transmission shall be the sum of the PUSCH preparation time and the SL to Uu RF switching time.

Proposal 3: For SL to Uu switching use PUSCH preparation time where N2 is given by table 2.

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 2- PUSCH preparation time for SL to Uu switching (table 6.4-1 [1])

# References

[1] TS38.214, v16.2.0, Physical layer procedures for data