3GPP RAN WG4 Meeting #101-e		R4-2117674

Electronic Meeting, November 1-12, 2021

Agenda item:	8.16.3.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Minimum peak EIRP for FR2-2

Document for:	Approval

1	Introduction

In RAN4 #100-e meeting antenna size and minimum peak EIRP was discussed, and further analysis on the UE EIRP requirements was needed [1]. This contribution shows our views on Handheld UE minimum peak EIRP for FR2-2.

2	Discussion

In RAN4 #100-e meeting, we discussed antenna element numbers. Several companies prefer 2x8 antenna elements, but several companies prefer smaller size for implementation. The wavelength of 60 GHz is 5.0 mm, so the antenna array width can be over 4 mm under the condition of 4.0 dBi antenna element gain and 2x8 array. A 2 line antenna array may exceed smart phone size constraints, so we believe it is beneficial to consider both 2x8 and 1x8 antenna arrays.

Observation 1:	Smart phone form factor size constraints may not accommodate 2x8 antenna array.

Table 1 : Minimum peak EIRP for handheld UE in FR2-2

| parameters | Unit | 52.6 - 71GHz |  |
| --- | --- | --- | --- |
| Pout per element | dBm | 5.0 | 5.0 |
| # of antennas in an array |  | 8 | 16 |
| Total conducted power per polarization | dBm | 14.0 | 17.0 |
| Average antenna element gain | dBi | 4.5 | 3.0 |
| Antenna gain roll-off loss versus frequency | dB | -3.0 | -3.0 |
| Realized antenna array gain | dB | 10.5 | 12.0 |
| Polarization gain | dB | 2.8 | 2.8 |
| Total implementation loss | dB | -11.7 | -11.8 |
| Minimum peak EIRP | dBm | 15.7 | 20.1 |

Observation 2:	The values of Minimum peak EIRP are 15.7 dBm with 8 antenna elements and 20.1 dBm with 16 antenna elements.

We calculated communication range between Handheld UE minimum peak EIRP and several BTS EISs by equations in TR38.901. Figure 1 shows its result.

Figure 1 : communication range with several BTS EISs

It would be severe communicating over 100m directly in NLOS condition. On the other hand, it would be possible communicating over 100m in LOS condition. It will be feasible using RIS (Reconfigurable Intelligent Surfaces) or repeater to communicate long range in NLOS condition.

Observation 3:	It will be severe communicating 100m range directly in NLOS condition.

Proposal 1:	It will be feasible using RIS or repeater to communicate long range in NLOS condition.

3	Conclusions

This contribution provides our view on the minimum peak EIRP for handheld UE:

Observation 1:	Smart phone form factor size constraints may not accommodate 2x8 antenna array.

Observation 2:	The values of Minimum peak EIRP are 15.7 dBm with 8 antenna elements and 20.1 dBm with 16 antenna elements.

Observation 3:	It will be severe communicating 100m range directly in NLOS condition.

Proposal 1:	It will be feasible using RIS or repeater to communicate long range in NLOS condition.

4	References

R4-2114989, “WF on 60 GHz UE RF requirements,” Qualcomm

R4-2117675, “Views on REFSENS for FR2-2,” Murata Manufacturing Co., Ltd.