3GPP TSG-RAN WG4 Meeting # 101-e      		R4-2117741

Electronic Meeting, November 01-12, 2021

Agenda Item:	8.12.3.1

Source: 	CMCC

Title: 	Simulation results for PUSCH 256QAM performance

Document for:	Discussion

- Introduction

In RAN4#100-e meeting, all the test parameters of BS demodulation for FR1 256QAM has been agreed except for the Tx EVM impact. In this contribution, we provide our simulation results for PUSCH 256QAM performance.

- Discussion

Tx EVM: No explicit Tx EVM modelling

–	Consider 3.5% Tx EVM impact in the impairment results submitted by companies, but it is up to companies

In the following impairment results, no additional margin is added due to Tx EVM.

- Simulation Results

Our simulation results are summarized in table 1.

Table 1. Simulation results for time domain resource allocation Type A

| Time domain resource allocation type | Tx*Rx | SCS&BW | Ideal results SNR(dB) | Impairment results SNR(dB) |
| --- | --- | --- | --- | --- |
| Type A | 1T2R | 15k 5M | 16.6 | 18.6 |
|  |  | 15k 10M | 17 | 19 |
|  |  | 30k 10M | 16.7 | 18.7 |
|  |  | 30k 40M | 17.2 | 19.2 |
|  |  | 30k 100M | 18.25 | 20.25 |
|  | 1T4R | 15k 5M | 12.75 | 14.75 |
|  |  | 15k 10M | 12.95 | 14.95 |
|  |  | 30k 10M | 12.9 | 14.9 |
|  |  | 30k 40M | 13.1 | 15.1 |
|  |  | 30k 100M | 13.8 | 15.8 |
|  | 1T8R | 15k 5M | 9.25 | 11.25 |
|  |  | 15k 10M | 9.55 | 11.55 |
|  |  | 30k 10M | 9.55 | 11.55 |
|  |  | 30k 40M | 9.7 | 11.7 |
|  |  | 30k 100M | 10.1 | 12.1 |

Table 2. Simulation results for time domain resource allocation Type B

| Time domain resource allocation type | Tx*Rx | SCS&BW | Ideal results SNR(dB) | Impairment results SNR(dB) |
| --- | --- | --- | --- | --- |
| Type B | 1T2R | 15k 5M | 16.6 | 18.6 |
|  |  | 15k 10M | 16.6 | 18.6 |
|  |  | 30k 10M | 16.6 | 18.6 |
|  |  | 30k 40M | 17.2 | 19.2 |
|  |  | 30k 100M | 18.2 | 20.2 |
|  | 1T4R | 15k 5M | 12.6 | 14.6 |
|  |  | 15k 10M | 13.1 | 15.1 |
|  |  | 30k 10M | 12.8 | 14.8 |
|  |  | 30k 40M | 13.15 | 15.15 |
|  |  | 30k 100M | 13.8 | 15.8 |
|  | 1T8R | 15k 5M | 9.2 | 11.2 |
|  |  | 15k 10M | 9.5 | 11.5 |
|  |  | 30k 10M | 9.4 | 11.4 |
|  |  | 30k 40M | 9.7 | 11.7 |
|  |  | 30k 100M | 10.1 | 12.1 |

- Conclusion

In this contribution, we provide our simulation results for 256QAM BS demodulation requirements.

- Reference

[1] R4-2115748, Way forward for FR1 PUSCH 256QAM performance requirements, Huawei.

- Annex

Table 3. Simulation assumptions for PUSCH 256QAM performance requirement

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

Figure 1. Simulation results for time domain resource allocation Type A

Figure 2. Simulation results for time domain resource allocation Type B