3GPP TSG-RAN4 WG4 Meeting #101-e	R4-2117771

# Electronic meeting, 1 November – 12 November, 2021

Source: 	vivo

Title: 	Further discussion on interruption for 52.6-71GHz

Agenda Item:	8.16.7.3

Document for:	Discussion

Introduction

In RAN4 #100e, Interruption requirements from 52.6GHz to 71GHz was discussed. The interruptions at SCell addition/release and SCell activation/deactivation for inter-band CA is still in discussion. A WF [1] related to the above was duplicated as below:

Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X1 for SCell addition/release for inter-band CA NR Slot length (ms) of victim cell; Interruption length X1 (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 64. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR2-2; 16. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS.

In this contribution, we further discuss the interruption requirements at SCell addition/release and SCell activation/deactivation for inter-band CA, at UL carrier RRC reconfiguration and when identifying CGI of an NR cell with autonomous gaps from 52.6GHz to 71GHz.

Discussion

Interruptions at SCell addition/release for inter-band CA

In the last meeting, the deployment scenario was agreed and we duplicate the detail as below:

- Define the requirements for the following deployment scenarios with equal priority: - Standalone single carrier and CA in FR2-2 - FR2-2 CA and DC with anchor on FR1 - Note: the scenario may be further adjusted pending further discussion in the RF session

The scenario for FR2-2 CA and DC with anchor on FR2-1 is out of scope in Rel-17. Therefore, we suggest to remove the ‘Aggressor is on FR2-1’ from the existing requirements.

Proposal 1: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, ‘Aggressor is on FR2-1’ need to be removed from the existing requirements.

Meanwhile, all companies agreed that the interruption requirements for FR2-2 shall consider the impact of MRTD. For inter-band NR CA carrier aggregation, the existing requirements are specified in [38.133] as below:

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2; 8 note1. Between FR1 and FR2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA..

For FR2-2, the slot length is 15.625us and 31.25us for 960kHz and 480kHz respectively. When the victim CC is in FR2-2 and the aggressor is in FR1, the MRTD would be 25us (assumed to be same as the legacy requirements) which is more than 1 slot for 960kHz. Therefore, 1 more slot would need to be interrupted for the case of the aggressor is on FR1. Even when the aggressor CC is in FR2-2, if reusing the legacy requirements, the MRTD would be 8 us which is also about 4 and 9 symbols for 480kHz and 960kHz and 1 more slot also need to be considered. So we suggest the interruption slots for the case of ‘Aggressor is on FR2-2’ could need to be reconsidered. Meanwhile, the MRTD value of FR2-2 would be discuss in the next meeting. We suggest to define the interruption requirements until there is conclusion for the MRTD requirements for FR2-2.

Proposal 2: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, the interruption requirements need to be defined until there is conclusion for the MRTD requirements for FR2-2.

Interruptions at UL carrier RRC reconfiguration

When an UL carrier or supplementary UL carrier is configured or de-configured, an interruption is allowed during the RRC reconfiguration procedure. The existing interruption requirements for Standalone NR CA aggregation are specified as below [38.133].

Table 8.2.2.2.4-1: Interruption duration for UL carrier RRC reconfiguration NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8.

The interruption duration is 1ms. We extend the requirements for 480kHz and 960kHz as shown is Table 1:

Proposal 3: For Standalone NR Carrier Aggregation and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements can be shown in Table 1:

Table 1: Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, an interruption is allowed on serving cells. For the deployment scenario of FR2-2 DC with FR1, if the victim CC is in FR2-2 and the aggressor is in FR1, namely UE is identifying CGI of the serving cell of FR1, this would cause an interruption on FR2-2 serving cells. The existing interruption requirements for Standalone NR CA aggregation are specified as below [38.133].

Table 8.2.2.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37.

In [2], it was agreed that the interruption length during MIB decoding time period is 6ms. As for interruptions during SIB1 decoding, the interruption length is as follows:

- 2 slots (target cell SCS) + margin for SSB and CORESET for RMSI scheduling multiplexing patterns 1

- 7symbols (target cell SCS) + margin for SSB and CORESET for RMSI scheduling multiplexing patterns 1

In [3], it was agreed that the margin for SIB1 decoding is as follows:

- 2*2ms + 1slot (victim cell SCS)

Based on the above information, we extend the requirements for 480kHz and 960kHz as shown is Table 2:

Proposal 4: For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps, the interruption requirements can be shown in Table 2:

Table 2: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

Conclusion

Proposal 1: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, ‘Aggressor is on FR2-1’ need to be removed from the existing requirements.

Proposal 2: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, the interruption requirements need to be defined until there is conclusion for the MRTD requirements for FR2-2.

Proposal 3: For Standalone NR Carrier Aggregation and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements can be shown in Table 1:

Table 1: Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

Proposal 4: For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps, the interruption requirements can be shown in Table 2:

Table 2: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

- References

[1] R4-2115351

[2] R4-2005843

[3] R4-2008685